3GPP TSG-SA2 Meeting #169	S2-2505882

Fukuoka, Japan, 19th May 2025 - 23rd May 2025	(revision of S2-2505628, S2-2505131)

Source:				CEWiT

Title:			Solution for KI#3: Sensing Entity and Sensing Function Discovery

Document for:				Approval

Agenda Item:				20.2.1

Work Item / Release:	FS_Sensing_ARC / Rel-20

Abstract: The contribution proposes a solution for Key Issue #3: Sensing Entity and Sensing Function Discovery and (Re-)Selection

- Introduction

This paper presents a solution that enables the 5G System (5GS) to select and appropriate Sensing entity(ies) and discover Sensing function during a Sensing procedure.

- Text Proposal

It is proposed to agree the following changes in 3GPP TR 23.700-14.

- * * * * Start of Changes * * * *

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Solutions | 1 | 2 | 3 | 4 | 5 | 6 |
| #X |  |  | X |  |  |  |

- 6.X	Solution #X: Sensing Function Discovery and Sensing Entity Selection for 5GA

- 6.X.0	High Level Principles

For the Sensing feature to be supported in the network, a NF namely "SeMF (Sensing Management Function)" introduced to control and manage sensing service operations.

During a sensing procedure, the SeMF discovery and selection happens as one of the initial steps to choose the appropriate SeMF based on multiple selection criteria. The NF service consumer discovers and selects the SeMF either by querying the NRF or based on local configuration.

SeMF selects appropriate Sensing Entity(ies) with the support of AMF during the sensing procedure. The Sensing Entity(ies) selection can take place based on multiple selection criteria. The sensing capabilities of the Sensing Entity(ies) are provided to the AMF in the initial steps during the registration of Sensing Entity(ies) with AMF. The AMF provides the list of candidate Sensing Entity(ies) along with their sensing capabilities to SeMF. Then, the SeMF chooses the appropriate Sensing Entity based on the sensing capabilities and other selection criteria.

Editors Note: The expansion of the procedures to UE based sensing is FFS.

- 6.X.1	Description

The proposed solution addresses Key Issue #3 on Sensing Function and Sensing Entity Discovery and (Re)Selection. It describes the procedure and key parameters required for the Discovery and (Re)Selection of Sensing Function and Sensing Entity.

In general, this solution is to be considered as common for Discovery and (Re)Selection of both NG-RAN and UE as sensing entity(ies).

- 6.X.2	Procedures

### 6.X.2.x		SeMF Discovery and Selection

- SeMF selection and discovery happens during a Sensing procedure. The Sensing Function selection functionality may be supported by the NF service consumer.

- Sensing Function re-selection is a functionality may be supported by the NF service consumer, when necessary, e.g. due to unavailability of SeMF to serve a specific area during a procedure.

- The Sensing function selection/re-selection may be performed by the NF service consumer based on either locally available information i.e. Sensing Function profiles are configured locally at the NF service consumer, or by querying NRF.

The following factors may be considered during the SeMF selection:

-	SeMF Target Service Area consisting of geographical co-ordinates-based representation of the sensing service area, one or more Cell ID(s), Tracking Area(s)

Editor’s Note: It is FFS if the Tracking area definition for sensing would be redefined.

-	Supported Quality of Service information to indicate the accuracy of the sensing e.g as listed below

-	Sensing range,

-	Sensing resolution (range and angle),

-	Response time (latency)

-	Identity of the applications supported by the network.

-	SeMF load.

-	Indication of either a single event report or multiple event reports i.e., duration of event reporting.

Figure 6.X.2.x: Procedure to support SeMF Discovery and Selection

- A pre-requisite step for SeMF Discovery would be SeMF registration to NRF with a profile containing it’s Sensing parameters. Upon a sensing request, the NF service consumer decides to select an appropriate SeMF to initiate sensing request.

Editor's note:	The details of parameters provided by SeMF during registration to NRF is FFS.

2.	The NF service consumer invokes a Nnrf_NFDiscovery_Request service operation to an NRF to discover a SeMF, the service operation includes the discovery parameters as listed above.

3.	The NRF sends a Nnrf_NFDiscovery_Request response which includes the profiles of the selected SeMFs to the NF service consumer.

4.	The NF service consumer selects the appropriate SeMF instance from the list of candidate SeMFs provided by NRF

5.	The NF service consumer invokes the Nsemf_Sensing Request for initiating sensing procedure.

### 6.X.2.y		Sensing Entity Discovery and Selection

Sensing Entity selection and discovery happens during a sensing service to collect sensing data to compute the sensing result. The Sensing Entity selection shall be performed by SeMF.

Sensing Entity re-selection is a functionality shall be supported by the SeMF when necessary, e.g. when the sensing measurement result does not meet the requested accuracy.

SeMF may obtain information about the sensing capabilities of Sensing Entity(ies).

The following factors may be considered during the Sensing Entity selection:

-	Sensing Entity(ies) Capabilities

-	Requested Service Area consisting of one or more Cell ID(s), Tracking Area(s).

Editor’s Note: It is FFS if the Tracking area definition for sensing would be redefined.

-	Sensing Entity(ies) location.

Sensing Entity selection/re-selection can also be based on locally configured information at SeMF.

Figure 6.X.2.y: Procedure to support Sensing Entity Discovery and Selection

0.	The sensing capabilities are provided to the AMF by SE(s) in prior. AMF would store these capabilities and expose it as part of AMF Event exposure service.

Editor's note:	The procedure of how SE(s) provides the sensing capabilities to AMF and where these sensing capabilities are stored is FFS.

1.	Upon a sensing request, the SeMF decides to select an appropriate Sensing Entity(ies) for the sensing procedure.

2.	The SeMF discovers an appropriate AMF based on the target sensing area by querying NRF or using local configuration.

Editor's note:	Further Clarity on what other parameters are required to select AMF is FFS.

3.	The SeMF requests the AMF to provide the sensing entities present in the target sensing area (Area of Interest) using Namf_EventExposure service. SeMF can also request to provide the sensing capabilities of sensing entities if available at AMF.

4.	The AMF checks for the sensing entities available in the requested target sensing area and provides the list of Sensing Entity(ies) along with their respective sensing capabilities to SeMF

5.	The SeMF selects the appropriate Sensing Entity(ies) from the list of candidates provided by AMF, based on the selection criteria provided

6.	The SeMF requests and co-ordinates with the selected Sensing Entity(ies) to provide sensing measurement data.

- 6.X.3	Impacts on Services, Entities and Interfaces

The following impacts may be present for sensing entity and SeMF discovery,

NF Service Consumer:

-	NEF/AF/NF can be NF service consumer supporting the discovery and selection of SeMF

-	NF Service Consumer queries NRF with required filters for SeMF Discovery.

-	Support at NF Service Consumer for Discovery and Selection of SeMF

SeMF:

-	Support for discovery and selection of Sensing Entity(ies)

-	Support for Sensing entity/AMF discovery and selection

-	Support for Event subscription for obtaining candidate sensing entities from AMF

NRF:

-	Support of NF Profile of SeMF

AMF:

### -	Support for Sensing Entity Selection/Re-Selection by SeMF through SE(s) capability exposure using Event exposure service.

### -	Support for storage and retrieval of sensing capabilities of Sensing Entities.

* * * * End of Changes * * * *