SA WG2 Meeting #169	S2-2505943

Fukuoka, Japan, 19th May 2025 – 23rd May 2025

# Title:	Reply LS on AI/ML Positioning Case 3a

# Response to:	LS on AI/ML Positioning Case 3a (S2-2504525/ R3-252487)

# Release:	Rel-19

# Work Item:	NR_AIML_air-Core

Source:	SA2

To:	RAN3

Contact Person:

# Name:	Dr. Thomas Belling

E-mail Address:	 thomas (dot) belling (at) nokia (dot) com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	none

1. Overall Description:

SA2 thanks RAN3 for their LS. SA2 would like to answer the questions raised by RAN3 as follows:

RAN3 question:

As part of the work on use case 3a for AI/ML assisted positioning, included in the WI NR_AIML_air-Core, RAN3 is working on solutions to retrieve positioning data at the NG-RAN. Such data can be used to manage NG-RAN sided models, e.g. to train or to monitor such models.

RAN3 has identified the following solutions and RAN3 would like to ask SA2 whether they are feasible.

Solution 1: Positioning Information collection for UEs under positioning

1. The LMF sends the NRPPa MEASUREMENT REQUEST message to one or more gNBs according to positioning procedures. 2. The gNB(s) determines that data collection is needed for the UE being positioned. 3. The gNB(s) sends the NRPPa MEASUREMENT RESPONSE message to the LMF and indicates that data collection is needed for the UE being positioned. Note: Steps 1 to 3 may occur while the LMF performs one or more of the positioning procedures described in clauses 6.11.1, 6.11.2 and 6.11.3 of TS 23.273 [35]. 4. The LMF sends the (new) NRPPa POSITIONING DATA COLLECTION REPORT message to the gNB(s) which indicated that positioning data collection is needed for the UE being positioned. The message includes information related to UE location and correlation information that enables the gNB to correlate the information related to UE location with information related to UL measurements (e.g., LMF Measurement ID and RAN Measurement ID).

Solution 2: Positioning Information collection for UEs served by the NG-RAN which are not being positioned

- The gNB wishes to receive positioning information, consisting of labels (e.g. UE location information) to form a data set for training. Based on gNB implementation, the gNB decides when/if to trigger a (new) NG: POSITIONING INFORMATION REQUEST message to the serving AMF of the served UE(s). - The POSITIONING INFORMATION REQUEST message is an NGAP non-UE associated message containing e.g. the NGAP UE IDs of the UEs for which positioning information is required or other criteria. - The AMF receives the request message from the NG-RAN node. If the requested information is already available at AMF as part of UE positioning context, the AMF responds directly with the message in step 6. Otherwise, it selects an LMF to request positioning of each UE for Positioning Information collection purposes. The AMF triggers a Positioning Information Request towards the LMF (it could consist of the (legacy) Nlmf_Location_DetermineLocation service operation) to request the current location of the UE. - The LMF performs one or more of the positioning procedures described in clause 6.11.1, 6.11.2 and 6.11.3 of TS 23.273. - The LMF returns the requested information towards the AMF. The message Nlmf_Location_DetermineLocation Response might be reused. - The AMF returns the requested label(s) to the NG-RAN node in a POSITIONING INFORMATION RESPONSE message. The AMF may return instead a failure message with appropriate cause value if the procedure does not succeed. - The gNB combines the received label with the TRP measurements to form the data needed.

2. Actions:

To SA2

ACTION: 	RAN3 asks SA2 to provide feedback on the solutions above and to check for feasibility of solution 2 in Rel19.

SA2 Answer:

For solution 1 it may be desirable to document related LMF procedures in TS 23.273. There are SA2 impacts to support the solution but SA2 could address those impacts as alignment with RAN in Rel-19.

For solution 2, SA2 concludes that this solution is not feasible in current frozen Rel-19.

SA2 is currently discussing for data collection for training models to be used by the LMF for positioning (RAN case 3b) whether a user consent check and/or a LCS privacy profile check is required. SA2 is anticipating that similar checks could also be required for case 3a (both for solution 1 and solution 2).

2. Actions:

To RAN3 group.

ACTION: 	SA2 kindly asks RAN3 to take the provided answers into account and update SA2 on the solutions selected by RAN3.

# 3. Date of Next SA2 Meetings:

TSG SA WG2 Meeting #170	25th August – 29th August 2025	Goteborg, Sweden

TSG SA WG2 Meeting #171	13th October – 17th October 2025	Wuhan, China