# 3GPP TSG-SA WG2 Meeting #169	S2-2506051

Fukuoka, JP, 19 - 23 May 2025	(Revision of S2-2505912)

Title:	[Draft] Reply LS on Handling of UE Radio Capability for Paging

Response to:	S2-2504518/R3-252276

Release:	Rel-17

# Work Item:	     FS_NewRAT-Core, 5GS_ph1, TEI17

Source:	SA2

To:	RAN3

Cc:	RAN2

Contact Person:	Qian Chen

E-mail Address:	qian (dot) xb (dot) chen (at) ericsson (dot) com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	23.501 CR 6247

1. Overall Description:

SA2 thanks RAN3 for the questions related to UE radio capability for paging information.

SA2 has provides the following feedback:

RAN3 would like to ask SA2 that when the NG-RAN node receives the UE Radio Capability for Paging IE in the INITIAL UE CONTEXT SETUP REQUEST message, and if the UE Radio Capability for Paging Information does not contain paging information on all NG-RAN RATs that the UE supports, then should the NG-RAN node send to the AMF the paging information on the missing NG-RAN RATs or all NG-RAN RATs that the UE supports in the UE RADIO CAPABILITY INFO INDICATION message?

SA2 Answer 1: SA2 considers that “UE Radio Capability for Paging” is an Information Element that will be stored in AMF and transferred between NG-RAN and AMF. The AMF never interprets the information in it, nor processes it. Hence SA2 believe that the RAN needs to send the information as a single IE. AMF replaces the old IE, if there is one stored, with the new received IE. NG-RAN is responsible to check and upload the proper information (e.g. gNB includes NR and additionally E-UTRA information, but not NB-IoT) to AMF. NG-RAN shall not just provide “missing NG-RAN RAT”.

RAN3 would also like to ask if the “that PLMN” in TS 23.501 clause 5.4.4.3 is interpreted as the serving PLMN?

SA2 Answer 2: SA2 confirms that “that PLMN” can be interpreted as serving PLMN.

SA2 has agreed the attached CR to clarify above aspects.

2. Actions:

To RAN3

ACTION: 	SA2 kindly asks RAN3 to take above information into consideration and updates corresponding specs applicable.

3. Date of Next SA2 Meeting:

SA2#170   				25 – 29 August, 2025				      Goteborg, SE

SA2#171   				13 – 17 October, 2025			       Wuhan, CN