SA WG2 Meeting ##169	S2-2504508

19 - 23 May, 2025, Fukuoka, Japan

3GPP TSG-CT WG4 Meeting #128	C4-251476

# Wuhan, P.R. China; 07th – 11th April 2025

# Title:	LS on Encoding of (S)RTP Multiplexed Media Identification Information

# Release:	Rel-19

# Work Item:	XRM_Ph2

Source:	CT4

To:	CT1, CT3, SA4

Cc:	SA2

Contact Person:

Name:	Bruno Landais

Tel. Number:

E-mail Address:	bruno.landais@nokia.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	C4-251347 [CR 29.244 #0959 on Encoding of (S)RTP Multiplexed Media Identification Information]

1. Overall Description:

CT4 observed that CT1, CT3 and CT4 have specified different encodings for the (S)RTP Multiplexed Media Identification Information.

CT4 has agreed the attached 29.244 CR aligning the encoding over N4 on the encoding defined over N1 to support encoding multiple instances of (S)RTP Multiplexed Media Identification Information in packet filters, whereby each instance of (S)RTP Multiplexed Media Identification Information can encode a combination of at most 1 SSRC, 1 PT, 1 MID and/or 1 RTCP Packet Type.

CT4 would like to point out the following remaining differences:

1) For the MID information, the (S)RTP Multiplexed Media Identification Information defined in TS 29.244 only encodes the Media Identification Tag included either in RTCP MID SDES item or RTP SDES header extension for MID, together with its length, whereas the (S)RTP Multiplexed Media Identification Information defined in TS 24.501 encodes the complete RTCP MID SDES item field (as a maximum length of 257 octets RTCP MID SDES item field as specified in IETF RFC 9143) or RTP SDES header extension for MID field (field as specified in IETF RFC 9143).

The encoding of the MID information needs to be aligned.

2) The encoding defined in TS 24.501 (copied below for convenience) does not seem to allow signaling a single instance of (S)RTP Multiplexed Media Identification Information that is applicable for both RTP and RTCP packets, e.g. when it is desired to apply the same QoS requirements for both RTP and RTCP packets of a same media flow identified by a specific SSRC ID or MID. For doing so, CT4 understands that the encoding defined in TS 24.501 would require encoding one instance of (S)RTP Multiplexed Media Identification Information for RTP traffic and as many instances of (S)RTP Multiplexed Media Identification Information for RTCP traffic as there are different applicable RTCP packet types.

An (S)RTP multiplexed media identification information entry for (s)RTCP shall contain the RTCP packet type field and at least one of the SSRC field and RTCP MID SDES item field.

An (S)RTP multiplexed media identification information entry for (s)RTP shall contain:

a)	SSRC field;

b)	payload type field;

c)	RTP SDES header extension for MID; or

d)	any combination of a) to c).

If the above CT4 understanding is correct, CT4 thinks that the above encoding could be optimized, especially considering the earlier SA4 Reply LS in S2-2412586 (S4-241812) that stated the following:

Regarding guidance of potential differentiated handling of (S)RTCP packets, SA4 believes that mapping both (S)RTP and (S)RTCP packets to same QoS Flow will be desirable in the absolute majority of cases. Any other approach would need explicit motivation and study of potential benefits and drawbacks.

SA4 believes that (S)RTCP Packet Type and SSRC value would suffice as extended (S)RTCP packet filters, in case it would be motivated to separate out specific (S)RTCP packets or Packet Types, recognizing that there also exist sub-divisions for some (S)RTCP Packet Types (e.g., RTCP FB in IETF RFC 4585 and RTCP-XR in IETF RFC 3611). SA4 is considering differentiated treatment of (S)RTCP packets within a QoS Flow using the PSI mechanism but did not come to a conclusion.

Accordingly, the attached 29.244 CR enables a single instance of (S)RTP Multiplexed Media Identification Information to apply to both (S)RTP and (S)RTCP traffic, when the (S)RTP Multiplexed Media Identification Information contains neither an RTCP Packet Type nor a Payload Type.

CT4 kindly asks SA4 group whether they confirm that it would be beneficial to optimize the encoding of (S)RTP Multiplexed Media Identification Information for use cases where the same QoS requirements apply to both RTP and RTCP traffic, e.g. for media streams from a same source, as described in 2).

CT4 kindly asks CT1 and CT3 groups to provide feedback on the encoding of (S)RTP Multiplexed Media Identification Information proposed in the attached 29.244 CR, and if this encoding is deemed preferable, to consider aligning their specifications accordingly.

2. Actions:

To SA4 group.

ACTION: 	CT4 kindly asks SA4 group whether they confirm that it would be beneficial to optimize the encoding of (S)RTP Multiplexed Media Identification Information for use cases where the same QoS requirements apply to both RTP and RTCP traffic, e.g. for media streams from a same source, as described in 2).

To CT1 and CT3 groups.

ACTION: 	CT4 kindly asks CT1 and CT3 groups to provide feedback on the encoding of (S)RTP Multiplexed Media Identification Information proposed in the attached 29.244 CR, and if this encoding is deemed preferable, to consider aligning their specifications accordingly.

3. Date of Next CT4 Meetings:

CT4 Meeting calendar can be found at:

https://www.3gpp.org/dynareport?code=Meetings-C4.htm