3GPP TSG-SA WG2 Meeting #169	S2-2504588

# Fukuoka, Japan, 19 – 23 May 2025	(was S2-2503220)

Source:	Qualcomm Incorporated

Title:	WT1: Solution for providing IMS voice over GEO via Control Plane

Agenda item:	20.1.1

Document for:	Discussion/Approval

1. Introduction

SA#107 approved the study for the support of IMS voice over GEO via NB-IoT NTN connecting to EPC.

Under WT-1, SA2 agreed to study CP and UP option for the support of IMS voice over GEO via NB-IoT NTN connecting to EPC. This contribution addresses the system architecture, the bearer mapping and the protocol overhead for the CP option.

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-19.

* * * First Change * * * *

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |
| --- | --- | --- | --- | --- |
| Solutions | 1 | 2 | 3 | 4 |
| X | X |  |  |  |

* * * Next Change * * * *

## 6.x	Solution X: Support of IMS voice over GEO via NB-IoT NTN connected to EPC using control plane bearers

### 6.x.1	High level principles

This solution addresses Key Issue 1 and supports IMS voice call GEO via NB-IoT NTN connected to EPC using CP bearers.

This solution is based on the usage of a new SRBs (Signaling Radio Bearers) dedicated to transmission of IMS signalling and voice packets using Control Plane CIoT EPS Optimisation.

### 6.x.2	Description

#### 6.x.2.1	Requirements

To support IMS voice service, two service data flows (SDFs) need to be supported: one for IMS signalling and the other for IMS voice packets. The voice packet flow is a real-time flow with stringent delay requirements. Due to the high round trip time over a GEO satellite link, this flow does not lend itself to acknowledged mode transmission on the radio. The IMS signalling flow on the other hand requires high reliability and can tolerate some delay.

Considering the above, when Control Plane CIoT EPS optimisation is used as defined in TS 23.401 [5], these two SDFs need to be carried over two separate signaling radio bearers (SRBs) over the air in order to meet the different QoS requirements.

The transport of the voice packet SDF should be performed with the minimum protocol overhead possible in order to minimize the delay.

#### 6.x.2.2	Network architecture

Figure 6.x.2.2-1 below shows high level architecture for NB-IoT NTN connected to EPC using EPS bearers that use Control Plane CIoT EPS optimisation. The details of the mapping of the SDFs to  EPS bearers are described in more detail in clause 6.x.2.4.

Figure 6.x.2.2-1: Architecture for NB-IoT NTN connected to EPC using CP CIoT

#### 6.x.2.3	PDN connection types

The PDN type of the PDN connection can be IP or non-IP. For the set of combinations, the following options are possible based on today’s standards:

a)	Both SDFs are transported over the same PDN connection with PDN type IP. In this case:

-	The IMS signalling flow uses SIP/UDP/IP stack;

-	The voice packet flow uses RTP/UDP/IP stack;

b)	Both SDFs are transported over the same PDN connection with PDN type non-IP.

-	The voice packet flow uses RTP/NIDD stack;

Editor's note:	The protocol stack for the IMS signalling flow for this case is FFS.

c)	IMS signalling flow and the voice packet flows use different PDN connections. Namely, the IMS signalling flow uses PDN type IP and the voice packet flow uses PDN type non-IP (RTP/NIDD stack).

#### 6.x.2.4	Bearer mapping

##### 6.x.2.4.1	Bearer mapping (General)

The IMS sigalling SDF and the voice packet flow have different transport requirements in terms of delay and reliability. For example, the voice data flow cannot use RLC Acknowledged Mode (AM), while the IMS signalling flow uses RLC AM. Therefore, the voice packet flow requires a separate SRB from the SRB used for IMS and NAS signalling. The architecture with the mapping of SDFs onto EPS bearers and then signalling radio bearers is shown in Figure x.y.z.4-1. This architecture includes a new SRB dedicated to transporting the voice packet flow (SRBx).

Figure 6.x.2.4-1: Bearer mapping for IMS voice over GEO via NB-IoT NTN connected to EPC using CP CIoT

Editor’s note:	Whether a separate SRB is needed for the IMS signalling flow depends on RAN and is FFS.

Editor's note:	The handling of the PDN connection(s) for the IMS signalling and the voice packet flow is FFS.

##### 6.x.2.4.2	Bearer mapping for IP type PDN connections

In IMS voice sessions for E-UTRAN/EPS the establishment of the dedicated EPS bearer that carries the voice packets is established “on demand” using dynamic PCC and the QCI-1 that is used for that purpose is GBR. Considering that NB-IoT does not support GBR bearers and establishment of dedicated EPS bearer in tandem with the IMS call setup will introduce additional delays some different arrangement is needed. What is proposed is:

- The default EPS bearer of the IMS PDN connection is that of QCI-5 and is used to carry IMS signalling;

- An additional dedicated EPS bearer in combination with the default EPS bearer activation at Attach or UE requested PDN connectivity procedures is established to carry the voice packets following the procedures of TS 23.401 [5], Annex F. This EPS bearer is also non-GBR and possibly needs a new QCI to be defined;

Editor’s note:	Whether a new non-GBR QCI is needed for the EPS bearer used for voice traffic is FFS.

- Following the steps of TS 23.401 [5], Annex F, in steps 7a/7b depending on whether the Attach or PDN connection establishment procedure is used MME sends the S1-AP Initial Context Setup Request or S1-AP Bearer Setup Request message to the eNodeB, including the NAS parts for both the Attach Accept message of the Attach procedure and the Bearer Setup Request of the Dedicated Bearer Activation Procedure. Similarly, in message 8, RRC Reconfiguration is used. Neither of these S1-AP and RRC messages and procedures are currently supported for NB-IoT UE, MME and eNodeB supporting only Control Plane CIoT EPS optimisation. They are though supported for UE, MME and eNodeB supporting S1-U and can be re-used.

- For the dedicated EPS bearer for the voice traffic the detailed UL and DL TFT containing all the 5-tuple information cannot be decided until the IMS session is established. At the beginning i.e. at time of Attach/PDN connection establishment, a “block all” filter shall be used for both the UL and DL. At the time of the IMS call setup, the DL TFT can be updated based on TS 23.203 [y], clause 7.4.2. At the time of the call setup the PDN GW initiated bearer modification without bearer QoS update procedure can be executed towards the UE using Downlink NAS Transport. The UL TFT in order to reduce the “over the air” signalling, the UE can locally derive based on the SDP information exchanged at the time of IMS call setup and replace the previously use “block all” packet filter.

Editor’s note:	 What information needs to be included in the Downlink NAS Transport and how to optimize the overhead is FFS.

#### 6.x.2.5	Protocol overhead for the voice packet flow

The protocol overhead for the voice flow (SRBx) is discussed below. This overhead is per voice packet.

MAC layer overhead: MAC layer overhead is one byte, since there will be only one MAC PDU per TBS.

RLC layer overhead: For RLC Unacknowledged Mode (UM) it is one byte. For RLC Transparent Mode (TM) it is zero.

Editor's note:	Whether RLC UM or RLC TM is used is FFS based on RAN2 feedback.

RRC overhead: The only overhead due to RRC encapsulation over SRBx is the length indicator for the NAS PDU, which is up to one byte. The length indictor could potentially be also eliminated (see section x.y.z.6).

Editor's note:	Whether RRC encapsulation overhead can be eliminated is FFS.

NAS layer overhead: In Rel-19, CT1 specified a new NAS message for data transfer over NAS a reduced NAS layer overhead of 3 bytes. The NAS layer overhead could potentially be reduced to zero (see section x.y.z.6).

Editor's note:	Whether further NAS layer overhead reduction is possible is FFS.

NAS security overhead: NAS layer security overhead consists of 4 bytes for MAC (integrity protection) and one byte for SN. However, VoLTE is currently transported without integrity protection in all commercial deployments since User Plane Integrity Protection in E-UTRAN/EPC even though it is standardized is not commercially deployed. If accordingly user plane integrity protection is not required, the NAS security overhead could be one byte.

Editor's note:	Whether voice packets can be transported via SRBx without integrity protection is FFS and needs to be discussed with SA3.

IP/UP/RTP overhead:

-	For IP PDN type: RoHC overhead after RoHC context establishment is 3 bytes. However, the IP/UP/UTP overhead without RoHC (e.g. during RoHC context (re-)establishment) is at least 30 bytes.

-	For non-IP PDN type (RTP/NIDD), the RTP overhead is one byte (for RTP sequence number).

Table x.y.z-1 summarizes the overhead size for different protocol layers and provides the total overhead for the best case and the worst-case scenario.

Table x.y.z-1 Protocol overhead for voice packet flow over the control plane

| Protocol layer | Description | Best Case | Worst case | Comments/impacts |
| --- | --- | --- | --- | --- |
| MAC | Single MAC PDU per TBS | 1B | 1B | Supported in the specs. |
| RLC | TM or UM | 0 (TM) | 1B (UM) | Requires spec support for RLC TM/UM for SRB. |
| RRC | RRC encapsulation of the NAS PDU | 0 | 1B | For zero RRC overhead see section x.y.z.6. |
| NAS protocol | NAS message overhead | 0B | 3B | CT1 work in Rel-19 reduced NAS OH to 3 bytes. For zero NAS overhead see section x.y.z.6. |
| NAS security | MAC and NAS SN | 1B | 5B | 1B requires MAC suppression. |
| IP/UDP/RTP |  | 1B | 3B-30B | For non-IP PDN type: 1B for RTP SN. For IP PDN type: 3B for RoHC. 30B during RoHC context (re-)establishment. |
| TOTAL |  | 3B | 14B-41B | Key factors affecting OH: - MAC (integrity protection) suppression - IP vs non-IP PDN type - For IP PDN type: RoHC OH during RoHC context (re-)establishment. - SRB identification over S1-AP (see section 6.x.2.6) - NAS PDU size pre-negotiation over SRBx (see section 6.x.2.6) |

The overhead data rate depends on the packet rate. For example, for a 3B and a 6B overhead per packet and 50 packets per second (one packet each 20 ms), the overhead data rate would be 1200 bps and 2400 bps, respectively.

#### 6.x.2.6	Protocol overhead reduction for the voice packet flow

It is possible to leverage the SRBx dedicated to the transport of the voice SDF to eliminate both the RRC and the NAS layer overhead. The NAS layer overhead discussed in this subclause does not include the NAS security overhead.

##### 6.x.2.6.1	RRC overhead reduction

Since SRBx is dedicated to the transport of the voice SDF, no RRC messages are needed over SRBx, i.e. SRBx can carry the DedicatedInfoNAS IE (NAS PDU) directly.  Furthermore, since voice packets have constant size for a given codec rate, the size of the NAS PDU can be configured during SRB setup based on the codec rate in use for the call. This way, the RRC overhead could be reduced to zero. In other words, the voice packet would be identified over the air using the logical channel ID of the SRBx at the MAC layer.

##### 6.x.2.6.2	NAS layer overhead reduction

Since SRBx is dedicated to the transport of voice pace SDF, any NAS PDU received by the MME over SRBx contains a voice packet. Therefore, NAS layer overhead is not needed to inform the MME about the content of the NAS PDU received over SRBx. To associate the NAS PDU with SRBx, the eNB could insert an indication e.g. signalling bearer identity in the S1-AP NAS transport messages (similar to how the legacy EDT session indication IE indicates the association with an EDT session in the S1-AP Initial UE message).

### 6.X.3	Procedures

Editor's note:	This clause describes high-level procedures and information flows for the solution.

### 6.X.4	Impacts to Services, Entities and Interfaces

Editor's note:	This clause captures impacts on existing 3GPP nodes and functional elements.

* * * Next Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[3]	3GPP TS 23.502: "Procedures for the 5G system, Stage 2".

[4]	3GPP TS 23.503: "Policy and Charging Control Framework for the 5G System".

[5]	3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access".

[y]	3GPP TS 23.203: " Policy and charging control architecture".

* * * End of Changes * * * *