3GPP TSG|WG-SA2 Meeting #169	S2-2504677

Fukuoka, Japan, May 19 – 23, 2025	(revision of S2-20xxxx)

Source:	META

Title:	TEI20: Resolving suboptimal user QoE with IETF’s SCONE in UPF

Agenda Item:	30.1 / TEI20

Document for:	Information

# 1.	Background

IETF is working on a simple (protocol based) solution to solve a suboptimal user experience (user QoE) issue for on demand video streaming applications due to "network enforce rate limiters" used in the networks (e.g, 5G cellular system). This work in IETF is being undertaking by Standard Communication with Network Elements WG (ref: https://datatracker.ietf.org/doc/charter-ietf-scone/).

Working Group Last Call (WGLC) protocol document and IESG approval is expected by end of 4Q 2025 and a final publication in 2026 which will be before Rel-20 CT/stage3 timeline (1Q2027).

# 2.	Discussion

For cellular system, a "network enforce rate limiters" role is to ensure the DL video data rate will not congest the radio networks by policing or shaping video flows at the ingress point at packet core user plane function (e.g., N6) when certain threshold is exceeded. There are some other motivations for using network enforce rate limiters such as supporting date rates based on users’ data plans or users’ RAT type.

Video flows are detected with proprietary traffic detection methodology (e.g. DPI, traffic patterns, etc). Policing/Shaping of video flows [by a network node on purpose] creates suboptimal user experience for video consumers.https://datatracker.ietf.org/doc/draft-zarifis-scone-taxonomy/ The current IETF protocol draft for SCONE (see: https://datatracker.ietf.org/doc/draft-thoji-scone-trone-protocol/) proposes a simple and yet effective way for "network enforce rate limiters" to communicate video bit-rate (throughput advice) directly to video application so that application itself can self-adapt the video bit rate accordingly (see below (Fig 1) with connotations added).

Fig 1, Overview of current IETF’s scone development, with added connotations.

IETF SCONE WG protocol proposes on-path communication interface between mobile network packet core user plane element (UPF) and VoD client end point on UE to enable network enforce rate limiters (for e.g. UPF) to send the video bit-rate advice to VoD client end point for video bit rate self-adaptation by application which improves the user experience for video consumers.

The overall principle is such that a supported network node (e.g., UPF) would insert the video-bit rate, in the on path TRONE/SCONE packet (flowing with video flow from server to client) as defined by IETF.

It reduces the OPEX/CAPEX for network operators as shaper/policer is not needed. It can also assist with SCONE compliant video flow detection and that makes DPI/traffic patterns-based flow detection optional.

Application client supporting this protocol would use OTT to communicate with the back-end server for video bit rate adjustment, etc. This is an optional feature and is backward compatible. There is no impact to any network or application that does not support this feature.

# 3.	Proposal.

A CR to TS 23.501 (as TEI20) to describe how UPF, based on local operator policy, may insert rate policy via on-path with reference to IETF’s SCONE, to allow client application to perform rate adaption. Informative Annex may be considered (TBD). A set of draft WID/CR will be provided at next WG meeting.