SA WG2 Meeting #169	S2-2504719

19 - 23 May, 2025, Fukuoka, Japan	(revision of S2-250xxxx)

Source:	NEC, Futurewei, Lenovo

Title:	AIoT priority information

Document for:	Approval

Agenda Item:	19.14.2

Work Item / Release:	AmbientIoT-ARC / Rel-19

Abstract: This pCR proposes AIoT priority information from AF as Assistance information provided to NG-RAN node.

1. Introduction

The AIoT Assistance information is used by the NG-RAN for performing service operations, e.g. radio resource allocation. Due to the importance and characteristic of AIoT devices and service can be different, the difference information being provided to NG-RAN can help radio resource allocation. AIoT assistance information can be provided by the service layer from AF.

In the AIoT commercial deployment, some AIoT devices or AIoT service can be more important than others. AF can provide the AIoT service / device priority in the group as assistance information for AIoT operation: when there are multiple devices in the group, AF may indicate the priority for different AIoT device; or AF may indicate the priority for certain AIoT service. When NG-RAN receives such information, NG-RAN can schedule resource accordingly with prioritizing the high-priority device or service.

This AIoT device/service priority is useful e.g., when there is congestion in NG-RAN, or there is AIoT operation time requirement from the AIoT application layer.

2. Proposal

It is proposed to agree AIoT priority information as AIoT Assistance information from AF to RAN in the following changes to 3GPP TS 23.369 v0.3.0.

* * * Start of Change * * * *

## 5.4	Assistance information provided to NG-RAN node

The AIOTF provides the following assistance information to the NG-RAN together with the service operation requests:

a)	AIoT service type (e.g. Inventory, Command).

b)	Optionally, approximate number of AIoT devices based on AF request.

c)	Optionally, approximate D2R message size based on AF request.

d)	Optionally, time interval as AIoT aggregation assistance information.

e)	Optionally, AIoT priority information. AF may indicate the priority for different AIoT devices or service requests. When NG-RAN receives such information, it prioritizes the high-priority AIoT device or service request in the corresponding AIoT operation.

Editor's note:	Other assistance information may be added later if necessary.

Editor's note:	It is FFS where detailed description on time interval as AIoT aggregation assistance information will be captured, i.e. in this clause or in another clause.

If not provided by the AF, bullets b) and c) in the above assistance information provided by the AIOTF may be based on local configuration based on SLA between the AIoT service provider represented by an AF and the operator.

If the AF has provided a time interval, then the AIOTF should set bullet d) to NG-RAN that is equal or shorter than the time interval received from the AF.

NOTE:	Based on local configuration, the AIOTF can reject the AF request, e.g. if the AF provided time interval is shorter than a locally configured minimum interval.

The assistance information is used by the NG-RAN for performing service operations, e.g. radio resource allocation by using bullets b), c) and e), AIoT Device responses aggregation by using bullet d).

* * * End of Change * * * *