3GPP SA WG2 Meeting #169	S2-2504733

19 - 23 May, 2025, Fukuoka, Japan

Title:	[DRAFT] LS Reply on RTP retransmission

Response to:	LS on RTP retransmission (S4-250739 / S2-2504566)

Release:	Release 19

Work Item:	5G_RTP_Ph2, XRM_Ph2

Source:	SA2

To:	SA4, RAN2

Cc:	-

Contact Person:

Name:	Xiaowan Ke

E-mail Address:	xiaowan.ke@vivo.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

1. Overall Description:

SA2 thanks SA4 for LS on RTP retransmission (S4-250739 / S2-2504566) and would to provide the feedback to the question.

Question to SA2: Considering the options identified above, are there any benefits to providing application-layer retransmission information to the 5GC when PDU Set based handling is enabled? If so, SA4 would like to receive feedback on what type of application-layer retransmission information would be beneficial for PDU Set based handling in the 5GC.

SA2 reply: As SA4 mentioned, “According to RFC 4588, source and retransmission PDUs associated to a media flow are transmitted in separate RTP streams.” And there are two options to handle the source PDUs and for retransmission PDUs in separate RTP streams:

-      Option1: one QoS flow is used for the two kinds pf RTP streams and different PDU set are used for source PDUs and retransmission PDUs. The retransmission PDUs that are N6-unmarked PDUs are marked to PDU Set. Although there is only one delay budget for the QoS flow, the NG-RAN should differentiate the delay budget for source PDUs and that retransmission PDUs and how to realize that is unclear.

-      Option2: distinct QoS flows are used for the two RTP streams respectively, w.r.t. different delay requirement of the source PDUs and retransmission PDUs. The N6-unmarked PDUs is not marked to PDU Set and there is no impact to the NG-RAN.

SA2 doesn’t see the benefit for option1 and determines application-layer retransmission information is not needed for 5GC, because the following:

- Marking N6-unmarked PDUs (i.e. the retransmission PDUs) to PDU Set is not efficient for retransmission since the PDU Set Identification information for the retransmission PDUs is redundant and such additional header will delay the retransmission.

- SA2 doesn’t expect NG-RAN to differentiate the delay budget of the source PDUs and retransmission PDUs which are mapped in the same QoS flow. A given QoS flow is expected to have one delay budget.

SA2 understands Option2 has been supported in r19 (see clause 5.37.11	Traffic identification and differentiated QoS for multiplexed media flows 23.501), i.e. different RTP streams with different QoS requirement are mapped to different QoS flows with the new packet filter used to distinguish separate RTP streams, e.g. SSRC, MID.  It is not necessary for the network to receive application-layer retransmission information.

In summary, SA2 does not see the benefit and necessity to introduce the application-layer retransmission information in 5GC.

2. Actions:

SA4/RAN2:

ACTION: 	SA2 kindly asks SA4 and RAN2 to take the above information into account.

3. Date of Next TSG SA WG2 Meetings:

TSG-SA2 Meeting #170		25-29 August 2025	Goteborg, SE

TSG-SA2 Meeting #171		13-17 October 2025	China