# 3GPP SA WG2 Meeting #169	S2-2504749

# Fukuoka, Japan, 19th – 23rd May, 2025,                                             (revision of S2-2505xxx)

# Title:	[Draft] Reply LS on AI/ML Positioning Case 3a

# Response to:	LS on AI/ML Positioning Case 3a (R3-252542 / S2-2504525)

# Release:	Release 19

# Work Item:	NR_AIML_air-Core

Source:	SA2

To:	RAN3

Cc:

Contact Person:

Name:	Zhang Fu

E-mail Address:	zhang DOT fu1 @ huawei DOT com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:

1. Overall Description:

SA2 thanks RAN3 for the LS about RAN3 progress on AI/ML Positioning case 3a. Given the input from RAN3, SA2 would like to provide the following reply:

Solution 1: Positioning Information collection for UEs under positioning

1. The LMF sends the NRPPa MEASUREMENT REQUEST message to one or more gNBs according to positioning procedures. 2. The gNB(s) determines that data collection is needed for the UE being positioned. 3. The gNB(s) sends the NRPPa MEASUREMENT RESPONSE message to the LMF and indicates that data collection is needed for the UE being positioned. Note: Steps 1 to 3 may occur while the LMF performs one or more of the positioning procedures described in clauses 6.11.1, 6.11.2 and 6.11.3 of TS 23.273 [35]. 4. The LMF sends the (new) NRPPa POSITIONING DATA COLLECTION REPORT message to the gNB(s) which indicated that positioning data collection is needed for the UE being positioned. The message includes information related to UE location and correlation information that enables the gNB to correlate the information related to UE location with information related to UL measurements (e.g., LMF Measurement ID and RAN Measurement ID).

Solution 2: Positioning Information collection for UEs served by the NG-RAN which are not being positioned

- The gNB wishes to receive positioning information, consisting of labels (e.g. UE location information) to form a data set for training. Based on gNB implementation, the gNB decides when/if to trigger a (new) NG: POSITIONING INFORMATION REQUEST message to the serving AMF of the served UE(s). - The POSITIONING INFORMATION REQUEST message is an NGAP non-UE associated message containing e.g. the NGAP UE IDs of the UEs for which positioning information is required or other criteria. - The AMF receives the request message from the NG-RAN node. If the requested information is already available at AMF as part of UE positioning context, the AMF responds directly with the message in step 6. Otherwise, it selects an LMF to request positioning of each UE for Positioning Information collection purposes. The AMF triggers a Positioning Information Request towards the LMF (it could consist of the (legacy) Nlmf_Location_DetermineLocation service operation) to request the current location of the UE. - The LMF performs one or more of the positioning procedures described in clause 6.11.1, 6.11.2 and 6.11.3 of TS 23.273. - The LMF returns the requested information towards the AMF. The message Nlmf_Location_DetermineLocation Response might be reused. - The AMF returns the requested label(s) to the NG-RAN node in a POSITIONING INFORMATION RESPONSE message. The AMF may return instead a failure message with appropriate cause value if the procedure does not succeed. - The gNB combines the received label with the TRP measurements to form the data needed.

# SA2 Feedback:

SA2 discussed Solution#1 and Solution#2 from RAN3. Regarding Solution#1, there is no major SA2 impact, the only impact we can see is LMF may check user consent during positioning procedure, e.g., after step 3 in Solution#1. Regarding Solution#2, SA2 would like to provide feedback based on two aspects: Problems and issues of Solution#2.

### Problems and issues of Solution#2:

- Step 2, gNB provides UE list to AMF. It is not clear if gNB checks the user consent of the UE before providing the UE list or it relies on the core network to check the user consent.

- If gNB relies on the core network to check the user consent, then AMF may need to check the user consent and/or LCS privacy profile. However, it is not clear what if some of the UEs does not provide its consent? If there is a need for UE reselection by gNB?

- Step 3, Nlmf_Location_DetermineLocation does not support multiple UEs, so AMF has to send a request for each of the UE, which is not signalling efficient.

- As RAN3 agree that data collection should include UEs which served by the gNB and UEs not served by the gNB (see RAN3 agreements from RAN3#127), how can a gNB provide a list of UEs that are not served by itself? Or we expect the AMF receives Positioning information request from multiple gNBs, and each gNB provide a list of UEs served by that gNB?

### SA2 proposal:

For RAN3 information, SA2 already defined that LMF can select UEs for AIML positioning data collection based on the data consumer (e.g., NWDAF) request. LMF can select UEs which are served by different gNBs, the selected UEs may include UEs being positioned, or not being positioned. It could be desirable if we can leverage the newly defined LMF functionality. Thus, SA2 propose the following procedure, which is illustrated by the following figure.

- Step 1, instead of providing UE list, gNB only sends an AI positioning data collection request to AMF, including the area of interest (AoI), e.g., the target area of the AI positioning model.

- Step 2, AMF invokes Nlmf_DataExposure_Subscribe service to LMF, providing the AoI. Then LMF will conduct the UE selection, user consent checking and trigger the positioning procedure to gNB(s), which is already defined in clause 6.22.3 in TS 23.273. Note that, LMF may select UEs that are not served by gNB-1, then it may contact other gNBs for data collection. Therefore, we cover the case that data collection should include UEs which served by gNB-1 and UEs not served by the gNB-1.

- Alternatively, if gNB-1 already has NRPPa session with LMF, it can send the data collection request to LMF via NRPPa as shown in step 1a. Then step1, 2, 4, 5 will be skipped, LMF sends back the AI positioning data back to the gNB with NRPPa.

2. Actions:

To RAN3 group

ACTION:

SA2 asks RAN3 to take SA2’s feedback into account for future work.

3. Dates of next TSG SA WG 2 meetings:

TSG-SA2 Meeting #170		August 25 – August 29, 2025		Goteborg, SE

TSG-SA2 Meeting #171		October 13 – October 17, 2025		TBD, CN