3GPP TSG-SA WG2 Meeting #169	S2-2504835

Fukuoka, Japan, May 19 – 23, 2025 	(revision 3125 of 2001)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 5240 | rev | 3 | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Negotiation for future background data transfer considering energy |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NTT DOCOMO, Toyota, KDDI |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | EnergySys |  |  |  |  |  | Date: |  |  | 2025-03-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The high-level principle of considering energy-related information is captured in clause 5.51.4 of TS 23.501. This concept is also reflected in clause 6.1.2.4 of TS 23.503. However, the updates in the corresponding procedures in TS 23.502 is missing. This CR updates the BDT negotiation procedure in TS 23.502 to consider energy-related information. Energy-related information retrieval from OAM is based on the options explained by SA5 in LS in S2-2400072 (S5-238106). Moreover, the interaction between PCF and NWDAF was not clear in the procedure which is corrected in this CR. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Adding consideration of energy-related information in BDT negotiation in the case of indication by AF. - Clarifying the interaction between PCF and NWDAF in the procedure figure. - Adding reference to SA5 specification |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistency among the specifications |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 4.16.7.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

********** First Change **********

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[3]	IETF RFC 7296: "Internet Key Exchange Protocol Version 2 (IKEv2)".

[4]	Void.

[5]	Void.

[6]	IETF RFC 4861: "Neighbor Discovery for IP version 6 (IPv6)".

[7]	3GPP TS 23.040: "Technical realization of the Short Message Service (SMS)".

[8]	IETF RFC 4862: "IPv6 Stateless Address Autoconfiguration".

[9]	3GPP TS 38.300: "NR and NG-RAN Overall Description; Stage 2".

[10]	3GPP TS 38.413: "NG-RAN; NG Application Protocol (NGAP)".

[11]	Void.

[12]	3GPP TS 38.331: "NR; Radio Resource Control (RRC); Protocol Specification".

[13]	3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access".

[14]	Void.

[15]	3GPP TS 33.501: "Security Architecture and Procedures for 5G System".

[16]	3GPP TS 36.331: "Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC); Protocol specification".

[17]	3GPP TS 29.500: "5G System; Technical Realization of Service Based Architecture; Stage 3".

[18]	3GPP TS 29.518: "5G System; Access and Mobility Management Services; Stage 3".

[19]	Void.

[20]	3GPP TS 23.503: "Policy and Charging Control Framework for the 5G System ".

[21]	IETF RFC 4191: "Default Router Preferences and More-Specific Routes".

[22]	3GPP TS 23.122: "Non-Access-Stratum (NAS) functions related to Mobile Station in idle mode".

[23]	3GPP TS 23.682: "Architecture enhancements to facilitate communications with packet data networks and applications".

[24]	3GPP TS 23.203: "Policy and charging control architecture".

[25]	3GPP TS 24.501: "Non-Access-Stratum (NAS) protocol for 5G System (5GS); Stage 3".

[26]	3GPP TS 23.402: "Architecture enhancements for non-3GPP accesses".

[27]	Void.

[28]	3GPP TS 23.167: "IP Multimedia Subsystem (IMS) emergency sessions".

[29]	Void.

[30]	Void.

[31]	Void.

[32]	3GPP TS 29.507: "Access and Mobility Policy Control Service; Stage 3".

[33]	3GPP TS 23.003: "Numbering, Addressing and Identification".

[34]	Void.

[35]	3GPP TS 23.251: "Network sharing; Architecture and functional description".

[36]	3GPP TS 29.502: "5G System; Session Management Services; Stage 3".

[37]	3GPP TS 29.510: "5G System; Network function repository services; Stage 3".

[38]	3GPP TS 23.380: "IMS Restoration Procedures".

[39]	3GPP TS 32.421: "Telecommunication management; Subscriber and equipment trace; Trace concepts and requirements".

[40]	IETF RFC 4555: "IKEv2 Mobility and Multihoming Protocol (MOBIKE)".

[41]	3GPP TS 24.502: "Access to the 3GPP 5G Core Network (5GCN) via Non-3GPP Access Networks (N3AN); Stage 3".

[42]	3GPP TS 32.290: "Services, operations and procedures of charging using Service Based Interface (SBI)".

[43]	3GPP TS 36.304: "Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) procedures in idle mode".

[44]	3GPP TS 38.304: "NR; User Equipment (UE) procedures in idle mode".

[45]	3GPP TS 32.255: "5G system; 5G data connectivity domain charging; Stage 2".

[46]	3GPP TS 36.300: "Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Overall description; Stage 2".

[47]	3GPP TS 29.513: "5G System; Policy and Charging Control signalling flows and QoS parameter mapping; Stage 3".

[48]	IEEE Std 802.11-2016 (Revision of IEEE Std 802.11-2012): "IEEE Standard for Information technology - Telecommunications and information exchange between systems Local and metropolitan area networks - Specific requirements - Part 11: Wireless LAN Medium Access Control (MAC) and Physical Layer (PHY) Specifications".

[49]	IETF RFC 2410: "The NULL Encryption Algorithm and its use with IPsec".

[50]	3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services; Stage 2".

[51]	3GPP TS 23.273: "5G System (5GS) Location Services (LCS); Stage 2".

[52]	3GPP TS 29.503: "5G System; Unified Data Management Services; Stage 3".

[53]	3GPP TS 23.316: "Wireless and wireline convergence access support for the 5G System (5GS)".

[54]	3GPP TS 23.222: "Functional architecture and information flows to support Common API Framework for 3GPP Northbound APIs; Stage 2".

[55]	3GPP TS 23.228: "IP Multimedia Subsystem (IMS); Stage 2".

[56]	3GPP TS 36.321: "Evolved Universal Terrestrial Radio Access (E-UTRA); Medium Access Control (MAC) protocol specification".

[57]	3GPP TS 29.512: "5G System; Session Management Policy Control Service; Stage 3".

[58]	3GPP TS 29.525: "5G System; UE Policy Control Service; Stage 3".

[59]	IETF RFC 6696: "EAP Extensions for the EAP Re-authentication Protocol (ERP)", July 2012.

[60]	IETF RFC 5295: "Specification for the Derivation of Root Keys from an Extended Master Session Key (EMSK)", Aug. 2008.

[61]	3GPP TS 23.272: "Circuit Switched (CS) fallback in Evolved Packet System (EPS); Stage 2".

[62]	3GPP TS 29.501: "5G System; Principles and Guidelines for Services Definition; Stage 3".

[63]	3GPP TS 29.561: "5G System; Interworking between 5G Network and external Data Networks; Stage 3".

[64]	3GPP TS 29.413: "Application of the NG Application Protocol (NGAP) to non-3GPP access".

[65]	Void.

[66]	IEEE Std 802.1Q-2022: "IEEE Standard for Local and Metropolitan Area Networks-Bridges and Bridged Networks".

[67]	Void.

[68]	3GPP TS 23.632: "User Data Interworking, Coexistence and Migration".

[69]	3GPP TS 29.244: "Interface between the Control Plane and the User Plane nodes".

[70]	3GPP TS 29.571: "5G System; Common Data Types for Service Based Interfaces; Stage 3".

[71]	3GPP TS 32.256: "Charging Management; 5G connection and mobility domain charging; Stage 2".

[72]	3GPP TS 38.423: "NG-RAN; Xn Application Protocol (XnAP)".

[73]	3GPP TS 23.287: "Architecture enhancements for 5G System (5GS) to support Vehicle-to-Everything (V2X) services".

[74]	3GPP TS 23.548: "5G System Enhancements for Edge Computing; Stage 2".

[75]	IEEE Std 802.1AS-2020: "IEEE Standard for Local and metropolitan area networks--Timing and Synchronization for Time-Sensitive Applications".

[76]	IEEE Std 1588-2019: "IEEE Standard for a Precision Clock Synchronization Protocol for Networked Measurement and Control".

[77]	3GPP TS 23.304: "Proximity based Services (ProSe) in the 5G System (5GS)".

[78]	3GPP TS 23.247: "Architectural enhancements for 5G multicast-broadcast services".

[79]	3GPP TS 23.060: "General Packet Radio Service (GPRS); Service description; Stage 2".

[80]	3GPP TS 23.256: "Support of Uncrewed Aerial Systems (UAS) connectivity, identification and tracking; Stage 2".

[81]	3GPP TS 23.216: "Single Radio Voice Call Continuity (SRVCC); Stage 2".

[82]	3GPP TS 29.519: "5G System; Usage of the Unified Data Repository service for Policy Data, Application Data and Structure Data for Exposure; Stage 3".

[83]	3GPP TS 23.558: "Architecture for enabling Edge Applications".

[84]	3GPP TS 23.540: "Technical realization of Service Based Short Message Service; Stage 2".

[85]	3GPP TS 29.598: "Unstructured data storage services".

[86]	3GPP TS 23.041: "Technical realization of Cell Broadcast Service (CBS)".

[87]	3GPP TS 29.522: "5G System; Network Exposure Function Northbound APIs; Stage 3".

[88]	3GPP TS 23.586: "Architectural Enhancements to support Ranging based services and Sidelink Positioning".

[89]	3GPP TS 29.214: "Policy and Charging Control over Rx reference point".

[90]	3GPP TS 23.015: "Technical realization of Operator Determined Barring (ODB)".

[91]	3GPP TS 29.505: "5G System; Usage of the Unified Data Repository service for Subscription Data".

[92]	3GPP TS 28.405: "Quality of Experience (QoE) measurement collection; Control and configuration".

[93]	3GPP TS 29.564: "User Plane Function Services; Stage 3".

[94]	3GPP TS 33.533: "Security aspects of ranging based services and sidelink positioning".

[95]	3GPP TS 33.122: "Security aspects of Common API Framework (CAPIF) for 3GPP northbound APIs".

[96]	3GPP TS 23.204: "Support of Short Message Service (SMS) over generic 3GPP Internet Protocol (IP) access; Stage 2".

[97]	IETF RFC 9298: "Proxying UDP in HTTP".

[98]	IETF draft-ietf-masque-quic-proxy: "QUIC-Aware Proxying Using HTTP".

Editor's note:	The above document cannot be formally referenced until it is published as an RFC.

[99]	IETF draft-ietf-tsvwg-udp-options: "Transport Options for UDP".

Editor's note:	The above reference will be revised to RFC when finalized by IETF.

[X]	3GPP TS 28.622: "Telecommunication management; Generic Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)".

[Y]	3GPP TS 28.532: "Management and orchestration; Generic management services".

********** Next Change **********

#### 4.16.7.2	Procedures for future background data transfer

Figure 4.16.7.2-1: Negotiation for future background data transfer

1.	The AF invokes the Nnef_BDTPNegotiation_Create (ASP Identifier, Number of UEs, Volume per UE, Desired time window and optionally External Group Identifier, Network Area Information, Request for notification, MAC address or IP 3-tuple of Application server). The Request for notification is an indication that BDT warning notification should be sent to the AF.

2a.	Based on an AF request, the NEF requests to translate the External Group Identifier into the Internal Group Identifier using Nudm_SDM_Get (Group Identifier Translation, External Group Identifier).

2b.	The NEF invokes the Npcf_BDTPolicyControl_Create (ASP Identifier, Number of UEs, Volume per UE, Desired time window and optionally Internal Group Identifier, the Network Area Information, Request for notification, MAC address or IP 3-tuple of Application server) with the H-PCF to authorize the creation of the policy regarding the background data transfer. If the PCF was provided with Request for notification, then PCF may send BDT warning notification to the AF as described in clause 4.16.7.3.

3	The H-PCF may request from the UDR the stored Background Data Transfer policies for all the ASPs using Nudr_DM_Query (Policy Data, Background Data Transfer) service operation.

NOTE 1:	If only one PCF is deployed in the PLMN, the Background Data Transfer policy can be locally stored and no interaction with UDR is required.

4.	The UDR provides all the stored Background Data Transfer policies and corresponding related information (i.e. volume of data to be transferred per UE, the expected amount of UEs) to the H-PCF.

5.	The PCF may interact with the NWDAF and request the Network Performance analytics information for the Desired time window and the Network Area Information as defined in TS 23.288 [50].

NOTE 2:	When the External Group Identifier was provided and the Network Area Information was not provided by the AF at step 1, the NWDAF derives the Network Area Information from the Internal Group ID as defined in clause 6.6 of TS 23.288 [50].

6.	[optional] In the case of Energy Saving Acceptable indicator in the AF request, the PCF may get energy-related information from OAM in NF level via creating management jobs (defined in TS 28.622 [X]) and invoking corresponding operations (defined in TS 28.532 [Y])

7.	The H-PCF determines, based on information provided by the AF and other available information obtained in steps 5-6, one or more Background Data Transfer policies. The PCF may interact with the NWDAF and request the Network Performance analytics information for the Desired time window and the Network Area Information as defined in TS 23.288 [50].

NOTE 2:	When the External Group Identifier was provided and the Network Area Information was not provided by the AF at step 1, the NWDAF derives the Network Area Information from the Internal Group ID as defined in clause 6.6 of TS 23.288 [50].

NOTE 3:	The maximum aggregated bitrate is not enforced in the network.

8.	The H-PCF send the acknowledge message to the NEF with the acceptable Background Data T Transfer policies and a Background Data Transfer Reference ID.

9.	The NEF sends a Nnef_BDTPNegotiation_Create response to the AF to provide one or more background data transfer policies and the Background Data Transfer Reference ID to the AF. The AF stores the Background Data Transfer Reference ID for the future interaction with the PCF. If the NEF received only one background data transfer policy from the PCF, steps 8-11 are not executed and the flow proceeds to step 12. Otherwise, the flow proceeds to step 8.

NOTE 4:	If the NEF receives only one Background Data T Transfer policy, the AF is not required to confirm.

10.	The AF invokes the Nnef_BDTPNegotiation_Update service to provide the NEF with Background Data Transfer Reference ID and the selected background data transfer policy.

11.	The NEF invokes the Npcf_BDTPolicyControl_Update service to provide the H-PCF with the selected background data transfer policy and the associated Background Data Transfer Reference ID.

12.	The H-PCF sends the acknowledge message to the NEF.

13.	The NEF sends the acknowledge message to the AF.

14.	The H-PCF stores the Background Data Transfer Reference ID together with the new Background Data T Transfer policy, the corresponding related information (i.e. volume of data to be transferred per UE, the expected amount of UEs), optionally MAC address or IP 3-tuple of Application server the information of request for notification, together with the relevant information received from the AF (as defined in clause 6.1.2.4 of TS 23.503 [20]) in the UDR by invoking Nudr_DM_Update (BDT Reference id, Policy Data, Background Data Transfer). This step is not executed, when the PCF decides to locally store the Background Data Transfer policy.

15.	The UDR sends a response to the H-PCF as its acknowledgement.

********** End of Changes **********