SA WG2 Meeting #169	S2-2504836

# Fukuoka, Japan, May 19 – 23, 2025

Source: 	NTT DOCOMO

Title: 	KI#1: New Sol: UP-based UE data collection

Document for: 	Approval

Agenda Item: 	20.30.1 (FS_AIML_CN_Ph2)

Work Item / Release:	FS_AIML_CN_Ph2 / Rel-20

Abstract of the contribution: This contribution proposes a solution for KI#1 to transfer data from UE via UP where the data transfer is controlled via CP.

# 1	Discussion

To transfer UE data in the scope of the KI#1 of TR 23.700-04, in this paper, in align with the control plane user plane separation concept in 5G, a new solution is proposed.

# 2 Proposal

It is proposed to update TR 23.700-04 according to the following text.

********** First Change (All new text) **********

# Solutions

## 6.0	Mapping of Solutions to Key Issues and Use Cases

Table 6.0-1: Mapping of Solutions to Key Issues and use cases

|  | Key Issues |  | Use Cases (optional) |  |
| --- | --- | --- | --- | --- |
| Solutions | <Key Issue #1> | <Key Issue #2> | <Use Case #1> | <Use Case #2> |
| #1 | x |  |  |  |
| #2 | x |  |  |  |
| #3 | x |  |  |  |
| #4 | x |  |  |  |
| #x | x |  |  |  |

## 6.x	Solution #X: UP-based UE data collection

### 6.x.1	High level principles

The high-level principles of the proposed solution are as follow:

- To take the “futureproof and extendable solutions” requirement by RAN WGs, a new network function is proposed as the “first termination entity” to fulfil the MON’s controllability and visibility requirements.

- The UE data collection entity is deployed as a dedicated DNNI in the 5GC, and it is configured to receive traffic from the user plane. The SMF configures the UPF(s) to forward the traffic from UE(s) to the DNNI via the User Plane to the UE data collection entity. The PCF is also configured to provide the appropriate PCC rules for usage monitoring of the traffic to the DNNI.

- The UE data collection entity manages (including initiating and terminating) the data transfer process based on operator’s policy and considering the consumer (e.g., the OTT server) request (which is already authorized by the NEF). It also provides visibility for the MNO via i) keeping trace of all requests from the consumer, ii) keep trace of all the standardized data requests sent to the UE and UE response, iii) providing data content visibility based on operator’s request.

- A new NAS message is introduced to control the data transfer. It includes i) The information of the data collection entity as termination point (including DNNI, IP, port), ii) A list of standardized data to be transferred, iii) Data transfer control (e.g., start, stop).

- Based on the information received by the NAS message, the UE may stablish/release QoS Flow (and PDU session) to the data collection entity and start/stop the data transfer process.

### 6.x.2	Description

In the solution, data collection and transfer from UE is realized via

- Control plane signalling to manage the process including informing the UE to collect data and configure the information (e.g., DNN, FQDN, Port #, …) of the data collection entity deployed in the 5GC

- User plane data transfer from the UE to the data collection entity in the 5GC

The solution is composed of three main phases:

- The data collection entity (e.g., a new NF) is deployed in the MNO’s network with a FQDN and with a dedicated DNN which is reachable to UPFs via N6 interface. SMF configures the UPFs to forward the traffic targeted to the specified FQDN and/or DNN to the data collection entity. Moreover, PCF provides the appropriate PCC rules for usage monitoring of the traffic.

- For the UEs that support data collection and also subject to UE consent (which would be extended to consider air interface data collection), the network sends the information of the data collection entity deployed in 5GC to the UE via e.g., in the UE registration procedure or after registration via existing “UE Policy Container” update mechanism or a dedicated NAS message. The network also indicates the standardized data and also instruction to the UE for start/stop/pause data collection/transfer, data collection period, etc.

- Based on the information and the instruction received from the network, UE optionally establishes a dedicated PDU session and request a QoS flow (either in a existing PDU session or a new one) to the specified data collection entity and then start collecting and transferring the data via the UP path.

An example realization of the proposed solution is depicted in Figure 6.x.2-1.

Figure 6.x.2-1: An example of proposed solution

In step 0, the SMF configures the appropriate FAR rules and also the usage monitoring rules received from the PCF in the UPF to forward traffic to the data collection entity via N6 interface (this could be an extension of “user plane tunnel established on N4 interface,” defined in TS 29.244). When there is request for UE data, e.g., UE model training server needs training data, the consumer requests the data from the data collection entity in step 1. In the case of external data consumer, NEF authorizes the request and applies the operator’s policies on the data collection. The data collection entity realizes that the requested data should be collected from the UE via UP and accordingly, in step 2, asks AMF to configure/instruct the UE. Based on the request from the data collection entity, AMF sends the information via N1 to the UE in step 3.

Based on the configuration/instruction received, the UE may stablish a PDU session and a QoS Flow to the specified FQDN/DNN in step 4 and then starts data collection in step 5 which is transferred via the PDU session to the UPF in step 6. Based on the configured Forwarding Action Rules, the UPF sends the received data from the UE via the N6 interface to the collection entity in step 7 and 8. Consequently, the data collection entity sends the data to the consumer (in step 9) and may also optionally store it in ADRF in step 10.

### 6.x.3	Procedures

The procedure of the proposed solution is shown in Figure 6.x.3-1.

Figure 6.x.3-1: CP-managed UP-based UE data Transfer Procedure

0a.	PCF provides usage monitoring rules for the traffic targeted to the data collection entity

0b.	The SMF configures the appropriate needed FARs in the UPF to forward data from UPF to the data collection entity via the N6 interface.

0c.	The network may optionally send the data transfer configuration information to the UE in the UE registration step.

1.	The consumer subscribes to the data collection entity for the UE data. The request is authorized by the NEF in the case of external data consumer.

2.	The data collection entity checks the UE consent for data collection and if it is granted, it prepares the appropriate UE configuration/instruction. Then it requests AMF to deliver it to the UE via invoking Namf_Communication_N1N2MessageTransfer.

3.	If the UE is not reachable, AMF pages the UE.

4.	AMF though the RAN delivers the configuration/instruction message to the UE.

5.	UE responds the result of the request.

6.	AMF notifies the data collection entity about the status of data transfer configuration/instruction in UE.

7.	If needed (e.g., UE has not established a data transfer PDU/QoS Flow already), the UE establishes a PDU session and a QoS flow to transfer the data according to the information received from AMF.

8.	[optional] UE collects the needed standardized data specified in the request from the network based on RAN instructions if it is not collected already.

9.	UE sends the standardized data via the established PDU session/QoS Flow.

10. UPF receives the standardized data and according to the configured FARs, it forward the data to the data collection entity.

11. The data collection entity sends the request data to the consumer.

### 6.x.4	Impacts on existing services, entities and interfaces

The solution has the following impacts:

The data collection entity (a new NF or an existing NF):

-	Support service operations to be used by the consumers to request UE data

-	Support providing appropriate configuration/instruction messages for the UE for data collection

- 	Support data reception via user plane

- 	Support data content visibility (e.g., tracking history of collected data)

- 	Support data transfer controllability (e.g., configuration of data transfer including allowed data to transfer, data collection period, …)

UE:

-	Understand the configuration/instructions received the network and perform the required action (e.g., establish a PDU session or QoS Flow, start/stop/pause data collection, …)

-	Support air interface data collection and transfer via UP

AMF:

-	Support delivery of new NAS messages to the UE to for data transfer configuration information

PCF:

-	Providing PCC rules for usage monitoring of UE data collection traffic

-	Providing configuration/instruction to the UE in the UE registration process (optional)

UDM:

-	Extension of user consent to control “air interface data collection”

********** End of Changes **********