3GPP TSG-WG2 Meeting #169	S2-2504948

# Fukuoka, Japan, May 19 - 23, 2025

Source:			Ericsson

Title:			Discussion on Introducing RI-LR

Document for:			DISCUSSION

Agenda item:			19.49

Work Item / Release:			AIML_CN, TEI19

# 1. Introduction

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.273 | CR | DraftCR | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introducing RI-LR |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN, TEI19 |  |  |  |  |  | Date: |  |  | 2025-05-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | RAN3 sent LS (S2-2504525) to SA2 about looking for solution when RAN needs location data related to a UE. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - introducing new RI-LR procedure - extending Ngmlc_Location_PrivacyCheck_IDMapping service functionality for privacy check in case of RI-LR. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NG-RAN can not consume location estimate for certain use cases (etc. AIML model training in NG-RAN) |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.x (New), 8.4.2.7, 4.1a.6 (New) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

>>>> Start of Changes <<<<

### 4.1a.6	RAN Induced Location Request (RI-LR)

With a RAN Induced Location Request (RI-LR), a RAN sends a request to the PLMN for the location of the UE.

>>>> Next Change <<<<

### 6.X	5GC-RI-LR

Figure 6.x-1 illustrates a network positioning requested by the RAN to the serving PLMN for obtaining the location related information of related to a UE.

Figure 6.X-1: 5GC-RI-LR Procedure

- NG-RAN sends an NGAP POSITIONING INFORMATION REQUEST to AMF to get UE location information. The Request includes UE NGAP ID. The request may include the required LCS QoS.

- AMF identifies SUPI of the UE using UE NGAP ID and send an Ngmlc_Location_PrivacyCheck_IDMapping request to (V)GMLC to check the UE LCS privacy profile via UDM.

NOTE 1: There could be alternative solutions how LCS privacy profile stored in UDM is checked by the core network.

- [Conditional] If UE is an inbound roamer UE, then (V)GMLC goes to (H)GMLC to check the UE LCS privacy profile via UDM.

- [Conditional] If UE is an inbound roamer UE, then (H)GMLC replies to (H)GMLC with result of LCS privacy check.

- (V)GMLC replies to AMF GMLC with result of LCS privacy check. If the target UE is not allowed to be located, steps 6-12 are skipped.

- AMF may send Location Notification Invoke Request to the UE as described in step 7. of 6.1.2-1.

- UE notifies the UE user of the location request as described in step 8. of 6.1.2-1.

- The AMF selects an LMF as described in clause 5.1 or based on AMF local configuration.

- The AMF invokes the Nlmf_Location_DetermineLocation service operation towards the LMF to request the current location of the UE as described in step 12. of 6.1.2-1.

- The LMF performs one or more of the positioning procedures described in clause 6.11.1, 6.11.2, 6.11.3, and 6.11.4.

- The LMF returns the Nlmf_Location_DetermineLocation Response towards the AMF as described in step 9. of 6.1.1-1.

- AMF returns UE location information to NG-RAN.

>>>> Next Change <<<<

#### 8.4.2.7	Ngmlc_Location_PrivacyCheck_IDMapping service operation

Service operation name: Ngmlc_Location_PrivacyCheck_IDMapping

Description: Consumer NF requests privacy check and ID mapping for certain UEs involved in Ranging or request only privacy check for 5G-RI-LR.

Input, Required: UEs identifier (GPSI, Application Layer ID).

Input, Optional: Indication that ID mapping is not required.

Output, Required: Success/Failure indication, UEs identifier (Application Layer ID, GPSI).

Output, Optional: Failure Cause (in the case of failure indication provided).

>>>> End of Changes <<<<