3GPP TSG-WG SA2 Meeting #169	S2-2505249

# Fukuoka, Japan, May 19 – 23, 2025	 	(revision of S2-2502021)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 5369 | rev | 1 | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update event exposure service of PCF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Telecom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2025-05-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | For Analiytics ID = “Signalling Storm”, NWDAF is able to collect NF Context as input data from NFs as described in Table 6.22.2-2 of TS 23.288 and Event ID “Service Signalling characteristics” has been introduced for collecting this NF context data from PCF in clause 5.2.5.7.1 of TS 23.502. In this case, this event reporting of PCF is for PCF itself rather than UEs. Thus, the description in related service operations should be modified and Group IDs, UE IDs should be move from Required parameters to Optional parameters. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Update the description on Npcf_EventExposure service operation to apply NWDAF collecting NF context data from PCF. - Move Group IDs, UE IDs from Required parameters to Optional parameters. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Ambiguous and misaligned content. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.5.7.2, 5.2.5.7.3, 5.2.5.7.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * 1st change * * * *

##### 5.2.5.7.2	Npcf_EventExposure_Subscribe service operation

Service operation name: Npcf_EventExposure_Subscribe.

Description: The consumer NF uses this service operation to subscribe to or modify event reporting for a group of UE(s) or any UE accessing a combination of (DNN, S-NSSAI) or for PCF itself when the consumer is NWDAF.

Inputs, Required: NF ID,  (set of) Event ID(s) defined in clause 5.2.5.7.1, Notification Target Address (+ Notification Correlation ID) and Event Reporting Information defined in Table 4.15.1-1.

Inputs, Optional: Target of Event Reporting (Internal Group Identifier or indication that any UE accessing a combination of (DNN, S-NSSAI) is targeted, Event Filter (s) associated with each Event ID, Expiry time.

Outputs, Required: Operation execution result indication. When the subscription is accepted: Subscription Correlation ID, Expiry time (required if the subscription can be expired based on the operator's policy).

Outputs, Optional: First corresponding event report is included, if corresponding information is available (see clause 4.15.1).

The NF consumer subscribes to the event notification by invoking Npcf_EventExposure to the PCF. The PCF allocates a Subscription Correlation ID for the subscription and responds to the consumer NF with the Subscription Correlation ID. Event receiving NF ID identifies the NF that shall receive the event reporting.

##### 5.2.5.7.3	Npcf_EventExposure_Unsubscribe service operation

Service operation name: Npcf_EventExposure_Unsubscribe.

Description: The NF consumer uses this service operation to unsubscribe for a specific event for a group of UE(s) or any UE accessing a combination of (DNN, S-NSSAI) or for PCF itself when the consumer is NWDAF.

Inputs, Required: Subscription Correlation ID.

Input, Optional: None.

Outputs, Required: Operation execution result indication.

Output, Optional: None.

##### 5.2.5.7.4	Npcf_EventExposure_Notify service operation

Service operation name: Npcf_EventExposure_Notify.

Description: This service operation reports the event to the consumer that has previously subscribed either using Npcf_EventExposure_Subscribe service operation or provided as part of the Data Set Application Data and Data Subset Service Parameters stored in UDR.

Inputs, Required: Event ID, Notification Correlation Information, time stamp.

The PCF reports the SUPI and if available GPSI(s) for each of the events. The PCF may report the reports multiple events when those happen at the same time and as indicated in the time stamp.

Inputs, Optional: Corresponding UE IDs (GPSI(s), SUPI), Internal-Group-Id, Event specific information.

Outputs, Required: None.

When the PCF detects the event subscribed by the NF consumer, the PCF reports the subscribed event together with the Notification Target Address (+ Notification Correlation ID) to the Event Receiving NF.

The optional event specific parameter list provides the values that matched for generating the event notification. The parameter values to match are specified during the event subscription (see clause 6.1.3.18 of TS 23.503 [20] and clauses 5.2.5.7.2 and 5.2.5.7.1).

See clause 4.15.6.7 and 4.1.5.6.10 for details on usage of this service operation toward Application Function.

The PCF provides, in addition to the SUPI and if available GPSI(s), the Internal-Group-Id if provided as Target UE in the Service Parameters stored in UDR.

# * * * * End of changes * * * *