3GPP TSG-SA WG2 Meeting #169	S2-2505284

Fukuoka, Japan, May 19 – 23, 2025

Source:	CATT, KDDI, Toyota

Title:	KI#3 new solution for UP path adjustment based on energy related information

Document for:	Approval

Agenda Item:	20.4.1

Work Item / Release:	FS_EnergySys_Ph2 / Rel-20

Abstract of the contribution: The contribution proposes a new solution for UP path adjustment based on energy related information for KI#3.

1. Introduction

Transmitting data traffic via different UP paths may have different energy efficiency and/or energy consumption, as different UP paths involves different 5GC NFs (e.g. different PSA UPFs, I-UPFs and/or UL CL/BP UPFs) which may have different energy performance. Therefore, one possible way of improving energy efficiency and/or reducing energy consumption in the network is to adjust the UP paths for transmitting the service data flows (i.e. UP paths of the PDU Sessions) based on energy related information.

The DNAI identifies a user plane access to one or more DN(s) where applications are deployed, and different DNAIs can correspond to different UP paths for transmitting the service data flows of the applications. Thus the existing mechanism of DNAI (re)selection mechanism, e.g. AF influence on traffic routing, can be extended by taking energy related information into consideration to achieve UP path adjustment for energy efficiency and energy saving.

The energy related information to be taken into account for DNAI (re)selection can include the following:

-	total energy consumption of the UP path associated with the DNAI, i.e. sum of the energy consumption of the UPFs (e.g. PSA UPF terminating N6, I-UPF, UL CL/BP UPF) in the UP path, per PDU Session or per Service Data Flow, and

-	transmitted data volume over this UP path per PDU Session or per Service Data Flow; or

-	energy efficiency of the UP path associated with the DNAI, i.e. the transmitted data volume over this UP path divided by total energy consumption of the UP path associated with the DNAI, per PDU Session or per Service Data Flow.

How the 5GC NF (e.g. AF, PCF or the SMF) decides to perform UP path adjustment based on the above information is up to implementation.

Compared to NF (re)selection based on energy related information, UP path adjustment considers the overall energy consumption and/or energy efficiency in the CN for transmitting the service data flow, instead of a partial picture (i.e. energy consumption and/or energy efficiency of only one NF).

2. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-67.

* * * Start of Change * * * *

6.x	Solution #X: UP path adjustment based on energy related information

6.x.0	High-level solution principles

This solution proposes to enhance the AF influence on traffic routing by taking into account of energy related information for DNAI (re)selection to adjust the UP path of the PDU Session.

6.x.1	Description

This solution addresses Key Issue #3.

Transmitting data traffic via different UP paths may have different energy efficiency and/or energy consumption, as different UP paths involves different 5GC NFs (e.g. different PSA UPFs, I-UPFs and/or UL CL/BP UPFs) which may have different energy performance. Therefore, one possible way of improving energy efficiency and/or reducing energy consumption in the network is to adjust the UP paths for transmitting the service data flows (i.e. UP paths of the PDU Sessions) based on energy related information.

The DNAI identifies a user plane access to one or more DN(s) where applications are deployed, and different DNAIs can correspond to different UP paths for transmitting the service data flows of the applications. Thus the existing mechanism of DNAI (re)selection mechanism, e.g. AF influence on traffic routing, can be extended by taking energy related information into consideration to achieve UP path adjustment for energy efficiency and energy saving.

As specified in clause 5.6.7 of TS 23.501 [2] and clause 4.3.6 of TS 23.502 [3], the AF may influence the DNAI selection by providing the list of DNAI(s) used by specific application(s), or the NEF can provide the mapping between the AF-Service-Identifier and a list of DNAI(s) based on local configuration when the DNAI(s) being used by the applications are statically defined. The list of DNAI(s) is then provided to the PCF (except in the case of HR SBO where Traffic Influence information is provided directly from V-NEF to V-SMF bypassing the PCF), and the PCF may update the list of DNAI(s), e.g. based on service experience analytics per UP path as defined in TS 23.288 [x]. Finally the PCF provides the list of DNAI(s) to the SMF and the SMF determines the target DNAI, using the target DNAI to trigger PDU Session modification or establishment to add, replace or remove UPF(s) (acting as UL CL/BP or PSA) in the UP path.

To support DNAI (re)selection based on energy related information, it is proposed to enhance the AF influence on traffic routing as following:

-	The AF may decide the list of DNAI(s) for the application(s) taking into account of energy related information, and provide the selected DNAI(s) with corresponding traffic steering policy identifier(s) to the NEF/PCF.

-	The PCF may update the DNAI(s) in the PCC rule taking into account of energy related information and operator policy.

-	The SMF may determine the target DNAI based on energy related information, and local configuration and/or the PCC rule (if available).

The energy related information to be taken into account for DNAI (re)selection can include the following:

-	total energy consumption of the UP path associated with the DNAI, i.e. sum of the energy consumption of each UPF (including PSA UPF terminating N6, I-UPF and UL CL/BP UPF if any) involved in the UP path, per PDU Session or per Service Data Flow, and

-	transmitted data volume over this UP path per PDU Session or per Service Data Flow; or

-	energy efficiency of the UP path associated with the DNAI, i.e. the transmitted data volume over this UP path divided by total energy consumption of this UP path associated with the DNAI, per PDU Session or per Service Data Flow.

The 5GC NF (i.e. AF, PCF or the SMF) can obtain the above information based on the notifications received from the EIF, i.e. Energy Consumption information in the CN at per UE per PDU Session and/or per UE per application (i.e. Service Data Flow) granularity, and the notifications received from the UPF(s) about the data volume.

NOTE:	How the 5GC NF (i.e. AF, PCF or the SMF) decides to perform UP path adjustment based on the above information is up to implementation.

Compared to NF (re)selection based on energy related information, UP path adjustment considers the overall energy consumption and/or energy efficiency in the CN for transmitting the service data flow, instead of considering energy consumption and/or energy efficiency of only one NF.

6.x.2	Procedures

The procedures in clause 4.3.6.2 and 4.3.6.4 of TS 23.502 [3] are enhanced as follows:

-	In step 0-1 of figure 4.3.6.2-1 and step 1-4 of figure 4.3.6.4-1, the AF may decide the list of DNAI(s) for the application(s) taking into account of energy related information which is exposed by the NEF, and provide the list of DNAI(s) to the NEF/PCF.

-	In step 5 of figure 4.3.6.2-1 and figure 4.3.6.4-1, the PCF may update the list of DNAI(s) in PCC rule taking into account of energy related information.

-	In step 6 of figure 4.3.6.2-1 and step 5 of figure 4.3.6.4-1, the SMF may determine the target DNAI taking into account of energy related information.

6.x.3	Impacts on services, entities and interfaces

AF:

-	Decides the list of DNAI(s) for the application(s) taking into account of energy related information.

PCF:

-	Updates the list of DNAI(s) taking into account of energy related information.

SMF:

-	Determines the target DNAI taking into account of energy related information.

* * * End of Change * * * *