3GPP TSG-SA2 Meeting #169	S2-2505432

Fukuoka, Japan, 19 – 23 May 2025

Source:	Nokia, Nokia Shanghai Bell

Title:	Solution to KI#5 Sensing Service Exposure

Document for:	Approval

Agenda Item:	20.2.1

Work Item / Release:	FS_Sensing_ARC / Rel-20

Abstract of the contribution: The contribution proposes a solution to KI#5, for Sensing Result Exposure

# Introduction

This contribution proposes the interface for the exposure of the sensing services.

# Discussion

There are various applications that could benefit by sensing services provided by an ISAC system. The Network Exposure Function (NEF) supports exposure of network capabilities to external application functions and shall be enhanced to also support exposing the ISAC capabilities. For managing the sensing service requests from different sensing consumers (also referred to as sensing clients in this document), a new core network function, Sensing Function (SF) needs to be introduced. The SF will interface with various network functions and sensing entities to perform sensing operation, calculate sensing result and send the result to the sensing clients. For an external/untrusted AF acting as sensing client, the sensing result shall be exposed via the NEF.

# Proposal

It is proposed update TR 23.700-14 v0.1.0 as below.

FIRST CHANGE

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Solutions | KI#1 | KI#2 | KI#3 | KI#4 | KI#5 | KI#6 |
| #X |  |  |  |  | X |  |

Second CHANGE (All new)

## 6.X	Solution #X: Sensing service request and result exposure

### 6.X.1	Description

For managing the sensing service requests from different sensing result consumers (also referred to as sensing clients in this document), a new core network function, Sensing Function (SF) is introduced. The SF can be a dedicated Network Function or a logical function within an existing NF of the core network. The goals of the SF is to provide one or more of the following capabilities:

-	Detect Objects according to Sensing Requirements (e.g., target area of sensing, accuracy etc.)

-	Determine one or more of following characteristics w.r.t. the detected object:

-	Location of the detected object

-	Velocity of the detected object

-	Attributes of the detected objects, e.g., type of object, size/shape of object, orientation of object etc.

-	Determine Point Cloud information

-	With attributes as an integrated whole including size, shape, velocity, etc.

-	With each point's respective attributes, e.g., time/movements frequency, velocity, etc.

-	Environment sensing (e.g. rain detection)

NOTE 1:	The  detected Object can be a passive object (e.g. a human or an animal or a drone/vehicle without UE on board or any other passive objects etc.) or an object with UE onboard.

Editor’s Note:	All the above capabilities are dependent on RAN work progress and RAN study outcome.

The SF peforms the following functionalities:

-	Receive different types of sensing request from a sensing client (e.g., Application Function (AF) or other NFs)

-	Peforms sensing service request authorization

-	Peforms privacy checks for sensing data privacy as well as the privacy of UEs/users that are potentially involved/impacted by sensing or are present in the target area for sensing

-	Determines the sensing method, the sensing entities to be involved for a sensing service request and the corresponding configuration to perform the sensing operation

-	Triggering sensing measurement data collection through the selected sensing entities

-	Result calculation and exposure of result to the sensing client

Based on deployment scenarios and depending on the sensing request types, use cases and QoS requirements, there could be the following sensing result exposure types:

-	Whether in a certain area an object (e.g., drone) is detected (e.g., intrusion deection)

-	Sensing information identified in a specific area, which can be described for instance in the form of point cloud, or in the form of an object map.

-	Sensing of environment conditions (e.g., rain).

The sensing request for any of the above may be a one time request or a subscription-based request for periodic or event based result notification. The events can be defined by the sensing client so as to trigger a specific sensing response. Examples of these events can include:

-	Sensing results are sent in the case that a specific type of object (e.g., drone) is detected in the target sensing area.

-	Sensing results are sent in the case that mobility is detected in the target sensing area

-	Sensing results are sent in that the velocity of an object in the target sensing area is above a defined speed threshold.

### 6.X.2	Procedures

#### 6.X.2.1	Sensing service procedure for on demand, periodic and event based result exposure

Figure 6.x.2.2-1: Sensing service request authorization and result exposure

Figure 6.x.2.2-1 above shows the information flow for sensing service request and result exposure. The sensing service consumer can be a trusted/untrusted AF or an NF (e.g. NWDAF, LMF etc). In case of untrusted AF the request is forwarded via the NEF.

1.	The sensing client (e.g. AF) sends a sensing service request to the SF. For untrusted AF the request is routed via the NEF. For the untrusted AF case, the NEF may perform AF authorization before forwarding the request to the SF. The sensing service request can be for a one-time (i.e. on demand) sensing result or it can be a subscription based request for periodic or event based sensing result notification.

Depending on the the sensing use case scenario and the application one or more of the following information elements can be provided in the sensing service request:

a)	Sensing service type:	e.g., detection of an object type, tracking of an object, detection of environment conditions etc.

b)	sensing target area; indicates the location where the sensing service needs to be performed and may contain:

-	Universal Geographical Area Description (GAD) as specified in TS 23.032 [xx]

-	UE ID(s); if the request is to perform sensing in the proximity of a UE (i.e. the target sensing area is determined by UE's current location) then the UE ID is included in the request

c)	sensing duration; indicates the time period over which the sensing is to be performed e.g., start time and end time or total duration.

f)	Sensing configuration/filter information

-	target object type (e.g., drone, car, human etc): indicates the type of objects that shall be detected in the specific sensing service request

-	target object mobility profile (e.g., static, low mobility etc): indicates whether fixed objects or objects with a specific mobility profile shall be detected in the specific sensing service request

-	Environment of sensing (e.g. indoor, outdoor etc.)

e)	Requested sensing QoS parameters; The QoS parameters include positioning accuracy, velocity accuracy and confidence level for the detected object(s), sensing range resolution for object detection, maximum latency to obtain the sensing result, missed detection probability, false alarm probability etc (see also TS 22.137 [x]).

f)	Sensing results configuration:

-	expected sensing output (e.g. detected object, location, velocity etc)

periodicity of result reporting; shall be provided for sensing request with periodic result notification

-	subscribed events for sensing result reporting; indicates when the SF shall trigger the provission of sensing results to the sensing client.

-	sensing result type; indicates the type of sensing results that the sensing client requests, which can be a report detected objects or report point cloud information etc.

-	Indicate specific list of features to be reported for the requested sensing result type.

The appropriate SF is discovered through the NRF based on one or more of the following criteria:

-	The expected sensing QoS

-	The target sensing area

-	The SF capabilities required (e.g., sensing request type, method, processing capability etc.)

-	By regulatory requirements e.g. the SF instance for emergency/public safety can be pre-configured.

2-3.	The SF peforms Sensing service request authorization and may also perform necessary privacy checks. If the target sensing area is defined by the target UE's current location then the LCS procedure as defined in TS 23.273 [x] may be performed before performing sensing operation.

For the On Demand Sensing scenario:

4.	The SF then determines the appropriate sensing method, discovers and selects the corresponding sensing entities (i.e. sensing Tx, Rx) and configures them to start sensing session and report the measurement data. The SF calculates the result based on the measurement data collected from the sensing entities.

5.	The SF provides the sensing result to the sensing client.

For the periodic or event based Sensing scenario:

4.	The SF then determines the appropriate sensing method, discovers and selects the corresponding sensing entities (i.e. sensing Tx, Rx) and configures them for sensing operation.

5.	The SF sends sensing service response with a positive acknowledgement to the subscriotion from sensing client (in step 1) for the periodic or event based sensing result notification. The sensing service response may also contain an immediate sensing result, if available.

6.	At the configured periodicity or at the detection of the configured event(s), sensing operation is performed, the sensing data measurements are collected and reported to SF. The SF may perform privacy checks at this point again. The SF calculates the sensing result based on the measurement data collected from the sensing entities and notifies the sensing result to the sensing client. This step is performed at the subscribed periodicity or  each time the subscribed event is detected.

NOTE 1:	The details of how the SF determines the sensing method, how the SF selects and configures the sensing entities and how the SF collects the measurement data from the sensing entities are not in scope of this solutions and assumed to be described in other solutions of this TR.

Editor’s Note:	Configuring the sensing entities and sensing measurement data collection is dependent on RAN study outcome

#### 6.X.2.3	Sensing request authorization and privacy check

Once a sensing service request is received by the SF, the SF shall perform authorization and privacy checks depending on the type of the sensing service request, the target sensing area, the target objects sensing.

The privacy and policy checks may be performed based on the provisioned privacy profiles and policies, that indicates whether to allow or disallow a sensing service or what information can be exposed in sensing result. The following types of profiles may be used:

-	Location-aware sensing profile that indicates whether to allow or not allow a sensing request in a specific area.

-	UE sensing privacy indication that indicates whether to allows or not allow a sensing request for a specific UE and/or whether a UE can be involved in the sensing procedure.

-	Object-aware sensing profile that indicates whether to allows or not allow a sensing request for a specific object.

-	Information that shall be obfuscated when sending sensing result

The above profiles can be used by the SF a) to configure a sensing procedure or b) to decide whether and what to expose the sensing results for a specific sensing service request.

### 6.X.3	Impacts on services, entities and interfaces

SF (New):

-	Receive sensing service request from a sensing client

-	Peforms sensing service request authorization

-	Peforms privacy checks for sensing data privacy as well as the privacy of UEs/users that are potentially involved/impacted by sensing or are present in the target area for sensing

-	Determines the sensing method, the sensing entities to be involved for a sensing service request and the corresponding configuration to perform the sensing operation

-	Triggering sensing measurement data collection through the selected sensing entities

-	Result calculation based on sensing measurement data collected from the sensing entities and exposure of sensing result to the sensing client

NEF:

-	Receive sensing service request from an untrusted AF (sensing client) and forward the request to an SF after AF authorization

-	Receive sensing result from SF and forward the result to the untrusted AF

AF:

-	Send sensing service request and receive sensing result

End of Changes