3GPP TSG-SA WG2 Meeting #169	S2-2505463

# 19-23 May, 2025, Fukuoka , Japan	(Revision of S2-2505204)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.228 | CR | 1637 | rev | 2 | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on PSI configuration on UE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile, Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RTC_Ph2 |  |  |  |  |  | Date: |  |  | 2025-04-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | When a PSI is used, e.g. when establishing a standalone IMS DC session with bootstrap DC, it is needed to clarify that the PSI shall be configured in the ME or UICC. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add clarification that a PSI shall be configured in the ME or UICC. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is not clear whether and how PSI is configured in the UE. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | AC.10.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

### AC.10.2.1	Originating Standalone Bootstrap DC Setup using PSI

The originating standalone Bootstrap DC setup using PSI is triggered by a UE for downloading application list and/or applications from DCSF. In this procedure, a PSI is configured in the UE for that purpose. The UE could be configured by the HPLMN either via Device Management, via the UICC or via UE implementation with a specific PSI used for standalone bootstrap DC sessions.

After the standalone bootstrap DC is established, the UE downloads the application list and/or applications via the bootstrap DC. The list of application information is used for selection, downloading, and/or initiating standalone application DC in a subsequent IMS data channel session. The UE terminates the standalone IMS data channel session when the session timer expires.

Whether and when to initiate the standalone Bootstrap DC setup using PSI is up to UE implementation.

Figure AC.10.2.1-1depicts a signalling flow diagram for originating standalone bootstrap data channel setup using PSI.

AC.10.2.1-1: Originating Standalone Bootstrap DC Setup using PSI

1.	UE#1 sends the SIP INVITE request with an initial SDP offer to the IMS AS, through P-CSCF and S-CSCF in the originating network. The Request URI of the SIP INVITE message is set to the configured specific PSI indicating standalone bootstrap DC. The initial SDP offer includes only the bootstrap data channel establishment request with bootstrap DC stream ID(s). The SIP precondition is not needed.

NOTE 1:	The PSI is pre-configured in the UE#1. How and when the PSI is pre-configured is left to implementation.

NOTE 2:	UE#1 can perform step 1 at any time before initiating a standalone application data channel, such as, immediately after completing the SIP registration procedure for downloading the list of application information and applications in order to save the duration of consequence (standalone) IMS DC session establishment.

2-4.	Steps 2-4 of clause AC.7.1 apply with the following difference:

-	The IMS AS rejects a session initiated by a UE not subscribed for a standalone IMS data channel session.

5.	Since the SessionEstablishmentRequestEvent indicates that served user is offered only a local bootstrap media, the DCSF, based on the value of the Called ID (i.e. the specific PSI) and the content of the initial SDP offer, determines that the UE#1 requests to establish a standalone bootstrap DC and reserves only originating side MDC1 media information, which is used to receive the UE request for application downloading from the MF.

6.	The DCSF invokes the Nimsas_MediaControl_MediaInstruction (Session ID, Media Instruction Set) service operation based on its policies instructing the IMS AS how to set up bootstrap data channel with the MF for originating side. The MediaInstructionSet provided by the DSCF, includes its MDC1 media endpoint addresses created in step 5, DC Stream ID, and the replacement HTTP URL representing the application list offered via the MDC1 interface.

7.	Step 7 of clause AC.7.1 applies.

8.	The IMS AS invokes Nmf_MRM_Create (List of Media Termination Descriptors) service operation to instruct the MF to allocate required data channel media resources for originating side only. The IMS AS requests creation of only one Media Termination which representing the local bootstrap media to be terminated.

9-10.	Steps 9-10 of clause AC.7.1 apply.

11.	The IMS AS does not forward the SIP INVITE request based on the specific Request URI (i.e. PSI) and sends the SIP 18X response with the SDP answer through S-CSCF and P-CSCF to UE#1.

12-13.	The standalone IMS data channel session with only bootstrap data channel is answered and bootstrap data channel between the originating MF and UE#1 is established.

14.	The UE#1 sends application request messages to the MF to request a data channel application or an application list if multiple DC applications are available, via the established bootstrap data channel. The MF forwards the message to the received media point of DCSF. The DCSF provides the application list and proper data channel applications further to the UE#1 based on their data channel capabilities and their choices through MF.

15-16.	If the established bootstrap DC is not used for a period of time (expiry of the session timer), UE#1 sends SIP BYE to release the bootstrap DC.

# * * * End of changes * * * *