SA WG2 Meeting #169	S2-2505479

# 7 - 11 April 2025, Fukuoka, Japan

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 6277 | rev | 1 | Current version: | 18.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Alignment of PDU set information |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | XRM |  |  |  |  |  | Date: |  |  | 2024-11-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | As stated in TS 26.522, SA4 has agreed for PDU set marking , the “Number of PDUs in the PDU SET[NPDS]” field could be added, see below from TS 26.522 clause 4.2.4: Number of PDUs in the PDU Set [NPDS] (16 bits): The number of PDUs within the PDU Set indicates the total number of PDUs belonging to the same PDU Set. This field is optional and subject to an SDP signaling offer/answer negotiation, where the RTP sender may indicate whether it will provide the number of PDUs within the PDU Set for that RTP stream. If enabled, but the RTP sender is not able to determine the Number of PDUs in the PDU Set, it shall set the value to 0 in all PDUs of that PDU Set. It is recommended to add the Number of PDUs in the PDU Set field when the PDU Set Size field is present. This CR proposes to do alignment change. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add number of PDUs in the PDU Set in the PDU set information |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misalignment between SA2 and SA4. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.37.5.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Change * * *

5.37.5.2	PDU Set Information and Identification

To support PDU Set based QoS handling, the PSA UPF identifies PDUs that belong to a PDU Set and determines the below PDU Set Information and sends it to the NG-RAN in the GTP-U header. The PDU Set information is used by the NG-RAN for PDU Set based QoS handling as described above.

The PDU Set Information comprises:

-	PDU Set Sequence Number.

-	Indication of End PDU of the PDU Set.

-	PDU Sequence Number within a PDU Set.

-	PDU Set Size in bytes.

- 	[Optional] Number of PDUs in a PDU set, which indicates the total number of PDUs belonging to the same PDU Set.

-	PDU Set Importance, which identifies the relative importance of a PDU Set compared to other PDU Sets within a QoS Flow.

The NG-RAN may use the Priority Level (see clause 5.7.3.3) across QoS Flows and PDU Set Importance within a QoS Flow for PDU Set level packet discarding in presence of congestion.

NOTE 1:	In addition to considering the PDU Set Importance within a QoS Flow, NG-RAN could also consider the relative PDU Set Importance across QoS Flows of the same Priority Level when determining which PDU Set needs to be discarded, which is up to implementation and configuration of operator.

NOTE 2:	The PDU Set Information can be different for different PDU Sets within a QoS Flow.

If the NG-RAN has provided a PDU Set based handling support Indication indicating that PDU Set handling is supported and a Protocol Description together with 1) a PSIHI and/or 2) PSDB and PSER is included in the PCC rule, the SMF instructs PSA UPF to perform PDU Set marking and may provide the PSA UPF the DL Protocol Description used by the service data flow. The DL Protocol Description may be received in the PCC rule, based on information provided by the AF or by PCF local policies as described in clause 5.37.5.1.

PSA UPF can identify the PDU Set Information using the DL Protocol Description and the received transport protocol headers and payload or using implementation specific means. The details of the RTP/SRTP headers, header extensions and/or payloads used to identify PDU Set Information are defined in TS 26.522 [179].

For each DL PDU received on N6 for which PDU Set based QoS handling is indicated from the SMF, the PSA UPF applies the rules for PDU Set identification and provides the available PDU Set Information to the RAN in the GTP-U header.

NOTE 3:	The PSA UPF is expected to assign a unique PDU Set Sequence Number in the GTP-U header to each

* * * End of Change * * *