3GPP TSG-SA2 Meeting #169	S2-2505503

Fukuoka, Japan, May 19 – 23, 2025                                                       (was S2-2505158)

Source:	Samsung

Title:	KI #1: New Sol for UP based IMS voice call over NB-IoT(Geo)

Document for:	Approval

Agenda Item:	20.1.1

Work Item / Release:	FS_5GSAT_Ph4_ARC / Rel-20

Abstract of the contribution: The contribution proposes a UP based solution to KI#1: Support of IMS voice call over NB-IoT NTN via GEO satellite connecting to EPC.

Discussion

The proposed solution is based on the NB-IoT access. And so UE has some limitation as follows:

- Per TS 23.401 clause 4.7.2.1, Dedicated bearers are not supported over NB-IoT. The PDN GW uses the RAT Type to ensure that no dedicated bearers are active when the UE is accessing over NB-IoT.

- Per TS 23.401 clause 4.3.5.9, In this Release of the specifications, inter-RAT mobility to/from the NB-IoT RAT is not supported, and GBR bearers are not supported in the NB-IoT RAT. Hence the UE should not include the "Voice domain preference and UE's usage setting" IE when sending an Attach Request or Tracking Area Update Request on the NB-IoT RAT.

- Per TS 36.331 clause 4.7.2.1, A NB-IoT UE only supports 0, 1 or 2 DRBs, depending on its capability. A NB-IoT UE that only supports the Control Plane CIoT EPS optimisation does not need to support any DRBs and associated procedures.

Observation 1: the solution based on the NB-IoT should be made with the restriction that neither dedicated nor GBR bearer is allowed, but  default EPS bearer only can be used.

So the default EPS bearer for IMS APN is used IMS media data as well as IMS signalling even if GBR traffic is requested for resource reservation during IMS voice call setup for UE attached by NB-IoT(GEO).

Observation 2:  default EPS bearer is used for both IMS signalling and IMS media data. And, so even if GBR traffic is negotiated during IMS voice call setup, resource reservation in EPS should be skipped or, if required, performed bearer modification on the non-GBR default EPS bearer.

As NB-IoT(GEO) link has not much enough bandwidth and long propagation delay to support voice call service, we need to lessen the voice call setup time and voice data size as much as possible. so we need mimize switching delay to send/receive data from Idle mode, and number of signalling transactions b/w UE and NW during voice call setup, and data size transmitted b/w UE and RAN.

Observation 3:  In order to lessen voice call setup time for UE camping on NB-IoT(GEO) RAT and voice data size on the air, we should apply that 1) resource reservation in EPS should be skipped or, if required, performed before IMS voice call setup, 2) Early Data Transmission(or CIoT EPS optimisation), 3) signaling compression of IMS SIP message, ROHC, etc.

Proposal

It is proposed to add this solution for KI#1 in the TR 23.700-19.

Start of 1st Change

6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |
| --- | --- | --- | --- | --- |
| Solutions | #1 | #2 | #3 | #4 |
| XX | x | x |  |  |

Start of 2nd Change (all new text）

6.X	Solution #X: IMS voice call over NB-IoT NTN via GEO satellite using a default EPS bearer for both IMS signalling and voice data

6.X.1	High-level solution Principles

This solution aims to resolve Key Issue #1, " Support of IMS voice call over NB-IoT NTN via GEO satellite connecting to EPC " with the following key principles

-	The UE negotiates with MME on its capability of voice call support over NB-IoT NTN via GEO satellite.

- 	The default EPS bearer is used for both IMS signalling and voice data transmission.

-     Bearer modification procedure for the QoS update or resource reservation is skipped or pre-initiated before the call setup in case that voice call would be setup over NB-IoT NTN via GEO satellite.

6.X.2	Description

In this proposal, it  is assumed to provide IMS voice services via the default EPS bearer. i.e. IMS signalling and IMS voice data are served via the default EPS bearer, which can be updated its non-GBR QoS characteristics if required. Such QoS update is performed before IMS call setup procedure if UE is attached and IMS registered by NB-IoT(GEO).

6.X.3	Procedures

6.X.3.1 Attach or TAU procedure negotiating Voice call support over NB-IoT(GEO):

On camping NB-IoT(GEO), UE who wants to use voice call service over the NB-IoT(GEO), negotiate its capability of voice call support over NB-IoT(GEO) .

Moreover, UE to use voice call service over the NB-IoT(GEO) needs to include the IE "Voice domain preference and UE's usage setting" as the voice domain preference = “IMS PS Voice only” and UE’s usage setting = voice-centric.

For IMS service, an IMS dedicated APN is assumed, which is well-known or pre-configured for voice call over NB-IoT(GEO). So the UE should execute UE requested PDN connectivity procedure for the IMS dedicated APN.

However, due to restriction of NB-IoT RAT that dedicated EPS bearer is not used and GBR bearer is not used, we assume the default EPS bearer is used for both IMS signalling and voice data.

6.X.3.2	Default EPS bearer for both IMS signalling and voice data

For IMS service,

we assume the default EPS bearer is used for both IMS signalling and voice data.

Editor’s NOTE It is FFS whether or how to differentiate QoS/priority between IMS signalling and voice data.

So in case that UE registers IMS as access type as NB-IoT(GEO), if the voice call is setup b/w caller and callee, the resource reservation in EPS for voice call should be handled specifically;

P-CSCF provides an indication of a voice call over NB-IoT(GEO) to PCRF, so that PCRF or P-GW neglects the request and not allocate dedicated & GBR bearer for the voice call traffic but if required, performs bearer modification procedure for the default EPS bearer to upgrade its QoS even as a non-GBR bearer.

Moreover, if the expected resource for the voice call over NB-IoT(GEO) is almost static and need upgrade QoS of the default EPS bearer for the voice call traffic, in order to lessen the call setup time such bearer modification procedure can be initiated before IMS call setup procedure.

### 6.X.4	Impacts on services, entities and interfaces

UE:

- Support and capability of voice call support over NB-IoT(GEO).

- Set "Voice domain preference and UE's usage setting" when camping on NB-IoT(GEO).

- Use default EPS bearer for both  IMS signalling and IMS voice data over NB-IoT(GEO) access.

MME:

- Support and capability of voice call support over NB-IoT(GEO).

- Initiate bearer modification procedure for resource reservation/QoS update for the default EPS bearer before IMS call setup.

P-GW:

- allocate IMS signalling and voice mapped to the default EPS bearer.

- neglect the GBR characteristics of IMS voice call data traffic for UE if provided, and perform bearer modification procedure for resource reservation/QoS update for non-GBR characteristics or skip the bearer modification procedure.

- Initiate bearer modification procedure for resource reservation/QoS update for non-GBR characteristics.of the default EPS bearer before IMS call setup.

PCRF:

- let IMS signalling and voice mapped to the default EPS bearer.

- neglect the GBR characteristics of IMS voice call data traffic for UE if provided, and trigger bearer modification procedure for resource reservation/QoS update for non-GBR characteristics or provide indication of voice call over NB-IoT(GEO) to P-GW.

- support bearer modification procedure for resource reservation/QoS update for the default EPS bearer before IMS call setup.

P-CSCF:

- Provide an indication of a voice call over NB-IoT(GEO) to PCRF.

End of Change