SA WG2 Meeting #169	S2-2505781

19 – 23 April 2025, Fukuoka, Japan

Title:	KI#1 solution using the generic architecture for collecting and exposing the collected UE data

Source:	Ericsson

Document for:	Approval

Agenda Item:	20.3.1

Work Item / Release:	FS_AIML_CN_Ph2/Rel-20

Abstract of the contribution: This contribution proposes a solution for KI#1 for FS_AIML_CN_Ph2.

# 1.		Introduction

This solution describes how to address KI#1: Data collection over user plane for UE side model training, reusing the generic data collection procedure from the UE application using an intermediate Application Function that is described in TS 26.531 and TS 23.288 clause 6.2.8.

In order to use the Data collection architecture, new EventIDs to allow collection of UE data that is needed for training ML Models at the UE training center are defined.

Based on the RAN requirements and the WT#1 description the following solution is proposed:

- The intermediate Application Function part of the generic architecture for data collection is only located in the MNO domain.

  - This is to comply with requirements for “the MNO has full control of the standardized data collection transfer process and can manage data transfer to the server for UE-side data collection, without the need of SLA for this purpose. This includes initiating, terminating, and fully managing data transfer”,

- Given that DCAF in the MNO domain initiates and terminates the UE data collection, the MNO has full visibility for standardized data.

- The information security model defined in TS 26.531, under SA4 scope, is understood to cover the following, may need validation with SA4:

  - The data collected is secured and data integrity and confidentiality for that data is ensured.

The solution proposed in this contribution is on high level, while additional aspects need to be covered, as such those are documented in the clause “Aspects for further consideration”.

# 2. Text Proposal

It is proposed to agree the following solution to be part of TR 23.700-04.

************ 1st change  **************

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[3]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[4]	3GPP TS 23.503: "Policy and Charging Control Framework for the 5G System; Stage 2".

[5]	3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services".

[6]	3GPP TS 23.273: " 5G System (5GS) Location Services (LCS); Stage 2".

[7]	3GPP TS 24.572: " 5G System (5GS); User plane Location Services (LCS) protocols and procedures; Stage 3".

[8]	IETF RFC 793: "TRANSMISSION CONTROL PROTOCOL".

[9]	IETF RFC 9000: "QUIC: A UDP-Based Multiplexed and Secure Transport".

[X]	3GPP TS 26.531: "Data Collection and Reporting; General Description and Architecture".

************ next change  **************

## 6.0	Mapping of Solutions to Key Issues and Use Cases

Table 6.0-1: Mapping of Solutions to Key Issues and use cases

|  | Key Issues |  | Use Cases (optional) |  |
| --- | --- | --- | --- | --- |
| Solutions | <Key Issue #1> | <Key Issue #2> | <Use Case #1> | <Use Case #2> |
| #1 | x |  |  |  |
| #2 | x |  |  |  |
| #3 | x |  |  |  |
| #4 | x |  |  |  |
| #N | x |  |  |  |

************ next change (all text is new) ************

## 6.x	Solution #N: User Plane solution for Data collection for UE-side model training

### 6.x.1	High-level solution principles

The solution follows the following principles:

-	Reuses the generic architecture for collecting and exposing the collected UE data, via an intermediate Application Function defined in TS 23.288 [5] clause 6.2.8 and in SA4 TS 26.531 [x].

- 	Reuses the generic architecture for provides events to the event consumer using the event exposure service, defined in TS 23.288 [5] 6.2.8 and in SA4 TS 26.531[x]. This allows for the MNO controllability and MNO visibility of the data with minor changes in the 3GPP specifications.

-	The intermediate Application Function, i.e. DCAF is always located in the operator´s domain.

The UE is configured by DCAF via user plane and the UE reports to DCAF the collected data via user plane. The DCAF configures the UE with the DCAF address or FQDN via NAS.

- 	Differentiation of the traffic for model training for charging purposes, traffic differentiation is achieved either by associating this traffic to a PDU Session to a dedicated DNN and S-NSSAI or if this traffic runs in a PDU Session that is not dedicated to the traffic for model training using PCC Rules that assigns a separate Charging Key to the traffic from/to the UE from/to the DCAF.

Editor's Note: It is FFS Whether this solution fulfils the architectural requirements to study UP Data Collection (for Option 2) are as documented in RAN LSs RP-243316 and RP-242389.

Editor's Note: Whether this solution addresses option 1b or option 2 needs to be checked with RAN before reaching conclusions.

NOTE 1: User consent aspects are out of the scope of this solution

NOTE 2: The interface between the client in the UE and the chipset is not covered in this solution as it is out of the scope of 3GPP.

### 6.x.2	Description

This solution is addressing KI#1: Data collection over user plane for UE side model training.

This solution reuses the generic architecture for collecting and exposing the collected UE data, via an intermediate Application Function that provides events to the event consumer using the event exposure service, defined in TS 23.288 [5] 6.2.8 and in SA4 TS 26.531 [x] with the following considerations:

- The intermediate Application Function, i.e. DCAF:

  - is always located in the MNO to fulfil the requirements on visibility and controllability of the data collection procedure.

  - Receives configuration data from the UE training center per each of the EventIDs.

  - Receives request to collect input data from the UE for model training, via existing Naf_EventExposure service.

  - Decides whether and when to initiate and terminate the data collection from the UE, e.g. the DCAF may use e.g. PDTQ to determine when to start or whether to terminate data collection or the RAN, or take decisions when requests from multiple UE training centres are received.

- The UE training center:

  - may be located in the trusted MNO domain or in an untrusted MNO domain.

  - provides configuration information to DCAF.

  - Triggers the request for data collection towards the DCAF.

  - Checks user consent before requesting data from DCAF.

- The UE:

  - request configuration information from DCAF.

  - performs data collection and provides it to the DCAF as indicated in the configuration information provided per EventID, as such the UE (e.g. DCAF client would need enhancements to report the EventID)

- How the data collected is secured and how data integrity and confidentiality for that data is ensured is described in TS 26.531 [x], clause 4.5 Information security model, whether additional requirements are foreseen will be discussed by SA3.

- It is assumed that the UE receives configuration information from DCAF, then the UE requests resources and configuration information from gNB, as such no requirements exists in SA2, it is rather between RAN and the UE when it comes to “SA2 can assume that the gNB is involved in providing radio measurement configuration (if needed) for beam management use case and LMF is involved in providing PRS measurement configuration (if needed).

The figure below schematically shows the solution for data collection from the UE via user plane, based on the reference architecture defined in TS 26.531 [x] clause 4.2. Note that the interface between the UE Application and the Direct Data collection that is defined in TS 26.531 [x]  is internal to the UE, as such this solution considers the UE only with no distinction between both. Interactions at R8 reference point are out the scope of 3GPP, according to 26.531 [x] clause 4.3, therefore not shown in the figure below. R8 interface is not used in this solution.

Figure 6.x.2_1. Data Collection from the UE and transfer to the UE training centre for Model training

### 6.x.3	Procedure for collecting UE data and reporting to the UE training center

This procedure follows TS 23.288 [5] clause 6.2.8 ad 26.531 [x] clause and for data collection from the UE and reporting the data collected to the UE training center. Figure 6.x.3_1 shows how the procedure is applied for collecting input data for training a ML Model.

Figure 6.x.3_1: UE training center triggers data collection from the UE, UE provisioning and UE reporting via user plane.

- The UE training center determines that data collection is needed for training an AI/ML Model, then discovers and selects the DCAF that provides data collection (based on the DCAF profiles registered in NRF). The DCAF profile is extended to indicate the new EventIDs that DCAF supports include CSI compression data, CSI prediction data, Beam management, or UE Positioning or combinations of those.

- The UE training center provides configuration information per Event ID to the DCAF, this includes information to enable the UE to collect the input data per Event ID, how data is to be sampled by the UE (e.g., sampling frequency, location filter) and how to report the required input data for ML Model training to the UE training center (e.g., reporting probability, reporting frequency, reporting format, data packaging strategy). This information is defined in TS 26.531 [x]. The DCAF stores the provisioning data until it decides to start the data collection.

- The UE training center subscribes to the DCAF for UE data collection, possibly via NEF, using Naf_EventExposure Subscribe including the Target for Event Reporting (i.e. any UE), both Event ID(s), Event reporting information including the type of measurements to be collected, Event Filters such as UE location (to enable to collect data in certain areas only) listed in clause 5.2.19.2.1 in TS 23.502 [3]. The DCAF stores the provisioning data until the UE needs to train the model. The DCAF accepts request for measurements only to those UEs that are in the scope of the UE Training center, e.g. mapping an Application Id provided by the UE training center into a list of SUPIs in the scope of the UE training center and support providing data for model training, and those UEs that support ML Model training and reporting input data.

- The DCAF contacts the AMF covering that area of interest to find the list of SUPIs registered in that area using Namf_EventExposure including EventID “UEs in an area of interest”, and include as Event Filter those UEs that support AI/ML Model training and reporting input data. Note that the UE reports its UE capabilities in the Registration procedure where the support for this feature is indicated.

- The DCAF decides when to provision its own address or FQDN to those UEs provided by AMF that consent to data collection, via Namf_CommunicationN1N2MessageTransfer. The provisioning of the DCAF address or FQDN is an indication that the UE requests configuration information to start data reporting.

- The UE stores the DCAF address or FQDN.

- The UE sends a confirmation that the DCAF address or FQDN is stored in the UE using Namf_CommunicationN1N2MessageNotify. The provisioning of the DCAF address is used by the UE as an indication that to establish a user plane connection to start the data collection to train the model.

- The UE establishes a user plane connection to retrieve the provisioning information and report collected measurements to the DCAF. The user plane connection may be established via an existing PDU Session or the UE may establish a PDU Session, this is decided by the UE using the provisioned URSP Rules by the PCF.

- The DCAF authorizes the UE request, as such the UE that requests the establishment to a user plane connection is matched with those UEs that the AMF provided in step 4.

- The DCAF provides provisioning information to indicate to the UE to report measurements, the DCAF may include a time if the data reporting to DCAF needs to take place at a certain time, i.e when the timer expires.

- When the UE receives the provisioning information from DCAF, the UE starts data collection and reporting to DCAF, when the timer expires.

NOTE: 	The UE requests the gNB for radio resources, then gNB provides reference signals and necessary configuration to the UE for data collection. When the UE gets radio resources and configuration information then the UE can start measurement. The UE reports to DCAF according to the instruction from DCAF.

Editor’s note: The UE and the gNB behaviour would need to be checked with RAN2

- The DCAF verifies that the data collected is according to the data requested by the UE training center, if it is so the DCAF may perform grouping of the UE data before reporting to the UE training center the collected data.

- The UE training center trains the ML Model with the input data provided by DCAF.

Other aspects:

- 	How to differentiate the traffic for UE data collection from the UE regular traffic in the 5GC for charging purposes. The following options are possible, a) the traffic for data collection is sent over the same PDU Session as traffic not related to data collection, as such not sent to DCAF, a PCC Rule can be used to assign a different charging key to the traffic sent to/from the UE to the DCAF, b) a URSP Rules associates the traffic sent to the DCAF to a dedicated DNN,S-NSSAI.

### 6.x.4	Aspects for further consideration

- What information is sent in the provisioning request to the UE.

- RAN requirements on User data privacy and anonymity are FFS.

- Relation with the LCS privacy profile when requesting positioning data.

### 6.x.5	Impacts on services, entities and interfaces

DCAF:

- Extensions to the AF exposure service with new EventIDs and configuration parameters from the UE training center, including registration in NRF, and subscription and notify service operations.

- Determines when to start and terminate the data collection, notifies the UE about the data to be collected and when the data collection and reporting to the UE training center starts.

- Determines what information to configure to the UE based on the provisioning request from the UE training center.

UE:

- Reports support for ML Model training and reporting input data during registration.

- Receive provisioning information for the new EventIDs and collects and report the collected data for an EventID.

- May receive a timer to indicate that reporting the data collected should start when the timer expires.

UE training center:

- Provide provisioning information for the new EventIDs.

- Subscribe to the reporting of measurements for training a model.

- May use AF influence on traffic routing to enable sending URSP Rules to their UEs.

AMF:

- Stores the UE capabilities to train a ML Model and to report data for training.

************ End of changes **************