3GPP SA WG2 Meeting #169	S2-2505884

Fukuoka, May 19th – 23rd 2025                         Revision of S2-2505631/4912

Source: 	China Mobile

Title: 	New Solution for KI#6 Configuration Parameter Provisioning of Sensing Entities

Document for: 	Approval

Agenda Item: 	20.2.1

Work Item / Release:	FS_ISAC/Rel-20

Abstract: Proposed solution to support KI#6 Configuration Parameter Provisioning of Sensing Entities.

1. Introduction/Discussion

## 5.6	Key Issue #6: Configuration Parameter Provisioning of Sensing Entities.

This Key Issue will study how parameter provisioning of sensing entities is performed.

In particular, this Key Issue will study:

-	How and what parameters need to be exchanged between the sensing entities and the Core Network.

NOTE:	This KI needs to be aligned with RAN WGs.

- Text Proposal

It is proposed to capture the following changes in TR 23.700-14.

# * * * * First change * * * *

6	Solutions

6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |
| --- | --- | --- | --- |
| Solutions | KI#x | KI#x | KI#6 |
| X |  |  | X |

# * * * * Second change (All text new) * * * *

6.X	Solution #X: Solution for Configuration Parameter Provisioning of Sensing Entities

6.X.1	High level principles

Different from sensing service request to trigger sensing service operation, configuration parameter provisioning aims to provision general configuration parameters towards the sensing entities, not for individual sensing service operation.

There may be two cases for configuration parameter provisioning towards sensing entities. The first case may be conducted by sensing operation server, which could belong to the operators or have SLA with the operators. The other case may be performed by the OAM or local configuration within the operator’s domain. This solution focuses on the first case.

The high level principles of the solution are:

1. For the untrusted sensing operation server, it sends configuration parameter provisioning request to the NEF. After authentication/authorization was made, the NEF forwards configuration parameter provisioning request to the SCF(s). For the trusted sensing operation server, it sends configuration parameter provisioning request directly to the SCF(s).

- 2. The SCF determines the targeted sensing entity(s) and performs the configuration parameter provisioning on sensing entity(s).

NOTE X: The SF (Sensing Function) splits into SCF (Sensing Control Function) and the SPF (Sensing Processing Function.

6.X.2	Description

This solution is to resolves Key Issue #6 about the configuration parameter provisioning of sensing entities .

6.X.3	Procedures

6.X.3.1   Sensing operation server initiated Configuration Parameter Provisioning procedure

Figure 6.X.3.1: Configuration Parameter Provisioning procedure

- 1a. The untrusted sensing operation server sends the configuration parameter provisioning request to the NEF with the following parameters: AF ID, sensing related parameters, external target area information and so on.

- 1b. The trusted sensing operation server sends the configuration parameter provisioning request to the SCF with the following parameters: AF ID, sensing related parameters, target area information and so on.

Editor’s NOTE: What are the provisioned configuration parameters is FFS.

NOTE: Sensing operation server is application function.

- 2a. The NEF authenticates and authorizes the untrusted sensing operation server to allow to perform the operation.

- 3a. The NEF converts the external target area information to internal target area information.

- 4a. The NEF obtains the SCF based on internal target area information by NRF query or local configuration. Then the NEF forwards the request to the SCF.

- 5. The SCF determines the targeted sensing entity(s) based on internal target area information.

- 6. The SCF performs the sensing configuration parameters provisioning on sensing entity(s).

- 7a. The SCF notifies the NEF with configuration parameter provisioning response.

- 7b. The SCF notifies the untrusted sensing operation server with configuration parameter provisioning response.

- 8a. The NEF forwards configuration parameter provisioning response to the trusted sensing operation server.

6.x.4	Impacts on services, entities, and interfaces

This solution impacts the following entities.

NEF：

-  Supports the selection of the SCF based on internal target area information.

SCF :

-	Supports configuration parameter provisioning procedures

Sensing Entity:

-  Supports sensing configuration parameter provisioning procedures.

# * * * * End of changes * * * *