3GPP TSG-SA WG2 Meeting #170	S2-2506262

Goteborg, Sweden, August 25 – 29, 2025

Source:	CATT

Title:	KI#2 Solution#6 update

Document for:	Approval

Agenda Item:	20.4.1

Work Item / Release:	FS_EnergySys_Ph2 / Rel-20

Abstract of the contribution: The contribution proposes updates to Solution #6 "Policy control based on notifications from the EIF".

1. Introduction

There are the following ENs in Solution #6 "Policy control based on notifications from the EIF" for KI#2:

Editor's note:	It is FFS how to trigger the PCF to subscribe to notifications from the EIF.

Editor's note:	It is FFS whether the PCF takes notifications from the EIF into account to decide the RFSP Index value in AM policy.

Editor's note:	It is FFS whether the PCF takes notifications from the EIF into account to decide the MA PDU Session Control information in the PCC rules.

1) How to trigger the PCF to subscribe to notifications from the EIF

The PCF may decide to subscribe to notifications from the EIF based on operator policy (e.g. to perform policy control considering energy related information for specific area or time period), AF request (e.g. Energy indicator as specified in clause 6.1.2.4 of TS 23.503 [4]), or notifications of UE subscription data change (e.g. the value of Energy Saving indicator changes) from the UDR.

2) Whether the PCF takes notifications from the EIF into account to decide the RFSP Index value in AM policy

Data transmission over different RAT types and/or frequency bands would cause different energy consumption in the network, due to various reasons e.g. different signalling propagation characteristics, data processing (e.g. modulation) and transmission rate, etc. For example:

-	5G Sub-6 GHz (3.5 GHz) offers moderately higher energy consumption than 4G but better energy efficiency.

-	5G mmWave (28 GHz+) causes high power consumption due to beamforming and dense small cells.

-	4G LTE (1.8 GHz, 2.6 GHz) have comparatively lowers energy consumption but lower data rate.

As specified in clause 5.3.4.3.1 of TS 23.501:

“To support radio resource management in NG-RAN the AMF provides the parameter 'Index to RAT/Frequency Selection Priority' (RFSP Index) to NG-RAN across N2. The RFSP Index is mapped by the RAN to locally defined configuration in order to apply specific RRM strategies, taking into account any available information in RAN. The RFSP Index is UE specific and applies to all the Radio Bearers. Examples of how this parameter may be used by the RAN:

-	to derive UE specific cell reselection priorities to control idle mode camping.

-	to decide on redirecting active mode UEs to different frequency layers or RATs (e.g. see clause 5.3.4.3.2).”

By adjusting RFSP Index value in the AM policy for (selected) UEs taking into account energy related information and based on operator policy, the (selected) UEs can be steered (via cell reselection or redirection) to the RATs or frequency bands with preferred energy performance (e.g. with lower Energy Consumper or higher Energy efficiency at specific time period or in specific area), therefore improving the overall energy performance of the network. How the UEs are selected is up to implementations, e.g. based on on-going applications / service data flows in the UEs.

3) Whether the PCF takes notifications from the EIF into account to decide the MA PDU Session Control information in the PCC rules

Similar to 2), data transmission over 3GPP access and non-3GPP access may have different energy performance. When energy consumption over 3GPP access is abnormally high, steering (selected) Service Data Flows or UEs from 3GPP access to non-3GPP access via MA PDU Session Control information may improve network performance.

Based on the above discussion, updates to Solution #6 are proposed, to resolve the ENs as well as modifying the procedure.

2. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-67.

* * * Start of Change * * * *

6.6	Solution #6: Policy control based on notifications from the EIF

6.6.0	High-level solution principles

This solution proposes that the PCF takes the energy related information in the notifications from the EIF into account for policy control.

NOTE:	It is required that the energy related information in the notifications from the EIF is accurate enough for the PCF to take into account for policy decision.

6.6.1	Description

This solution addresses Key Issue #2.

The PCF can subscribe to notifications from the EIF, e.g. Energy Consumption information at UE, S-NSSAI, PDU Session and Service Data Flow (e.g. per UE per application) granularity as described in clause 5.51 of TS 23.501 [2]. The PCF may decide to subscribe to notifications from the EIF based on operator policy (e.g. to perform policy control considering energy related information for specific area or time period), AF request (e.g. Energy indicator as specified in clause 6.1.2.4 of TS 23.503 [4]), or notifications of UE subscription data change (e.g. the value of Energy Saving indicator changes) from the UDR.

The PCF may determine and update the following policies based on energy related subscription data (i.e. Energy Saving Indicator), notifications from the EIF and operator policy:

-	Access and mobility related policy, e.g. UE-AMBR, RFSP Index.

-	Session management related policy, e.g. Session-AMBR, MA PDU Session Control information.

Examples for AM policy control taking into account the notifications from the EIF are as follows:

-	If notification(s) from the EIF indicates energy consumption for the UE is high based on operator policy (e.g. higher than a predefined UE average energy consumption threshold per PLMN or per network slice), the PCF may modify the UE-AMBR value to be lower than the subscribed UE-AMBR or the UE-AMBR in use for the UE based on operator policy, to reduce the energy consumption for transmitting the Non-GBR QoS Flows of the UE.

-	If notification(s) from the EIF indicates energy consumption for the UE over 5G access is high based on operator policy (e.g. higher than a predefined UE average energy consumption threshold per PLMN or per network slice), the PCF may modify the RFSP Index value to indicate that EPC/E-UTRAN access is prioritized over the 5G access for the UE with validity time based on operator policy as specified in clause 6.1.2.1.1 of TS 23.503 [4].

Examples for SM policy control taking into account the notifications from the EIF are as follows:

-	If notification(s) from the EIF indicates energy consumption for the UE or for a PDU Session of the UE is high based on operator policy (e.g. higher than a predefined UE average or PDU Session average energy consumption threshold per PLMN or per network slice), the PCF may modify the Session-AMBR value to be lower than the subscribed Session-AMBR or the Session-AMBR in use for the PDU Session based on operator policy, to reduce the energy consumption for transmitting the Non-GBR QoS Flows of the PDU Session.

-	If notification(s) from the EIF indicates energy consumption for the UE over 3GPP access is high based on operator policy (e.g. higher than a predefined UE average energy consumption threshold per PLMN or per network slice), the PCF may modify the MA PDU Session Control information (e.g. Steering Mode and associated threshold values) in the PCC rules to switch some or all the Non-GBR QoS Flows of the MA PDU Session from 3GPP access to non-3GPP access as described in clause 6.1.3.20 of TS 23.503 [4].

In addition, based on notifications from the EIF, the PCF may provision and update validity conditions (e.g. validity time period, validity area) in the policies, for network energy saving within a specific time period and/or specific area.

6.6.2	Procedures

The existing policy control procedures as specified in TS 23.502 [3] and TS 23.503 [4] apply, with the enhancements that based on possible triggers (e.g. operator policy, AF request or UE subscription data change) the PCF subscribes to and receives notifications from the EIF before making policy decisions.

Figure 6.6.2-1: Policy control based on notifications from the EIF

1.	The PCF decides to subscribe to notifications from the EIF based on operator policy, AF request (e.g. Energy indicator as specified in clause 6.1.2.4 of TS 23.503 [4]), or notifications from the UDR on UE subscription data change (e.g. the value of Energy Saving indicator changes).

2.	The PCF subscribes to and receives notifications from the EIF as specified in clause 4.29.2 of TS 23.502 [3].

3.	The PCF makes policy decisions taking into account the notifications from the EIF.

4.	The PCF performs policy control as specified in TS 23.502 [3] and TS 23.503 [4].

6.6.3	Impacts on services, entities and interfaces

PCF:

-	Subscribes to notifications from the EIF based on possible triggers (e.g. operator policy, AF request or UE subscription data change).

-	Performs policy control taking into account of notifications from the EIF.

EIF:

-	Sends energy related notifications to the PCF.

* * * End of Change * * * *