3GPP TSG-SA WG2 Meeting #170	S2-2506343

Goteborg Sweden, August 25 -29, 2025

Source:	Ericsson

Title:	KI#2: Introducing UE Proxy Capability in P-CSCF for IMS signalling optimization.

Document for:	Approval

Agenda Item:	20.1.1

Work Item / Release:	FS_5GSAT_Ph4_ARC/ Rel-20

Abstract of the contribution: The contribution proposes a solution to optimize IMS signalling.

# 1. Introduction

This contribution proposes a solution to Key Issue #2:

-	Support of IMS voice call over NB-IoT NTN via GEO satellite connecting to EPC

# 2. Proposal

It is proposed to document the following solution to 3GPP TR 23.700-19.

* * * Start of change * * * *

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |
| --- | --- | --- | --- | --- |
| Solutions | #1 | #2 | #3 | #4 |
| #1 | x | x |  |  |
| #2 | x |  |  |  |
| #3 | x |  |  |  |
| #4 | x | x | x | x |
| #5 | x |  |  |  |
| #6 | x |  |  |  |
| #7 | x | x |  |  |
| #8 | x |  |  |  |
| #9 | x |  |  |  |
| #10 | x |  |  |  |
| #11 | x |  |  |  |
| #12 |  | x |  |  |
| #X |  | X |  |  |

* * * Next Change all new * * * *

## 6.X	Solution #x: Signalling Optimization via IWK in P-CSCF

### 6.X.1	High level principles

This solution addresses KI#2 to support IMS voice call over NB-IoT via GEO connecting to EPC.

The following principles are applied to this solution:

-	Binary encoding of SIP over UNI using the framework defined in TS 24.294 [14] for I1 encoding. The UNI reference point is labelled GEO Gm for the purpose of this solution.

-	No restrictions are applied in this solution on UNI signalling in terms of what the UE negotiates, e.g. pre-conditions, 100rel, etc.

-     Only necessary information elements are included in the UNI to enable interoperability and support originating/terminating UE negotiated needs.

-	In summary, P-CSCF supports a UE proxy functionality, as depicted in Figure 6.X.2-1, that performs the following:

-	Translation from binary encoded SIP to text-based SIP encoding and vice versa.

-	Maintaining an association between the incoming and outgoing sessions to ensure compliance to GEO Gm and Mw reference points for interworking purposes. Separate session Identifiers are used for the incoming and outgoing sessions.

-	Storing needed information elements, and updating stored values where applicable, to ensure compliance to the GEO Gm and Mw reference points.

The UE proxy functionality does not hold session state machine and does not interfere with the state machine implemented in the P-CSCF.

-	IMS Registration over the GEO Gm reference point is performed as is today, i.e. it complies to TS 24.229 [16]. No binary encoding is performed.

-	This solution does not require any update to the remote end for interworking. Additionally, the solution assumes dedicated bearer for IMS voice.

Additional Assumptions:

- The identity used for initiating an IMS session is the default IMPU.

- P-CSCF removes the SIP headers/ SIP Header fields, which do not need to be sent to the UE. The P-CSCF stores the removed information should this be needed for the Mw interactions with the IMS Core.

- The P-CSCF does not perform additional security checks in relation to IPSec during IMS session initiation or termination procedure.

- UE Location formatted according to 24.229 PANI encoding is not required. Only the Cell ID is sent, when needed, using the 23.003 ECGI encoding.

- PAI coding is supported over the UNI towards the UE when needed.

- P-CSCF does not need to send to terminating UEs Information elements whose values have not changed. Similarly, UE does not need to transmit to the P-CSCF IEs whose values have not changed. This is however not reflected in the tables below.

- Support for privacy header (OIR) can be included with no impact on call session setup time.

- AGW not impacted by binary encoding of SIP.

### 6.X.2	Description

The Figure below depicts the functional architecture for this solution. In this architecture; GEO Gm is the new reference point for the UNI for binary encoding of SIP using the 24.294 [14] framework.

GEO P-CSCF is the P-CSCF that performs the necessary interworking between the GEO Gm and Mw, based on the high-level principles depicted above and detailed below.  As depicted in the figure, the UE Proxy functionality is strictly restricted to signalling interworking between binary-encoded SIP and text-based SIP, with no SIP state machine.

Figure 6.x.2-1: Architecture to Support GEO IMS audio sessions, and Messaging

In this architecture, SIP Registration is proposed to be  used as is today, according to TS 24.229 [16], i.e. SIP Registration is text-based.

The solution supports the two alternatives for transporting IMS voice over the dedicated bearer within an IMS IP PDN connection: as non-IP (NIDD) traffic, or as IP traffic. The difference being the SIP message sizes in both cases and well as the interaction between the P-CSCF and PCC/PCRF during dedicated bearer setup.

The framework defined in TS 24.294 [14] for the binary-encoded SIP protocol over I1 interface  depicted below is deployed for supporting the GEO Gm interface.

The structure has a minimum length of 7 octets. The size of any GEO Gm message varies based on the necessary information elements needed to support the specific aspects being negotiated within an IMS session

| 8 | 7 | 6 | 5 | 4 | 3 | 2 |  | 1 | Octet |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Protocol version number |  |  |  | Protocol identifier |  |  |  |  | 1 |
| Message type |  |  |  |  | R |  | Reason |  | 2 |
| Reason |  |  |  |  |  |  |  |  | 3 |
| Call-Identifier (Part-1) |  |  |  |  |  |  |  |  | 4 |
| Call-Identifier (Part-2) |  |  |  |  |  |  |  |  | 5 |
| Call-Identifier (Part-2) |  |  |  |  |  |  |  |  | 6 |
| Sequence-ID |  |  |  |  |  |  |  |  | 7 |
| Information element #1 |  |  |  |  |  |  |  |  |  |
| Information element #2 |  |  |  |  |  |  |  |  |  |

Figure 6.x.2-1: I1 message structure

To ensure compliance to TS 24.229 [16] the protocol over I1 reference point has been extended with:

- new protocol Messages: ACK, PRACK and UPDATE.

- additional IEs depicted in yellow in the table below

The use and processing of these elements and others is described in the procedure clause and referenced tables therein. Additionally, some of the elements currently supported in I1, indicated in green, are not required in this solution as indicated in the two tables below

Table 6.x.2-1 below is intended to be used for the case of NIDD  for IMS voice.

NOTE:  	User plane and bearer types including content is handled in other solutions and may impact some of the IEs in the tables below.

Table 6.x.2-1 SIP proposed headers in case of NIDD of IMS Voice

EXISTING

NEW

Table 6.x.2-2 below is intended to be used for the case of IP transport for IMS voice.

Table 6.x.2-2 SIP proposed headers in case of IP of IMS Voice

### 6.x.3	Procedures

#### 6.X.3.1 IMS Session Initiation with preconditions and 100rel

The call flow depicts an example for session establishment using preconditions and reliable provisional response for 183.

The tables referenced in the call flow steps assume a NIDD for IMS voice.

Figure 6.x.3.1-1 Originating IMS Session

The following is a brief description in the call flow:

- In step 1 the UE registers as per existing procedures in TS 24.229 [16]. The P-CSCF stores the contact information without the feature tags, Cell-ID information, Allow, and the registered identity information.

- In step 2, a UE is in IDLE mode performs the Service Request Procedure to get connected and subsequently initiates an IMS session.

- In step 3, the P-CSCF receives a GEO Gm INVITE that includes the information in Table 6.X.3.1-1 per the column entitled “IE included in Geo Gm” under the INVITE UL banner. The P-CSCF stores the received call ID, Cseq, To-id, Cell-ID, Supported, Require and precondition & codec negotiation information for later use.

- In step 4, the P-CSCF answers with a GEO Gm progress (Reason 100) message to the UE that includes the call ID, and appropriate Cseq. No other information is required in this message.

- In step 5 the P-CSCF reserves the necessary resources in the AGW using existing procedures in TS 23.228 [6]

- In step 6, P-CSCF generates a SIP INVITE towards the S-CSCF and populate the needed SIP headers to comply to Mw as depicted in Table 6.X.3.1-1 per the column entitled “P-CSCF processing towards the next hop (Mw) under the “INVITE (UL)” banner. For interworking purposes, P-CSCF holds an association between the incoming and outgoing session with the appropriate IE stored for each session.

- In step 7, P-CSCF receives a 183 Session Progress from the remote end, including the SDP Answer.

- In step 8, P-CSCF initiates the dedicated bearer establishment using existing procedures. The detail of the procedure varies depending on how IMS Voice is transported over the dedicated bearer:

  - If IMS voice transport as IP  traffic is negotiated, then current behaviour does not change.

  - If NIDD for IMS voice is negotiated, then the P-CSCF includes the address of the AGW for forwarding the traffic. This IP address and port will be added by the PGW for UL traffic and removed for DL traffic. The PGW maintains a binding between the UE and the AGW IP address allocated by the AGW for the duration of the session.

NOTE: Other solutions will provide details for this step.

- In step 9, P-CSCF sends a GEO Gm progress message to the UE that includes the IEs per the column entitled “IE included in Geo Gm” under the “183 with SDP (DL)” banner in Table 6.X.3.1-1. The P-CSCF handling of SIP headers received in the 183 SDP Session Progress from the remote end and the formulation of elements in the GEO GM progress message is depicted in the column entitled “P-CSCF processing towards the next hop (UE)” under the “183 with SDP” (DL) banner.

- In step 10, the P-CSCF receives a GEO Gm PRACK that includes the information in Table 6.X.3.1-2 per the column entitled “IE included in Geo Gm” under the “PRACK UL” banner.

- In step 11 P-CSCF generates a SIP PRACK towards the S-CSCF and populate the needed SIP headers to comply to Mw as depicted in Table 6.X.3.1-2 per the column entitled “P-CSCF processing towards the next hop (Mw)” under the “PRACK (UL)” banner.

- In step 12, P-CSCF receives a SIP 200 OK acknowledging the SIP PRACK.

- In step 13, P-CSCF sends a GEO Gm success message to the UE that includes the IEs per the column entitled “IE included in Geo Gm” under the “200 OK PRACK (DL)” banner in Table 6.X.3.1-2. The P-CSCF handling of SIP headers received in the 200 OK from the remote end and the formulation of elements in the GEO GM success message is depicted in the column entitled “P-CSCF processing towards the next hop (UE)” under the “200 OK PRACK (DL)” banner.

- In step 14, the P-CSCF receives a GEO Gm UPDATE to indicate that precondition is fulfilled at the UE, and that includes the information in Table 6.X.3.1-3 per the column entitled “IE included in Geo Gm” under the “UPDATE UL” banner.

- In step 15 P-CSCF generates a SIP UPDATE towards the S-CSCF and populate the needed SIP headers to comply to Mw as depicted in Table 6.X.3.1-3 per the column entitled “P-CSCF processing towards the next hop (Mw)” under the “UPDATE (UL)” banner.

- In step 16, P-CSCF receives a SIP 200 OK acknowledging the SIP UPDATE.

- In step 17, P-CSCF sends a GEO Gm success message to the UE that includes the IEs per the column entitled “IE included in Geo Gm” under the “200 OK UPDATE (DL)” banner in Table 6.X.3.1-3. The P-CSCF handling of SIP headers received in the 200 OK from the remote end and the formulation of elements in the GEO GM success message is depicted in the column entitled “P-CSCF processing towards the next hop (UE)” under the “200 OK UPDATE (DL)” banner.

- In step 18, P-CSCF receives a 180 Ringing from the remote end.

- In step 19, P-CSCF sends a GEO Gm progress message to the UE that includes the IEs per the column entitled “IE included in Geo Gm” under the “180 Ringing (DL)” banner in Table 6.X.3.1-4. The P-CSCF handling of SIP headers received in the 180 ringing from the remote end and the  formulation of elements in the GEO GM success message is depicted in the column entitled “P-CSCF processing towards the next hop (UE)” under the “180 Ringing (DL)” banner in depicted in Table 6.X.3.1-4.

- In step 20, P-CSCF receives a SIP 200 OK final response.

- In step 21, P-CSCF opens the gate for traffic to start flowing in both directions

- In step 22, P-CSCF sends a GEO Gm success message to the UE that includes the IEs per the column entitled “IE included in Geo Gm” under the “200 OK (DL)” banner. The P-CSCF handling of SIP headers received in the 200 OK from the remote end and the  formulation of elements in the GEO GM success message is depicted in the column entitled “P-CSCF processing towards the next hop (UE)” under the “200 OK (DL)” banner as depicted in Table 6.X.3.1-4.

NOTE:	In the tables below P-CSCF is assumed to be GEO P-CSCF.

Table 6.x.3.1-1 Originating INVITE/100/183 Signalling Information

Table 6.x.3.1-2 PRACK/200 OK Signalling Information

Table 6.x.3.1-3 UPDATE/200 OK Signalling Information

Table 6.x.3.1-4 180/200 OK Signalling Information

#### 6.X.3.2 Terminating IMS Session with preconditions and 100rel

The same Figure 6.x.3.1-1 applies in this case as well. However, the focus will be on the terminating UE.

For the terminating call case the seme principles described for originating call apply, however with focus on the terminating call leg and UE

The next 4 tables depict the treatment by the P-CSCF.

Table 6.x.3.2-1 Terminating INVITE/100/183 Signalling Information

Table 6.x.3.2-2 PRACK/200 OK Signalling Information

Table 6.x.3.1-3 UPDATE/200 OK Signalling Information

Table 6.x.3.1-4 180/200 OK Signalling Information

#### `6.X.3.3 Support for IP PDN connection for IMS Voice

The impacted messages from using an IP PDN connection for IMS voice in an originating scenario with impact on session setup time is an initial INVITE, a 183-session progress message.  The impacts are illustrated in the table 6.X.3.3-1 below.

Table 6.x.3.3-1 Originating INVITE/100/183 Signalling Information – IP PDN connection

For a terminating scenario, the impacted messages are an incoming INVITE and 183-session progress message from the UE. The impacts are illustrated in the table 6.X.3.3-2 below.

Table 6.x.3.3-2 Terminating INVITE/100/183 Signalling Information – IP PDN connection

#### 6.X.3.4 IMS Session Setup Time calculation

##### 6.X.3.4.1 IMS session setup time in case of NIDD for IMS Voice

The table below shows the IMS session up time, under good conditions and excluding the time needed by EPS activities, to be below 10 seconds, in the worst-case scenario of overhead (Orange column) with IPSec, IPv6, TCP, and no ROHC.

NOTE: TCP is not required in this case.

The SIP size (Grey column) is based on the four tables in clause 6.X.3.1.

##### 6.X.3.4.2 IMS session setup time in case of IP transport for IMS voice

The table below shows the IMS session up time, under good conditions and excluding the time needed by EPS activities, to be below 10 seconds, in the worst-case scenario of overhead (Orange column) with IPSec, IPv6, TCP, and no ROHC.

### 6.X.4	Impacts to Services, Entities and Interfaces

P-CSCF:

- Implements UE proxying capability to manage protocol interworking between UNI and Mw.

UE:

- Implements UNI protocol based on GEO Gm.

* * * End of change * * * *