SA WG2 Meeting #170	S2-2506363

25 – 29 Aug, 2025, Goteborg, SE

Source:	OPPO

Title:	[WT#5, all topics] WT updates and proposed KIs

Document for:	Agreement

Agenda Item:	20.6.5

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: This paper is to propose Work Tasks (WTs) and Key Issues (KIs), including the rationale, expected scope, and structuring of WT#5.

# 1. Justifications

Editor's Note: This justification section is not included in the TR; it is intended solely to justify the scope of the proposed WT.

Beyond traditional connectivity services, 6G is expected to introduce new data-centric services characterized by providing various data products to consumers. These data products may include sensing results, AI models, training datasets for AI models, and other valuable data assets. This evolution will enable operators to expand their business models from simply providing connectivity to offering comprehensive data services and solutions.

WT#5 has been defined in the 6G SID (SP-250806) as below:

WT#5: Study data framework for all aspects related to efficient and scalable data handling including, for example, data collection, distribution, processing, storage, data access and data exposure, with consideration of access control/user consent and privacy where relevant. The example of data may include data for AI and Sensing. This WT can also study any potential enhancements on system and procedure needed for user consent framework. NOTE 6: The work split with SA3, SA5 and RAN WGs will require TSG coordination

In the meantime, R20 5G-A Sensing service study and AI/ML service study are on-going to fulfil operator’s current requirements, the frameworks of which are show in Figure 1 and Figure 2.

Figure 1 Framework of Sensing service

Figure 2 Framework of AI/ML service

As seen in Figure 1 and Figure 2, there are many common aspects between framework of Sensing service and Framework of AI/ML service:

- Target area: target sensing service area or target data collection area

- Measurement entity: sensing entity (i.e. transmitter/receiver) for Sensing or Data Collection entity for AI/ML

- Logical functions: measurement entity configuration, measurement entity selection, data processing (either sensing result generation for sensing or Model Training/Inference for AIML), service authorization & exposure, data collection, data storage (sensing data, sensing result, AI dataset or AI model)

- Subscriber data and service data: user profile and data profile to enable user consent and data consent

- Service consumer: to consume sensing result, AI dataset for model training, or AI model

While distinct QoS demands (e.g., ultra-low latency sensing vs. high-throughput AI/ML model transfer) and potential protocol differences are valid concerns, they underscore the necessity of a common data framework. Managing separate silos for each service type creates significant operational complexity, redundant costs, and hinders resource optimization. Crucially, a unified framework facilitates transformative value by enabling seamless correlation of real-time sensing data with AI/ML models and historical datasets. It also simplifies service delivery: a single consumer request often implicitly requires both sensing and AI/ML services. A Unified Exposure Function within the framework intelligently interprets such requests, orchestrates the necessary underlying services (each potentially using optimized QoS paths and protocols), and delivers a consolidated result. Furthermore, a common framework provides centralized governance, inherent adaptability for future services, and dynamic, policy-driven resource allocation across all data flows. Ultimately, it transforms diverse requirements from an operational burden into a managed asset, delivering efficiency, agility, and superior data value that far outweigh the perceived complexity of integration.

In short, developing isolated service frameworks for sensing and AI/ML, despite their shared functional requirements, leads to redundancy. Instead, a common foundational framework should be established. This unified approach prevents overlapping solutions, promotes multi-purpose data reuse, and allows for the integrated provisioning of both sensing and AI/ML services to consumers.

Figure 3 Data Operation

In the common data framework, the most essential aspect is the data operation. As shown in Figure 3, the data operation includes the following components:

- data collection from data source (UE, RAN node, CN NF) for processing and exposure, including configuration to data source, measurement at data source and data transfer

- data processing for result generation, model training or model inference

- data storage for future reuse

- data exposure to consumer (AF, UE, RAN node, CN NF)

Prior to data operation, authorization for both data source access and service usage is required. Furthermore, throughout the data operation process, continuous validation ensures data remains owner-consented and adapts to changing conditions.

Additionally, effective data operation requires the discovery & selection of data operation entities, including data source, data processing entity and data storage entity, to fulfil service requirements and ensure a positive user experience. Crucially, the selection of appropriate data sources has a profound impact on the ultimate quality of service. The verification from data source is also important when it is a UE to make sure that it can work properly (e.g. with enough power and CPU) to provide the required data.

To support data-centric services, this WT proposes an overlay architecture augmenting the baseline 3GPP system. Key elements under consideration include:

- Introduction of new Network Functions (NFs).

- Definition of new interfaces (UE↔New NF, New NF↔New NF, RAN↔New NF, Existing NF↔New NF).

- Potential extensions to existing interfaces.

- Potential enhancements to existing NFs.

5G-A architecture will be reused as much as possible.

Considerring that the common solutions will be developped for AIML and Sensing, it is proposed to study common KIs of AIML and Sensing under WT#5, i.e. common data framework, to prevent potential overlaps in study and ensure aligned solutions. WT#3 and WT#4 focus on dedicated KIs of its own, which is also benefitial to TU saving.

Figure 4 WT#3/4/5 dependencies

**** First Change ****

# Annex A.X. WT#5 Scope

Editor's Note: Describe the technical scope of the proposed Work Task. If applicable, suggest logical subdivision of this WT into smaller sub-WT. This clause is part of the TR Annex.

WT#5: Study data framework for all aspects related to efficient and scalable data handling including, for example, data collection, distribution, processing, storage, data access and data exposure, with consideration of access control/data consent (i.e. data consented by the owner) and privacy where relevant,, including:

-	End to end architecture to support data-centric services, i.e. Sensing service, AI/ML service and others

-	Service authorization and data consent

-	Discovery and selection of data operation entities (i.e. data source, data collection entity, data processing entity, data storage entity) and data source authorization

-	Data operation, including data collection, data processing, data storage and data exposure.

NOTE 6:	The work split with SA3, SA5 and RAN WGs will require TSG coordination

**** Second Change (all new) ****

# 5.X1. Key Issue #X1:  Data service authorization and data consent

Editor's Note: This clause defines the potential scope of KI(s) and is part of the TR.

Securing the unified framework's operation, especially multi-service orchestration across diverse data (e.g., sensing, AI/ML), demands robust authorization and consent mechanisms.

Objectives of this KI include:

- Study data service access authorization mechanism, including the authorization to the service initiator, consumer and the target object/environment

- Study data consent mechanism during data operation, including data ownership consent, data validation management and data ownership management

NOTE: 	Data service access authorization, data consent and other security aspects will be coordinated with SA WG3.

# 5.X2. Key Issue #X2:  Discovery and selection of data operation entities and data source availability verification

Efficiently composing data services across diverse types (e.g., sensing, AI/ML datasets) requires dynamic discovery of data operation entities (UE, RAN, CN NF) and verification of their availability.

Objectives of this KI include:

- Discovery and selection of data operation entities (i.e. data source, data collection entity, data processing entity, data storage entity) for various data types (e.g. sensing data, dataset for AI model training), where the data operation entity includes UE, RAN node and CN NF

- Data source (i.e. UE, RAN node, CN NF) availability verification

# 5.X3. Key Issue #X3:  Data operation

Objectives of this KI is to study end to end architecture and process of data collection, storage and exposure, where the data includes sensing data, sensing result, AI dataset or AI model, in the following aspects:

- data collection from data source for processing and exposure, including

  - configurations to data operation entities (e.g. UE, RAN node, CN NF) for measurement and data transfer

  - data transfer from data source to the data processing entity (e.g. UE, RAN node, CN NF) or to the data collection entity

- data storage and management of virous data types

- data exposure to consumer (AF, UE, RAN node, CN NF)

**** End of Changes ****