3GPP SA WG2 Meeting #170	S2-2506380

# 25 - 29 August, 2025, Goteborg, Sweden									  				  (revision of)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 1499 | rev |  | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on confidence of prediction |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph3 |  |  |  |  |  | Date: |  |  | 2025-08-14 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Some input data definitions do not include an explicit timestamp or time-related information, which is essential for correlating multiple input data sources when training ML models in the NWDAF. In addition, some output analytics definitions omit the confidence of predictions, or specify it at an inappropriate granularity. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the missing timestamp in input data or confidence in output analytics. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unable to accurately correlate data from different sources, leading to reduced effectiveness or accuracy of ML model training. Consumers of NWDAF analytics may misinterpret the reliability of the predictions and affect the management decisions. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5.2, 6.6.2, 6.7.2.3, 6.7.3.3, 6.7.5.2, 6.8.2, 6.11.2, 6.11.3, 6.13.2, 6.17.2, 6.18.3, 6.19.2, 6.21.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * Start of change* * * *

### 6.5.2	Input data

For the purpose of NF load analytics, the NWDAF may collect the information as listed in Table 6.5.2-1 for the relevant NF instance(s).

Table 6.5.2-1: Data collected by NWDAF for NF load analytics

| Information | Source | Description |
| --- | --- | --- |
| Timestamp | NRF / OAM | A time stamp of time associated with the collected information. |
| NF load | NRF | The load of specific NF instance(s) in their NF profile as defined per TS 29.510 [18]. |
| NF status | NRF | The status of a specific NF instance(s) (registered, suspended, undiscoverable) as defined per TS 29.510 [18]. |
| NF resource usage | OAM | The usage of assigned virtual resources currently in use for specific NF instance(s) (mean usage of virtual CPU, memory, disk) as defined in clause 5.7 of TS 28.552 [8]. |
| NF resource configuration | OAM | The life cycle changes of specific NF resources (e.g. NF operational or interrupted during virtual/physical resources reconfiguration) as defined in clause 5.2 of TS 28.533 [19]. |

NOTE 1:	The OAM information can be used as a complement to NRF information for some or all of the following aspects: resources utilization, NRF information correlation and alternative source of information if NRF information on load is not available.

NOTE 2:	NWDAF can request NRF for data related to NF instances, as described in TS 29.510 [18].

NOTE 3:	NWDAF can correlate the NF resources configuration with NF resource usage for generating the analytics output.

If target NF type is UPF, the NWDAF may collect the information as listed in Table 6.5.2-2, in addition to information listed in Table 6.5.2-1.

Table 6.5.2-2: Data collected by NWDAF for UPF load analytics

| Information | Source | Description |
| --- | --- | --- |
| Timestamp | UPF | A time stamp of time associated with the collected information. |
| Traffic usage report | UPF | Report of user plane traffic in the UPF for the accumulated usage of network resources (see clause 5.2.1.3.3 of TS 29.564 [51]). |

For the purpose of NF load analytics, the NWDAF may collect the information as listed in Table 6.5.2-3 (from OAM via MDT) and Table 6.5.2-5 via the AF (for trusted AF) or NEF (for untrusted AF) in addition to other information described above.

Table 6.5.2-3: MDT input data for UE

| Information | Source | Description |
| --- | --- | --- |
| Timestamp | OAM (see NOTE 1) | A time stamp of time associated with the collected information. |
| UE Speed | OAM (see NOTE 1) | UE Speed (see TS 37.320 [20]). |
| UE Orientation | OAM (see NOTE 1) | UE Orientation (see TS 37.320 [20]). |
| NOTE 1: UE input data collection for a specific UE from OAM (via MDT), is as captured in clause 6.2.3.1. |  |  |

Table 6.5.2-4: Per UE attribute to be collected and processed by the AF

| Information | Source | Description |
| --- | --- | --- |
| Per UE attribute | UE Application (see NOTE 1) | UE application data to be collected from UE. |
| > Destination |  | Expected final location of UE based on the route planned. |
| > Route |  | Planned path of movement by a UE application (e.g. a navigation app). The format is based on the SLA. |
| > Average Speed |  | Expected speed over the route planned by a UE application. |
| > Time of arrival |  | Expected Time of arrival to destination based on the route planned. |
| > Timestamp |  | A time stamp of time that the collective attribute derived. |
| NOTE 1: The procedure for data collection from UE Application is as covered in clause 6.2.8. |  |  |

Table 6.5.2-5: AF input data to the NWDAF for Collective Behaviour of UEs

| Information | Source | Description |
| --- | --- | --- |
| Collective Attribute | AF / NEF (see NOTE 1, NOTE 2) | Characterise collective attribute per set of UEs (see Table 6.5.2-4) within the area of interest. |
| > Number of UEs |  | Total number of UEs that fulfil a collective behaviour within the area of interest. |
| > Timestamp |  | A time stamp of time that the collective attribute derived. |
| > Application ID(s) | (see NOTE 3) | Identifying the application providing this information |
| > List of UE IDs | (see NOTE 4) | UE IDs that fulfil a collective behaviour within the area of interest. |
| NOTE 1: For collective behaviour attribute, data processing procedure is as defined in clause 6.2.8. NOTE 2: Per collective attribute, the AF may provide several collective attribute sets, if several sets of UEs with similar behaviour are identified. A similar behaviour can be identified to specific ranges if the AF performs data processing (Data Anonymisation, Aggregation or Normalization) based on NWDAF request. UEs falling in the same range per UE attribute can form a collective attribute set. NOTE 3: The application ID(s) (either external or Internal) is optional. If the application ID(s) is not provided, the relevant application ID(s) can be identified by NWDAF based on the relevant event ID as registered in NRF as covered in clause 6.2.8.2.2. NOTE 4: List of UE IDs is optional and subject to support by the AF when processing the data based on NWDAF request. |  |  |

Based on network configuration, NWDAF may discover the AF from the NRF as defined in 6.2.8.2.2 (based on Collective Behaviour as Event ID or a corresponding Application ID).

For AF in trusted domain, the NWDAF invokes step 3a in clause 6.2.8.2.3 by using Naf_EventExposure_Subscribe service (Event ID = Collective Behaviour, Event Filter information, Target of Event Reporting). The collective attribute (see Table 6.5.2-5) can be indicated as part of event filter information as defined in TS 23.502 [3]. Otherwise, the AF notifies for all collective attributes within the area of interest.

For AF in untrusted domain, the NWDAF invokes step 3b in clause 6.2.8.2.3 by using Nnef_EventExposure_Subscribe (Event ID = Collective Behaviour, Event Filter information, Target of Event Reporting). The collective attribute (see Table 6.5.2-5) can be indicated as part of event filter information as defined in TS 23.502 [3]. Otherwise, the AF via NEF notifies for all collective attributes within the area of interest.

For Collective Behaviour of multiple UEs, NWDAF based on the configuration by MNO may request certain type of data processing from the AF as part of event filter information (e.g. for anonymisation, normalisation, aggregation). The data processing requested by NWDAF is used to anonymise, normalise or aggregate the same UE attribute from multiple UEs at the AF before notifying to the NWDAF.

For each UE attribute of a specific UE, whether and how AF is processing the data that is received from the UE depends on the SLA configured in AF (defined in clause 6.2.8.1) and is not known by the NWDAF.

To determine NF load (per area of interest), NWDAF may collect and take into account UE trajectory input data from the AF, defined in clause 6.7.2.2, Table 6.7.2.2-2 for UE mobility analytics in addition to MDT input data and /or collective behaviour input data, defined in clause 6.5.2, Table 6.5.2-3 and Table 6.5.2-5, respectively.

# * * * * Next change * * * *

### 6.6.2	Input Data

The NWDAF collects Load and Performance information in an Area of Interest from the sources listed in Table 6.6.2-1 and number of UEs within Area of Interest from the sources listed in Table 6.6.2-2.

Table 6.6.2-1: Load and Performance information collected by NWDAF

| Load information | Source | Description |
| --- | --- | --- |
| Timestamp | NRF / OAM | A time stamp of time associated with the collected information. |
| Status, load and performance information | OAM | Statistics on RAN status (up/down), load (i.e. Radio Resource Utilization) and performance per Cell Id for the traffic type of interest and in the Area of Interest as defined in TS 28.552 [8]. |
| NF Load information | NRF | Load per NF |

Table 6.6.2-2: Number of UEs in Area of Interest information collected by NWDAF

| Number of UEs information | Source | Description |
| --- | --- | --- |
| Timestamp | AMF | A time stamp of time associated with the collected information. |
| Number of UEs | AMF | Number of UEs in an Area of Interest |

The NWDAF shall be able to collect UE mobility information as stated in clause 6.7.2.2.

# * * * * Next change * * * *

#### 6.7.2.3	Output Analytics

The NWDAF supporting data analytics on UE mobility shall be able to provide UE mobility analytics to consumer NFs or AFs. The analytics results provided by the NWDAF could be UE mobility statistics as defined in table 6.7.2.3-1, UE mobility predictions as defined in Table 6.7.2.3-2:

Table 6.7.2.3-1: UE mobility statistics

| Information | Description |
| --- | --- |
| UE group ID or UE ID | Identifies the UE(s) for which the statistic applies by a list of SUPIs, or a group of UEs by a list of Internal-Group-Ids defined in clause 5.9.7 of TS 23.501 [2] (see NOTE 1). |
| Time slot entry (1..max) | List of time slots during the Analytics target period. |
| > Time slot start | Time slot start within the Analytics target period. |
| > Duration | Duration of the time slot. If a Temporal granularity size was provided in the request or subscription, the Duration is greater than or equal to the Temporal granularity size. |
| > UE location (1..max) | Observed location statistics (see NOTE 2). |
| >> UE location (NOTE 5) | TAs or cells which the UE stays or geographical location (longitude and latitude level) (see NOTE 3). |
| >> Ratio (NOTE 5) | Percentage of UEs in the group (in the case of a UE group). |
| >> UE's geographical distribution (NOTE 5) | The geographical distribution of the UEs among the TAs or cells or location coordinates. |
| >> Requested Linear Distance Threshold (NOTE 4) | The linear distance threshold used for UE location reporting. |
| >> Geographical Identifier (NOTE 5) | Geographical Identifier as specified in TS 23.228 [47] (see NOTE 6). |
| > UE's direction (NOTE 5) | The direction of the UEs in the Area of Interest. |
| NOTE 1: When Target of Analytics Reporting is an individual UE, one UE ID (i.e. SUPI) will be included, the NWDAF will provide the analytics mobility result (i.e. list of (predicted) time slots) to NF service consumer(s) for the UE. NOTE 2: If Visited AOI(s) was provided in the analytics request/subscription, the UE location provides information on the observed location(s) that the UE or group of UEs had been residing during the Analytics Target Period. NOTE 3: When possible and applicable to the access type, UE location is provided according to the preferred granularity of location information and Spatial granularity size. NOTE 4: The requested Linear Distance Threshold is provided only when in the analytic filter information of the analytics request there are multiple linear distance thresholds and the target is a single UE. NOTE 5: Analytics subset that can be used in "list of analytics subsets that are requested" and "Preferred level of accuracy per analytics subset". NOTE 6: It depends on the implementation how the NWDAF collects the geographic identifier. |  |

Table 6.7.2.3-2: UE mobility predictions

| Information | Description |
| --- | --- |
| UE group ID or UE ID | Identifies the UE(s) for which the prediction applies by a list of SUPIs, or a group of UEs by a list of Internal-Group-Ids defined in clause 5.9.7 of TS 23.501 [2] (see NOTE 1). |
| Time slot entry (1..max) | List of predicted time slots. |
| >Time slot start | Time slot start time within the Analytics target period. |
| > Duration | Duration of the time slot. If a Temporal granularity size was provided in the request or subscription, the Duration is greater than or equal to the Temporal granularity size. |
| > UE location (1..max) | Predicted location prediction during the Analytics target period. |
| >> UE location (NOTE 3) | TAs or cells where the UE or UE group may move into or geographical location (longitude and latitude level) (see NOTE 2). |
| >> Confidence | Confidence of this prediction. |
| >> Ratio (NOTE 3) | Percentage of UEs in the group (in the case of a UE group). |
| >> UE's geographical distribution (NOTE 3) | The geographical distribution of the UEs among the TAs or cells or location coordinates. |
| >> Geographical Identifier (NOTE 3) | Geographical Identifier as specified in TS 23.228 [47] (see NOTE 4). |
| > UE's direction (NOTE 3) | The direction of the UEs in the Area of Interest. |
| >> Confidence | Confidence of this prediction. |
| NOTE 1: When Target of Analytics Reporting is an individual UE, one UE ID (i.e. SUPI) will be included, the NWDAF will provide the analytics mobility result (i.e. list of (predicted) time slots) to NF service consumer(s) for the UE. NOTE 2: When possible and applicable to the access type, UE location is provided according to the preferred granularity of location information and Spatial granularity size. NOTE 3: Analytics subset that can be used in "list of analytics subsets that are requested" and "Preferred level of accuracy per analytics subset". NOTE 4: It depends on the implementation how the NWDAF collects the geographic identifier. |  |

The results for UE groups address the group globally. The ratio is the proportion of UEs in the group at a given location at a given time.

The number of time slots and UE locations is limited by the maximum number of objects provided as part of Analytics Reporting Information.

The time slots shall be provided by order of time, possibly overlapping. The locations shall be provided by decreasing value of ratio for a given time slot. The sum of all ratios on a given time slot must be equal or less than 100%. Depending on the list size limitation, the least probable locations on a given Analytics target period may not be provided.

If a UE Location order indicator is included in the Analytics Reporting information, the NWDAF does not aggregate the UE locations in a long duration but provides the UE locations one by one in their own time period, i.e. the "UE location (1..max)" in the UE Mobility analytics has only one UE location (TA, Cell or a finer granularity UE Location smaller than cell) which indicates the UE is located in this UE location in the duration from the time slot start (i.e. time stamp when the UE enters this location as described in clause 6.7.2.2).

# * * * * Next change * * * *

#### 6.7.3.3	Output Analytics

The NWDAF supporting UE Communication Analytics provides the analytics results to consumer NFs. The analytics results provided by the NWDAF include the UE communication statistics as defined in Table 6.7.3.3-1 or predictions as defined in Table 6.7.3.3-2.

Table 6.7.3.3-1: UE Communication Statistics

| Information | Description |
| --- | --- |
| UE group ID or UE ID | Identifies the UE(s) for which the statistic applies by a list of SUPIs, or a group of UEs by a list of Internal-Group-Ids defined in clause 5.9.7 of TS 23.501 [2]. |
| UE communications (1..max) (NOTE 1) | List of communication time slots. |
| > Periodic communication indicator (NOTE 1) | Identifies whether the UE communicates periodically or not. |
| > Periodic time (NOTE 1) | Interval Time of periodic communication (average and variance) if periodic. Example: every hour |
| > Start time (NOTE 1) | Start time observed (average and variance) |
| > Duration (NOTE 1) | Duration of communication (average and variance). |
| > Traffic characterization | S-NSSAI, DNN, ports, other useful information. |
| > Traffic volume (NOTE 1) | Volume UL/DL (average and variance). |
| > Ratio | Percentage of UEs in the group (in the case of a UE group). |
| Applications (0..max) (NOTE 1) | List of applications in use. |
| > Application Id | Identification of the application. |
| > Start time | Start time of the application. |
| > Duration time | Duration interval time of the application. |
| > Occurrence ratio | Proportion for the application used by the UE during requested period. |
| > Spatial validity | Area where the service behaviour applies. If Area of Interest information was provided in the request or subscription, spatial validity may be a subset of the requested Area of Interest. |
| N4 Session ID (1..max) (NOTE 1) (NOTE 2) | Identification of N4 Session. |
| > Inactivity detection time | Value of session inactivity timer (average and variance). |
| NOTE 1: Analytics subset that can be used in "list of analytics subsets that are requested" and "Preferred level of accuracy per analytics subset". NOTE 2: This analytics subset shall only be included if the consumer is SMF. |  |

Table 6.7.3.3-2: UE Communication Predictions

| Information | Description |
| --- | --- |
| UE group ID or UE ID | Identifies the UE(s) for which the statistic applies by a list of SUPIs, or a group of UEs by a list of Internal-Group-Ids defined in clause 5.9.7 of TS 23.501 [2]. |
| UE communications (1..max) (NOTE 1) | List of communication time slots. |
| > Periodic communication indicator (NOTE 1) | Identifies whether the UE communicates periodically or not. |
| > Periodic time (NOTE 1) | Interval Time of periodic communication (average and variance) if periodic. Example: every hour. |
| > Start time (NOTE 1) | Start time predicted (average and variance). |
| > Duration time (NOTE 1) | Duration interval time of communication. |
| > Traffic characterization | S-NSSAI, DNN, ports, other useful information. |
| > Traffic volume (NOTE 1) | Volume UL/DL (average and variance). |
| > Confidence | Confidence of the prediction. |
| > Ratio | Percentage of UEs in the group (in the case of a UE group). |
| Applications (0..max) (NOTE 1) | List of applications in use. |
| > Application Id | Identification of the application. |
| > Start time | Start time of the application. |
| > Duration time | Duration interval time of the application. |
| > Occurrence probability | Probability the application will be used by the UE. |
| > Spatial validity | Area where the service behaviour applies. If Area of Interest information was provided in the request or subscription, spatial validity may be a subset of the requested Area of Interest. If a Spatial granularity size was provided in the request or subscription, the number of elements of TAs or cells in the area is smaller than or equal to the Spatial granularity size. |
| > Confidence | Confidence of the prediction. |
| N4 Session ID (1..max) (NOTE 1) (NOTE 2) | Identification of N4 Session. |
| > Inactivity detection time | Value of session inactivity timer (average and variance). |
| > Confidence | Confidence of the prediction. |
| NOTE 1: Analytics subset that can be used in "list of analytics subsets that are requested" and "Preferred level of accuracy per analytics subset". NOTE 2: This analytics subset shall only be included if the consumer is SMF. |  |

NOTE:	When Target of Analytics Reporting is an individual UE, one UE ID (i.e. SUPI) will be included, the NWDAF will provide the analytics communication result (i.e. list of (predicted) communication time slots) to NF service consumer(s) for the UE.

The results for UE groups address the group globally. The ratio is the proportion of UEs in the group for a given communication at a given time and duration.

The number of UE communication entries (1..max) is limited by the maximum number of objects provided as part of Analytics Reporting Information. The communications shall be provided by order of time, possibly overlapping.

Depending on the list size limitation, the least probable communications on a given Analytics target period may not be provided.

# * * * * Next change * * * *

#### 6.7.5.2	Input Data

The Exceptions information from AF is as specified in Table 6.7.5.2-1.

On request of the service consumer, the NWDAF shall collect and analyse UE behavioural information from the 5GC NFs (SMF, AMF, AF), or OAM as specified in clauses 6.7.2.2 and 6.7.3.2 and/or expected UE behavioural parameters from UDM as defined in clause 4.15.6.3, TS 23.502 [3], depending on Exception IDs.

NOTE:	Care needs to be taken with regards to load by avoiding to cause major extra signalling when collecting data for any UE.

Table 6.7.5.2-1: Exceptions information from AF

| Information | Description |
| --- | --- |
| IP address 5-tuple | To identify a data flow of a UE via the AF (such as the Firewall or a Threat Intelligence Sharing platform) |
| Exceptions (1..max) (NOTE 1) |  |
| > Exception ID | Indicating the Exception ID (such as Unexpected long-live/large rate flows and Suspicion of DDoS attack as defined in Table 6.7.5.3-2) of the data flow. |
| > Exception Level | Scalar value indicating the severity of the abnormal behaviour. |
| > Exception trend | Measured trend (up/down/unknown/stable) |
| > Timestamp | A time stamp of time that the abnormal behaviour occurred. |
| NOTE 1: The Exceptions information and the UE behavioural information as defined in clauses 6.7.2.2 and 6.7.3.2 could help NWDAF to train an Abnormal classifier, which could be used to classify a UE behaviour data into Normal behaviour or Exception. |  |

# * * * * Next change * * * *

### 6.8.2	Input data

The detailed information collected by the NWDAF is defined in Table 6.8.2-1.

NOTE 1:	Performance Measurements defined in TS 28.552 [8] represent resource utilisation but do not, by themselves, indicate the event of congestion or congestion levels. The NWDAF collects measurements from the OAM and how the NWDAF derives Network Status Indication (NSI) is not specified.

Table 6.8.2-1: Data Collected from the NF and OAM related to User Data Congestion Analytics

| Information | Source | Description |
| --- | --- | --- |
| Timestamp | AMF / OAM | A time stamp of time associated with the collected information. |
| UE Location | AMF | UE location information that NWDAF can use to derive the Area of Interest. |
| Measurements | OAM | Performance Measurements that will be used by the NWDAF to determine congestion levels. Performance Measurements are related to information transfer over the user plane and/or the control plane (e.g. UE Throughput, DRB Setup Management, RRC Connection Number, PDU Session Management and Radio Resource Utilization as defined in TS 28.552 [8]). The NWDAF may obtain measurements by invoking management services that are defined in TS 28.532 [6] and TS 28.550 [7]. |

Table 6.8.2-2: Data Collected from the UPF or from the AF related to User Data Congestion Analytics

| Information | Source | Description |
| --- | --- | --- |
| Application ID | UPF or AF | Application identifier as defined in clause 5.8.2 of TS 23.501 [2] (see NOTE 1). |
| IP Packet Filter Set | UPF or AF | IP Packet Filter set as defined in clause 5.8.2 of TS 23.501 [2] (see NOTE 1). |
| Measurement period | UPF or AF | Measurement period. |
| Throughput UL/DL | UPF or AF | Average Throughput UL/DL over the measurement period. |
| Throughput UL/DL (peak) | UPF or AF | Peak Throughput UL/DL over the measurement period. |
| Timestamp | UPF or AF | Time when measurements are taken. |
| Achieved sampling ratio | UPF | Sampling ratio achieved by UPF (see NOTE 2). |
| NOTE 1: Application Id and IP Packet Filter Set are mutually exclusive. NOTE 2: UPF may apply data sampling to reduce the load on the UPF. This parameter is provided when no sampling ratio is configured at the UPF or the UPF could not fulfil the configured sampling ratio. NOTE 3: Multiple outputs are provided by the UPF when multiple Service Data Flows are running at the UPF for the same UE and measurement period. |  |  |

NOTE 2:	Care needs to be taken with regards to load and major signalling caused when requesting "any UE". This can be achieved via utilization of event filters (e.g. Area of Interest), Analytics Reporting Information (e.g. maximum number of objects), or preferred sampling ratio provided by NWDAF to the UPF and/or local UPF configuration of data collection for specific application IDs, Packet Filter Sets and/or PFDs.

Additionally, NWDAF may use statistics or predictions on service experience as specified in clause 6.4.3 as an input, e.g. for service experience in a given area or service experience for some specific applications such as high bandwidth applications.

# * * * * Next change * * * *

### 6.11.2	Input Data

For the purpose of generating WLAN performance analytics results, the NWDAF collects the data as listed in Table 6.11.2-1.

Table 6.11.2-1: Data collected by NWDAF for WLAN performance analytics

| Information | Source | Description |
| --- | --- | --- |
| WLAN measurement results | OAM | The WLAN measurement results per wireless network served by the WLAN AP. |
| > SSID / BSSID / HESSID |  | SSID / BSSID / HESSID of the selected WLAN during the period of analysis. |
| > RSSI |  | Measured RSSI of the selected WLAN during the period of analysis. |
| > RTT |  | Measured RTT of the selected WLAN during the period of interest. |
| > UE Location |  | Location information tagged by UE when it reports WLAN MDT measurement (e.g. Cell ID and/or longitude/latitude if available). |
| > Timestamp |  | A time stamp of time associated with the collected information. |
| Information on PDU Session for WLAN (1..max) | SMF | Information on PDU session for which Access Type is Non-3GPP and RAT Type is TRUSTED_WLAN. |
| > UE ID |  | SUPI |
| > SSID / BSSID |  | SSID / BSSID that the PDU session is related to. |
| > Start time of the PDU Session for WLAN |  | The time stamp that indicates when the existing PDU Session's access type changes to WLAN or when the new PDU Session for WLAN is established. |
| > End time of the PDU Session for WLAN |  | The time stamp that indicates when the existing WLAN based PDU Session's access type is not WLAN any more or when the PDU Session for WLAN is released. |
| UE communications (1..max) | UPF | List of communication time slots |
| > Communication start |  | The time stamp that PDU session(s) for WLAN starts. |
| > Communication stop |  | The time stamp that PDU session(s) for WLAN ends. |
| > UL data rate |  | UL data rate of PDU session(s) for WLAN. |
| > DL data rate |  | DL data rate of PDU session(s) for WLAN. |
| > Traffic volume |  | Traffic volume of PDU session(s) for WLAN. |

NOTE 1:	WLAN Data from OAM is collected via MDT and aligned with the WLAN measurement reporting list described in clause 5.1.1.3.3 of TS 37.320 [20]. It is assumed that not all UEs support MDT WLAN measurements.

NOTE 2:	UE Location from OAM can be used to deduce WLAN location.

# * * * * Next change * * * *

### 6.11.3	Output Analytics

The NWDAF generates WLAN performance analytics. Depending on the Analytics Target Period, the output consists of statistics or predictions. The detailed information provided by the NWDAF is defined in Table 6.11.3-1 for statistics and Table 6.11.3-2 for predictions.

Table 6.11.3-1: WLAN performance statistics

| Information | Description |
| --- | --- |
| Area of Interest | A list of TAIs or Cell Ids |
| List of Analytics per SSID | SSIDs of WLAN access points deployed in the Area of Interest |
| > Time slot entry (1..max) | List of time slots during the Analytics target period |
| >> Time slot start | Time slot start time within the Analytics target period |
| >> Duration | Duration of the time slot. If a Temporal granularity size was provided in the request or subscription, the Duration is greater than or equal to the Temporal granularity size. |
| >> RSSI (NOTE 1) | Measured RSSI |
| >> RTT (NOTE 1) | Measured RTT |
| >> Traffic Information (NOTE 1) | UL/DL data rate, Traffic volume |
| >> Number of UEs (NOTE 1) | Number of UEs observed for the SSID |
| List of Analytics per UE | UE ID(s) of WLAN performance analytics |
| > Time slot entry (1..max) | List of time slots during the Analytics target period |
| >> Time slot start | Time slot start time within the Analytics target period |
| >> Duration | Duration of the time slot |
| >> RSSI (NOTE 1) | Measured RSSI |
| >> RTT (NOTE 1) | Measured RTT |
| >> Traffic Information (NOTE 1) | UL/DL data rate, Traffic volume |
| NOTE 1: This information element is an analytics subset that can be used in "list of analytics subsets that are requested" and "preferred level of accuracy per analytics subset". |  |

Table 6.11.3-2: WLAN performance predictions

| Information | Description |
| --- | --- |
| Area of Interest | A list of TAIs or Cell Ids |
| List of Analytics per SSID | SSIDs of WLAN access points deployed in the Area of Interest |
| > Time slot entry (1..max) | List of time slots during the Analytics target period |
| >> Time slot start | Time slot start time within the Analytics target period |
| >> Duration | Duration of the time slot. If a Temporal granularity size was provided in the request or subscription, the Duration is greater than or equal to the Temporal granularity size. |
| >> RSSI (NOTE 1) | Predicted RSSI |
| >> RTT (NOTE 1) | Predicted RTT |
| >> Traffic Information (NOTE 1) | Predicted UL/DL data rate, Traffic volume |
| >> Number of UEs (NOTE 1) | Number of UEs predicted for the SSID |
| >> Confidence | Confidence of the prediction |
| List of Analytics per UE | UE ID(s) of WLAN performance analytics |
| > Time slot entry (1..max) | List of time slots during the Analytics target period |
| >> Time slot start | Time slot start time within the Analytics target period |
| >> Duration | Duration of the time slot |
| >> RSSI (NOTE 1) | Measured RSSI |
| >> RTT (NOTE 1) | Measured RTT |
| >> Traffic Information (NOTE 1) | UL/DL data rate, Traffic volume |
| >> Confidence | Confidence of the prediction |
| NOTE 1: This information element is an analytics subset that can be used in "list of analytics subsets that are requested" and "preferred level of accuracy per analytics subset". |  |

# * * * * Next change * * * *

### 6.13.2	Input Data

The NWDAF supporting data analytics on Redundant Transmission Experience shall be able to collect UE mobility information from OAM, MDAS/MDAF, 5GC and AFs and service data from AF, as described in clause 6.7.2.2. In addition, NWDAF shall be able to collect the information for PDU session which is established with redundant transmission from the SMF. UE mobility information is specified in Table 6.7.2.2-1 and service data from AF related to UE mobility in Table 6.7.2.2-2. In addition, the NWDAF shall be able to collect performance measurements on user data congestion as specified in Table 6.8.2-1 for user data congestion analytics.

Additionally, the NWDAF collects the following input (see Table 6.13.2-1 and Table 6.13.2-2) according to existing measurements defined in clause 5.33.3 QoS Monitoring to Assist URLLC Service of TS 23.501 [2]. The NWDAF also collects the input from MDAS/MDAF of the end to end latency analysis in Table 6.13.2-3, as defined in clause 8.4.2.4.3 of TS 28.104 [45].

Table 6.13.2-1: Packet drop and/or packet delay measurement per QFI or GTP level

| Information | Source | Description |
| --- | --- | --- |
| UL/DL packet drop rate GTP-U | OAM (see NOTE 2) | UL/DL packet drop rate measurement on GTP path on N3. |
| UL/DL packet delay GTP | OAM (see NOTE 2) | UL/DL packet delay measurement round trip on GTP path on N3. |
| E2E UL/DL packet delay | UPF (see NOTE 3) | End-to-End (between UE and UPF) packet delay measurement between the UE and the UPF. |
| UL/DL packet drop/loss rate of RAN part | OAM (see NOTE 2) | UL/DL packet drop/loss rate measurement of RAN part. |
| Timestamp | UPF / OAM (see NOTE2, NOTE 3) | A time stamp of time associated with the collected information. |
| NOTE 1: The information in this table is provided both as the base to compare with the redundant transmission performance as well as when redundant transmission is enabled. NOTE 2: Refer to clause 5.1 of TS 28.552 [8] for the performance measurement in NG-RAN and clause 5.4 of TS 28.552 [8] for the performance measurement in UPF. In addition, Annex A of TS 28.552 [8] describes various performance measurements. NOTE 3: Refer to clause 5.33.3 of TS 23.501 [2] for the packet delay measurement in the UPF. |  |  |

Table 6.13.2-2: The information related to PDU Session established

| Information | Source | Description |
| --- | --- | --- |
| DNN | SMF | Data Network Name associated for URLLC service. |
| UP with redundant transmission | SMF | Redundant transmission is setup or terminated. |
| Timestamp | SMF | A time stamp of time associated with the collected information. |

Table 6.13.2-3: Data collection from MDAS/MDAF of end-to-end latency analysis

| Information | Source | Description |
| --- | --- | --- |
| E2ELatencyIssueType | MDAF | Indication the type of the E2E latency issue. The allowed value is one of the enumerated values: RAN latency issue, CN latency issue. |
| AffectedObjects | MDAF | The managed object instances of subnetwork, managed elements or network slices where the latency issue happens. |
| Timestamp | MDAF | A time stamp of time associated with the collected information. |

# * * * * Next change * * * *

### 6.17.2	Input Data

In Input data for the analytics ID is listed in Table 6.17.2-1. A training should be done by performing a specific LCS positioning method and comparing the position result with the ground truth UE location obtained by another method.

NOTE:	There are at least two conceivable options for the ground truth data used for the ML Model training of this analytics ID. One option is to use the known location of Positioning Reference Units (PRUs), another option is to use the positioning result of one positioning method (preferably Global Navigation Satellite System (GNSS) information such as GPS information). Collecting PRU related information by NWDAF is not supported in this release. There could be other options for ground truth data which can be determined in the future releases.

Table 6.17.2-1: Data Collected by NWDAF for Location Accuracy Analytics

| Information | Source | Description |
| --- | --- | --- |
| UE IDs | AMF | (List of) SUPI(s) |
| Finer granularity UE location | LCS NOTE 1 | UE location information. |
| >UE location |  | Geographical location or location in local coordinate (as defined in TS 23.032 [34]) |
| >positioning methods |  | The positioning methods used by the LMF when the location measurement was made |
| >LCS QoS |  | LCS QoS accuracy as defined in clause 4.1b in TS 23.273 [39] |
| >Indoor/outdoor indication |  | Indication of indoor/outdoor when LMF can distinguish indoor/outdoor environment |
| >NLOS/LOS indication |  | Indication whether the UE measurements were based on LOS/NLOS measurements |
| >Timestamp |  | A time stamp of time associated with the UE location. |
| UE location (ground truth) | AF | Timestamped UE positions |
| >UE location | NOTE 2, | GNSS location information |
| >Timestamp | NOTE 3 | Time stamp for the UE location |
| >Accuracy |  | The accuracy of the GNSS location information |
| NOTE 1: The procedure to collect location data using LCS is described in clause 6.2.12. NOTE 2: How the AF obtains timestamped GNSS location information of a UE ID is out of 3GPP scope. Whether to trust the UE location from AF is up to the implementation. NOTE 3: When AF is deployed in the untrusted domain, NEF is employed to mediate the interactions between NWDAF and AF via the Naf_EventExposure_Subscribe service specified in clause 5.2.19.2 of TS 23.502 [3], as described in clause 6.2.8.2.3. |  |  |

# * * * * Next change * * * *

### 6.18.3	Output Analytics

The NWDAF supporting E2E data volume transfer time analytics provides the analytics results to consumer NFs, e.g. AF, or NEF. The analytics results provided by the NWDAF could be E2E data volume transfer time statistics as defined in Table 6.18.3-1 or predictions as defined in table 6.18.3-2.

Table 6.18.3-1: E2E data volume transfer time statistics

| Information | Description |
| --- | --- |
| UE ID or list of UE IDs (1..SUPImax) | Identifies the UE(s)for which the statistic applies by a list of SUPIs. |
| Time slot entry (1..max) | List of time slots during the Analytics target period. |
| > Time slot start | Time slot start within the Analytics target period. |
| > Duration | Duration of the time slot. |
| > E2E data volume transfer time information (1…max) | List of E2E data volume transfer time statistics per UE. Max. is the number of UEs, if applicable. |
| >> Application ID | Identifies the application in use during the time slot |
| >> UE location | Indicate the UE location information when the service is delivered. |
| >> DNN | DNN for the PDU Session which contains the QoS flow. |
| >> S-NSSAI | Identifies the Network Slice used to access the application. |
| >> RAT Type | Indicate the list of RAT types for which the E2E data volume transfer time applies. |
| >> Access Type | Access Type when the UE establishes a PDU Session for the AF. |
| >> Validity period | The validity period within the time slot for the E2E data volume transfer time statistics as defined in clause 6.1.3. |
| >> Spatial validity | Area where the E2E data volume transfer time statistics applies. |
| >> E2E data volume transfer time UL | E2E data volume transfer time UL indicators. Statistics of E2E data volume transfer time for uplink over the time slot (e.g. average, variance). |
| >>> Data volume UL | Indicates uplink data volume used to derive E2E data volume transfer time UL. |
| >> E2E data volume transfer time DL | E2E data volume transfer time DL indicators. Statistics of E2E data volume transfer time for downlink over the time slot (e.g. average, variance). |
| >>> Data volume DL | Indicates downlink data volume used to derive E2E data volume transfer time DL. |
| > Classified E2E data volume transfer time information for a list of UEs (NOTE 1) | Classified E2E data volume transfer time statistics for multiple UEs with respect to one or more reporting thresholds (e.g. NWDAF may provide the ratio of UEs that have reached certain Reporting Threshold(s)). The list of UEs is indicated in the request of service consumer. |
| >> E2E data volume transfer time classes (1…max) (NOTE 2) | List with group of UEs classified by ranges of E2E data volume transfer time. |
| >>> UE ID(s) | Identifies the UE(s) in the transfer time class with respect to the threshold of the corresponding transfer time class. |
| >>> Ratio of UEs per E2E data volume transfer time class | Ratio of UEs. |
| >> Validity period | The validity period within the time slot for the Classified E2E data volume transfer times for a list of UEs statistics as defined in clause 6.1.3. |
| >> Spatial Validity | Area where the Classified E2E data volume transfer times statistics for a list of UEs analytics applies. |
| > Geographical distribution of the UE(s) | If requested, a list of UEs per location information. |
| NOTE 1: Analytics subset that can be used in "list of analytics subsets that are requested", "Preferred level of accuracy per analytics subset" and "Reporting Thresholds". NOTE 2: The number of transfer time classes may be pre-configured by the operator or provided by the service consumer via reporting thresholds. |  |

Table 6.18.3-2: E2E data volume transfer time predictions

| Information | Description |
| --- | --- |
| UE ID or list of UE IDs (1..SUPImax) | Identifies the UE(s) for which the predictions applies by a list of SUPIs. |
| Time slot entry (1..max) | List of time slots during the Analytics target period. |
| > Time slot start | Time slot start within the Analytics target period. |
| > Duration | Duration of the time slot. |
| > E2E data volume transfer time information (1…max) | List of E2E data volume transfer time predictions per UE. Max. is the number of UEs, if applicable. |
| >> Application ID | Identifies the application in use during the time slot. |
| >> UE location | Indicate the UE location information when the service is delivered. |
| >> DNN | DNN for the PDU Session which contains the QoS flow. |
| >> S-NSSAI | Identifies the Network Slice used to access the application. |
| >> RAT Type | Indicate the list of RAT types for which the E2E data volume transfer time applies. |
| >> Access Type | Access Type when the UE establishes a PDU Session for the AF. |
| >> Validity period | The validity period within the time slot for the E2E data volume transfer time prediction as defined in clause 6.1.3. |
| >> Spatial validity | Area where the E2E data volume transfer time prediction applies. |
| >> E2E data volume transfer time UL | E2E data volume transfer time indicators. Predictions of E2E data volume transfer time for uplink over the time slot (e.g. average, variance). |
| >>> Data volume UL | Indicates uplink data volume used to derive E2E data volume transfer time UL. |
| >> E2E data volume transfer time DL | E2E data volume transfer time DL indicators. Predictions of E2E data volume transfer time for downlink over the time slot (e.g. average, variance). |
| >>> Data volume DL | Indicates downlink data volume used to derive E2E data volume transfer time DL. |
| > Classified E2E data volume transfer time information for a list of UEs (NOTE 1) | Classified E2E data volume transfer time prediction for multiple UEs with respect to one or more reporting thresholds (e.g. NWDAF may provide the ratio of UEs that have reached certain Reporting Threshold(s)). The list of UEs is indicated in the request of service consumer. |
| >> E2E data volume transfer time classes (1…max) (NOTE 2) | List with group of UEs classified by ranges of E2E data volume transfer time. |
| >>> UE ID(s) | Identifies the UE(s) in the transfer time class with respect to the threshold of the corresponding transfer time class. |
| >>> Ratio of UEs per E2E data volume transfer time class | Ratio of UEs. |
| >> Validity period | The validity period within the time slot for the Classified E2E data volume transfer times for a list of UEs prediction as defined in clause 6.1.3. |
| >> Spatial Validity | Area where the Classified E2E data volume transfer times prediction for a list of UEs applies. |
| > Geographical distribution of the UE(s) | If requested, a list of UEs per location information. |
| > Confidence | Confidence of this prediction. |
| NOTE 1: Analytics subset that can be used in "list of analytics subsets that are requested", "Preferred level of accuracy per analytics subset" and "Reporting Thresholds". NOTE 2: The number of transfer time classes may be pre-configured by the operator or provided by the service consumer via reporting thresholds. |  |

NOTE:	When Target of Analytics Reporting is an individual UE, one UE ID (i.e. SUPI) will be included, the NWDAF will provide the analytics transfer time result (i.e. list of (predicted) time slots) to NF service consumer(s) for the UE.

# * * * * Next change * * * *

### 6.19.2	Input data

The detailed input data collected by the NWDAF for relative proximity analytics is listed in Table 6.19.2-1, Table 6.19.2-2 and Table 6.19.2-3.

Table 6.19.2-1: OAM input data for relative proximity analytics

| Information | Source | Description |
| --- | --- | --- |
| Per UE information | OAM |  |
| > Speed |  | UE Speed |
| > Orientation |  | UE Orientation |
| > Timestamp |  | A time stamp of time associated with the UE information. |

NOTE 1:	To collect the input data in Table 6.19.2-1, NWDAF can invoke TraceJob with the UE ID (e.g. SUPI) of the UE as described in clause 4.3.30 of TS 28.622 [41] using Generic Provisioning Management Service as described in clause 11.1 of TS 28.532 [6] to get Sensor Information as part of MDT data collection mechanism. The UE sensor information includes Barometric pressure, UE speed and UE orientation as described in clause 5.10.29 of TS 32.422 [42].

Table 6.19.2-2: Proximity related input data collected via DCAF/NEF

| Information | Source | Description |
| --- | --- | --- |
| Proximity Attribute | DCAF / NEF | Characterise a set of UEs in relative proximity. |
| > Number of UEs |  | Total number of UEs in proximity of target UE. |
| > Timestamp |  | A time stamp of time that the proximity attribute derived. |
| > Application ID(s) |  | Identifying the applications(s) providing this information. |
| > List of UE IDs |  | UE IDs in proximity of target UE if available. |
| > Other attribute(s) |  | Other attributes for the set of UEs i.e. destination, route, average speed, time of arrival. |
| > Confidence |  | Confidence on relative proximity data. |

NOTE 2:	The list of UE IDs that fulfil a proximity criterion can be determined by NWDAF based on the input information collected from the UE application via DCAF. The UE does not need to determine proximity information related to other UEs.

NOTE 3:	When DCAF is deployed in untrusted domain, NEF is employed to mediate the interactions between NWDAF and DCAF via the Naf_EventExposure_Subscribe service specified in clause 5.2.19.2 of TS 23.502 [3], as described in clause 6.2.8.2.3.

Table 6.19.2-3: Proximity related input data from 5GC/AF

| Information | Source | Description |
| --- | --- | --- |
| Timestamp | AMF / GMLC / LCS / AF | A time stamp of time associated with the collected information. |
| UE ID(s) | AMF | SUPI(s) of individual UE(s). |
| UE location | AMF, GMLC | Absolute location of an UE. |
| UE location accuracy | GMLC | Accuracy of the location estimation. |
| Velocity estimate | LCS | UE velocity. |
| Collision risk distance | AF | Derived from the UE connected physical object dimensions e.g. length, width, height. |
| UE heading | AF | UE moving direction. |
| > Absolute heading |  | Heading of the UE movement with respect to the true north. |
| > Relative heading |  | Heading of the UE movement with respect to another UE. |
| UE trajectory | AF | Timestamped UE positions. |
| >UE location |  | Geographical area where the UE is located. |
| >Timestamp |  | Time stamp for the UE location. |

# * * * * Next change * * * *

### 6.21.3	Output analytics

The output for analytics of movement behaviour provided by NWDAF is defined in Table 6.21.3-1 and Table 6.21.3-2.

Table 6.21.3-1: movement behaviour statistics

| Information | Description |
| --- | --- |
| Applicable Area | A geographical area as defined in TS 23.273 [39] that the analytics applies to. |
| Applicable UEs | A list of SUPIs for the applicable UEs (NOTE 2) |
| Time slot entry (1..max) | List of time slots during the Analytics target period |
| > Time slot start | Time slot start within the Analytics target period |
| > Duration | Duration of the time slot |
| > Total number of UEs (NOTE 1) | Total number of UEs in the area of interest |
| > Ratio of moving UEs (NOTE 1) | Ratio of moving UEs in the area of interest |
| > Average speed (NOTE 1) | Average speed of all UEs in the area of interest |
| > Speed threshold (1...max) (NOTE 1) | Threshold utilized to filter the UEs |
| >> Number of UEs | The number of UEs whose speed is faster than the speed threshold |
| >> Ratio | Percentage of UEs whose speed is faster than the speed threshold |
| > Direction (1…max) (NOTE 1) | Heading directions of the UEs in the target area, such as north, south, east, west etc. |
| >> Number of UEs | The number of UEs in the specific direction |
| >> Average speed | Average speed of UEs in the specific direction |
| >> Ratio | Percentage of UEs in the specific direction |
| NOTE 1: Analytics subset that can be used in "list of analytics subsets that are requested" and "Preferred level of accuracy per analytics subset". NOTE 2: If Target of Analytics Reporting is set to a "List of UEs", the statistics are estimated considering only UEs in the area of interest that are also in the provided List of UEs. |  |

Table 6.21.3-2: movement behaviour predictions

| Information | Description |
| --- | --- |
| Applicable Area | A geographical area as defined in TS 23.273 [39] that the analytics applies to |
| Applicable UEs | A list of SUPIs for the applicable UAV UEs (NOTE 2) |
| Time slot entry (1..max) | List of time slots during the Analytics target period |
| > Time slot start | Time slot start within the Analytics target period |
| > Duration | Duration of the time slot |
| > Total number of UEs (NOTE 1) | Total number of UEs in the area of interest |
| > Ratio of moving UEs (NOTE 1) | Ratio of moving UEs in the area of interest |
| > Average speed (NOTE 1) | Average speed of all UEs in the area of interest |
| > Speed threshold (1...max) (NOTE 1) | Threshold utilized to filter the UEs |
| >> Number of UEs | The number of UEs whose speed is faster than the speed threshold |
| >> Ratio | Percentage of UEs whose speed is faster than the speed threshold |
| > Direction (1..max) (NOTE 1) | Heading directions of the UEs in the target area, such as north, south, east, west etc. |
| >> Number of UEs | The number of UEs in the specific direction |
| >> Average speed | Average speed of UEs in the specific direction |
| >> Ratio | Percentage of UEs in the specific direction |
| > Confidence | Confidence of the prediction |
| NOTE 1: Analytics subset that can be used in "list of analytics subsets that are requested" and "Preferred level of accuracy per analytics subset". NOTE 2: If Target of Analytics Reporting is set to a "List of UEs", the statistics are predicted considering only UEs in the area of interest that are also in the provided List of UEs. |  |

# * * * * End of changes * * * *