3GPP TSG SA WG2 Meeting #170	S2-2506481

25- 29 August, 2025, Goteborg, Sweden	(revision of S2-220xxxx)

Source:	China Mobile

Title:	[WT#1.2] Working tasks and key issues for 6G localized network

Document for:	Agreement

Agenda Item:	20.6.6

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: This paper proposes the discussion of 6G architecture to support 6G localized network, and proposes the WT and KI for 6G localized network

1. Motivation and Justification of 6G localized network

  - Summary of SA1 requirements related to localized network

As discussed in TR 22.870 of SA1, some of the solutions are related to the localized network requirements aspects. In Table 1.1-1, it shows the computing related PRs in SA1 that till SA1#100 meeting.

Table 1.1-1: SA1 localized network related requirements collection till SA1#110 meeting

| Category | PR details |
| --- | --- |
| 6G localized network for vertical | - Support authorizing UEs owned by a trusted authorized 3rd party (e.g. vertical industry) to access services provided by a localized network (deployed by the PLMN operator for the authorized 3rd party). - Support a mechanism to authorize and authenticate UEs owned by a trusted authorized 3rd party to access services from the PLMN operator |
| 6G Local Area Networks | Support a mechanism to start and stop offering certain network service(s) in a local area network adapting to the demand of e.g. the users, 3rd party or the network operator. |

As shown above, the main uses case for localized network is providing localized networks for 3rd party(e.g. vertical industry).

Verticals prefer to have their own localized network, due to concerns about data privacy and security. Verticals are more inclined to exercise more local control over localized networks under the lightweight control of the PLMN, such as requiring the ability to locally UE authorization and authentication within the localized network, local subscription, local signaling control, and local forwarding of UE traffic. Different vertical industries always have different requirements for the localized network. The operator should provide differentiated networks or capabilities for different vertical industries. For some specific industries, such as government affairs, education, culture and tourism, and healthcare, the users have the need to access both the dedicated localized network and the PLMN services at the same time. This requires that the UE can access to both the localized networks and the PLMN simultaneously. A vertical industry may have multiple dedicated localized networks deployed in different areas, so some localized networks need to be able to interconnect with each other.

  - Analysis of technical impacts

- Network localization

In order to achieve the local control of localized networks by verticals, such as the ability to locally UE authorization and authentication, local subscription, local signaling control, and local forwarding of user traffic, it is necessary to deploy both the control plane NF and the user plane NF locally. Therefore, it is required to push down the relevant network functions to the local level, such as AMF, SMF, UDM, AUSF, PCF, NEF, and UDM.

Preferring more convenient lightweight management, the vertical requires to have the capability to authorize or authenticate UEs locally within the local network. The UE that is only authorized and authenticated by the local network for the vertical can only access the services provided by the local network. If the UE also requires access to the PLMN, the interaction between local operator’s network for the vertical and PLMN is needed, while AAA server and NSSAAF may not be required.

Regarding the authorized activation of localized networks, it is advisable to consider that the PLMN pre-activates the localized network permissions, and the localized network performs the addition, deletion and modification local UE subscription data. After the operations are successful, the localized network reports network data such as policies, subscriptions, and other information to the PLMN. Therefore, the localized network requires a local UDM, the UDM of PLMN and the UDM of localized network need to interoperate.

For vertical industries to achieve signaling control centered on the localized network, it is necessary for the signaling control anchor point and policy decision-making authority to be in the localized network. The UE should register in the localized network, and the PDU session establishment, service request, policy control, etc., should also be conducted within the localized network.

Observation 1: In order to support the relevant capabilities of the localized network, it is necessary to study which network functions should be deployed to the localized network, as well as the localization of related procedures such as authentication, authorization, access management, and session management.

- Interaction between localized network, and interaction between localized network and the PLMN

The UE needs to access both the localized network and the PLMN simultaneously, and the localized networks need to be able to interconnect with each other. This requires mutual access between different localized networks and inter connectivity between the localized networks and the PLMN. In addition, to ensure the security and privacy of vertical industries, the localized networks need to implement topology hiding from the outside. Therefore, interactions between the localized network or between the localized network and the PLMN need to be conducted through a localized network proxy to achieve isolation and inter connectivity of the the localized network. As a result, enhancements of the service-based framework are needed, and the localized network proxy design based on the NRF/SCP can be considered.

Observation 2: It is necessary to study how to expand capabilities based on the mechanism of NRF/SCP, to achieve the management of localized networks expanding from the management of functions/services. The distributed topology management and control mechanism of the localized networks should also be studied.

**** First Change ****

Annex A.X. WT#X Scope

The study includes the following high level work tasks, and the conclusion will consolidate the 6G architecture among all work tasks:

WT#1: Define the overall 6G architecture as collection of capabilities and high level functionalities considering the following sub work tasks and other work tasks to support 6G access network:

1.2.	Study whether and how to support and/or enhance the following aspects in 6G: the SBA framework, network slicing, network sharing, user plane architecture, QoS framework, policy framework, network exposure framework, architecture for specific scenarios e.g. fixed wireless access, localized service access.

**** Second Change ****

5.X. Key Issue #X: Support of Localized Network Services

### 5.X.1	Description

This key issue will address the general system architecture and function requirement to support localized network service. Different verticals always have different requirements for the localized network. The 6G network should be able to provide customized local network for different verticals, e.g., localized authentication for vertical UEs.

For NPN network, this cannot be used currently because of NID, which is not supported by UE. Therefore, for localized network service access, there should be no UE impact.

What’s more, network simplification should be considered when deployed the localized network.

The aspects should be studied in this key issue include:

-	Whether and how to support the localized NFs to support basic network mechanism.

-	How to support the authentication and authorization locally, and how to keep the information alignment between the local subscription information and central subscription information.

-	How to support the UE access to localized network.

-	How to decide that the UE should be able to access to localized network or centralized network.

-	During the mobility, how to decide that the UE that should be access to localized network or centralized network, and how to support the service continuity.

**** End of Changes ****