SA WG2 Meeting #170	S2-2506635

25 – 29 Aug, 2025, Goteborg, SE

Source:	vivo

Title:	[General aspects] Architectural Assumptions and Requirements for 6G

Document for:	Agreement

Agenda Item:	20.6.0

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: this contribution introduces the Architectural Assumptions and Requirements.

1. Justifications

1.1 Discussion on Architectural Assumptions

In 5GS, there are some overlapping functionalities, such as, both the CN and RAN can trigger paging, which leads to redundant signalling, and higher resource consumption.

Defining a clear functional split between the 6G core and access network (AN) is crucial to avoid the existing problem in 5GS.

Proposal 1：The functional split between 6G core and 6G AN shall be defined and avoid the overlapping functionality in both 6G core and 6G AN (e.g. CN triggered paging, RAN trigger paging).

In 5GS, Network Slice is one of the core innovations. However, after multiple release of study and evolution as the following table, network slicing has become extremely bloated, greatly increasing the processing logic and complexity of networks and UE.

Studying simplified and non - mandatory network slice support for UEs is necessary to accommodate the diverse range of 6G devices. Not all UEs, especially low - cost and resource - constrained devices, require the full complexity of network slicing. Simplifying this feature reduces UE implementation costs, power consumption, and complexity, while still allowing for the delivery of tailored services when needed.

Proposal 2：If network slice is supported, it is not mandatory for a UE and the feature should be simplified to minimize the impact to the UE.

The Policy mechanism of 5G has achieved significant breakthroughs in resource scheduling and service differentiation, however in the face of the higher-dimensional requirements of 6G such as intelligence, ubiquity, intrinsic security and autonomy, new service (i.e., beyond communication), its architecture, capabilities and adaptability still have obvious limitations needing to be overcome. Overall, 6G policy framework needs to support both communication services and new services that beyond communication, specifically it needs to consider how to simplified UE policy

Proposal 3：if the UE policy is supported, it should be simplified to minimize the impact to the UE.

In 5GS, interworking with 4G was an important aspect. The QoS parameter of 5G and 4G is different, to make the UE and PGW can get the 4G QoS parameters locally when UE transferring from 5G to 4G, the 4G QoS parameter should be preconfigured to UE when UE the QoS flow is established in 5G.

In 6GS, if the UE transfers from 6G to 5G, if the UE does not have corresponding 4G parameters, the UE can not further transfer to 4G from 5G, which will cause service interruption for UE.

Proposal 4：For supporting interworking, support the scenario that the UE transfer to 4G immediately after transfer from 6G to 5G.

In 5G, the QoS and policy frameworks were mainly focused on traditional communication services.

Beyond communication services, such as integrated sensing and communication is introduced in 6G, a uniform QoS/Policy framework for both communication and beyond - communication services is necessary.

Proposal 5：A uniform QoS/Policy framework is assumed to compatible for both communication and beyond communication services.

In 6G, supporting new services like the integration of sensing and communication over 3GPP access and distributed computing is crucial. On - demand deployment of these services allows operators to quickly respond to market demands, and researching this helps in defining the necessary network capabilities, interfaces, and resource management strategies for their successful implementation.

Proposal 6：Support beyond the following new services and each of them is allowed on demand deployment.

1.2. Discussion on Architectural Requirements

The 6G can inherits and enhance some 5G requirements.

Proposal 7: it suggests that the 6G inherits and enhance the following 5G requirements:

- Support the new 6G RAT(s) and non 3GPP access.

- Support the functional split between 6G core and 6G AN.

- Define the interface between 6G Core and 6G RAN supporting 6G RAT(s).

- Support multiple simultaneous connections of a UE via multiple access technologies.

- Support and/or enhance the essential/regulatory services (i.e. voice, Messaging, location services, Emergency services, MPS, Mission Critical services, PWS).

- Support network sharing, roaming, different levels of UE mobility, etc.

In 5G, the AMF is the anchor of NAS signalling, it couples with other NFs, e.g., SMF. When adding a new NF or updating an existing NF, the AMF will also be updated. In 6G, this problem should be avoided.

Proposal 8: Leverage techniques for simplicity and flexibility for deploying and offering new services, e.g., support decoupling among different functionalities, e.g., when introducing new non-access stratum functionality to the 6G architecture, minimize the impact to other non-access stratum functionalities.

The 6G introduces some new services, e.g., AI, data framework. These new services may have influence to the 6G architecture.

Proposal 9: The following new services should be considered.

- Support and enable network for AI and AI for network.

- Support data framework for all aspects related to efficient and scalable data handling

- Support different means for reducing UE power consumption, especially optimize the energy efficiency without reduce the large bandwidth transmission for a light-weight UE (e.g., AR glasses).

- Support interworking to 5GS and support interworking between 6GS and 4G/5G NTN/satellite access that use EPS/5GS.

**** First Change ****

4	Architectural Assumptions and Requirements

4.1	Architectural Assumptions

Editor's Note: This clause documents the common architecture assumptions identified for the study.

The architecture assumption of the 6G System shall:

1. A unique kind of 6GS architecture.

Editor's Note:  standalone architecture or non-standalone architecture will be determined in SA2#171.

2.  The functional split between 6G core and 6G AN shall be defined and avoid the overlapping functionality in both 6G core and 6G AN (e.g. CN triggered paging, RAN trigger paging).

3.  If network slice is supported, it is not mandatory for a UE and the feature should be simplified to minimize the impact to the UE.

4.  if the UE policy is supported, it should be simplified to minimize the impact to the UE.

5.  For supporting interworking, support the scenario that the UE transfer to 4G immediately after transfer from 6G to 5G.

6. A uniform QoS/Policy framework is assumed to compatible for both communication and beyond communication services.

7.  Support the following new services and each of them is allowed on demand deployment.

-    Support integration of Sensing and Communication over 3GPP access.

-    Support of computing for UE, core network and application server in 6G.

8. 6G UE may or may not have AI capabilities.

**** Second Change ****

4.2	Architectural Requirements

Editor's Note: This clause defines the architectural requirements that serve as the foundation for the study.

1.	Support the new 6G RAT(s) and non 3GPP access.

2.	Support the functional split between 6G core and 6G AN.

3.	Define the interface between 6G Core and 6G RAN supporting 6G RAT(s).

4.   Support multiple simultaneous connections of a UE via multiple access technologies.

5.   Support and/or enhance the essential/regulatory services (i.e. voice, Messaging, location services, Emergency services, MPS, Mission Critical services, PWS).

6. Support network sharing, roaming, different levels of UE mobility, etc.

7.  Leverage techniques for simplicity and flexibility for deploying and offering new services, e.g. support decoupling among different functionalities, e.g. when introducing new non-access stratum functionality to the 6G architecture, minimize the impact to other non-access stratum functionalities.

8.  Support and enable network for AI and AI for network.

9. Support data framework for all aspects related to efficient and scalable data handling

10.  Support different means for reducing UE power consumption, especially optimize the energy efficiency without reduce the large bandwidth transmission for a light-weight UE (e.g., AR glasses).

11.  Support interworking to 5GS and support interworking between 6GS and 4G/5G NTN/satellite access that use EPS/5GS.

**** End of Changes ****