3GPP TSG-SA WG2#170	S2-2506915

Goteborg, Sweden, 25-29 August 2025	(was S2-2505046)

Source:	Qualcomm Incorporated

Title:	KI#4: Support for location information for emergency calls over NB-IoT NTN

Document for:	Discussion/Approval

Agenda Item:	20.1.1

Work Item / Release:	FS_5GSAT_Ph4_ARC/ Rel-20

Abstract of the contribution: Solution to address the support for location information for emergency calls over NB-IoT NTN.

# 1.	Discussion

Proposes changes in the procedures to support IMS emergency calls over NB-IoT NTN connected to EPC.

# 2.	Text proposal

It is proposed to agree the following changes vs. TS 23.700-19:

# >>>>BEGINNING OF CHANGES<<<<

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[3]	3GPP TS 23.502: "Procedures for the 5G system, Stage 2".

[4]	3GPP TS 23.503: "Policy and Charging Control Framework for the 5G System".

[5]	3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access".

[6]	3GPP TS 23.228: "IP Multimedia Subsystem (IMS); Stage 2".

[7]	3GPP TS 23.167: "IP Multimedia Subsystem (IMS) emergency sessions".

[8]	3GPP TS 22.261: "Service requirements for the 5G system".

[9]	3GPP TS 22.101: "Service aspects; Service principles".

[y]	3GPP TS 23.271: "Functional stage 2 description of LCS".

# >>>>NEXT CHANGE<<<<

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |
| --- | --- | --- | --- | --- |
| Solutions | #1 | #2 | #3 | #4 |
| #1 | X | X |  |  |
| #2 | X |  |  |  |
| #3 | X |  |  |  |
| #4 | X | X | X | X |
| #5 | X |  |  |  |
| #6 | X |  |  |  |
| #7 | X | X |  |  |
| #8 | X |  |  |  |
| #9 | X |  |  |  |
| #10 | X |  |  |  |
| #11 | X |  |  |  |
| #12 |  | X |  |  |
| X |  |  |  | X |

# >>>>NEXT CHANGE (new text)<<<<

## 6.X	Solution #X: Support for location service for emergency call over NB-IoT NTN

### 6.X.1	High level principles

This solution has the following high level principles, in order to support location service for emergency call over NB-IoT NTN connected to EPC:

- 	UE reports its coarse location information to the MME following the procedures defined in TS 23.401 [5] clause 4.13.10.

-	UE reports its location information to the IMS Core during session establishment. The ECGI provided in the location information by the UE can differ from the ECGI provided by eNB.

-	If the location information provided by UE is insufficient:

-	If the operator policy requires network provided location using PCC-based solutions for the UE location (e.g. serving cell), the P-CSCF retrieves location information from the access network and includes it in the emergency service request, as described in TS 23.228 [6]. The UE location is marked as being network provided to distinguish it from location information that the UE provided.

-	If the operator policy requires network provided location using the GMLC/LRF-based solution for the UE location (e.g. serving cell), the P-CSCF forwards the emergency request to E-CSCF. The E-CSCF retrieves location information from LRF as described in TS 23.167 [7] clause 6.2.2.

### 6.X.2	Description

This is a solution for Key issue #4 "Location service for IMS emergency call and regulatory services over NB-IoT NTN".

Location information is needed for 2 main reasons in emergency services. The initial purpose of the location information is to enable the IMS network to determine which PSAP serves the area where the UE is currently located, so that the IMS network can route the emergency session to the correct PSAP. The second purpose is for the PSAP to get more accurate or updated location information for the terminal during or after the emergency session where required by local regulation.

As specified in TS 23.167 [7], following options for retrieving location information for routing purposes are considered:

1.	The UE knows its own location;

2.	The UE retrieves its location information from the network;

3.	The IMS core retrieves the location information;

4.	Location information is not needed to route the emergency call by the IMS core, optionally the emergency routing determination and location information retrieval may be performed by the Emergency Call Server (ECS) as part of the emergency session establishment procedure.

Option 2 and option 4 requires more signalling exchange via control plane or data transmission via user plane, which may cause significant call setup delay in the context of IMS emergency call over NB-IoT NTN. While option 1 and option 3 are partially supported in the specifications of Rel-19.

Based on the above observation, option 1 and option 3 are recommended as baseline for retrieving location information. More details are proposed based on the existing specifications:

Option 1, UE reported location information:

As specified in TS 23.401 [5], in the case of satellite access for NB-IoT, a UE with location capability (i.e. GNSS capability) should use its awareness of its location to select a PLMN.

The requirement for emergency service is the UE reporting its location information to the IMS Core during session establishment.

Option 3, IMS Core retrieved location information:

As specified in TS 23.401 [5], in order to ensure that regulatory requirements are met, the network may verify the UE location.

As specified in TS 23.271 [y] clause 9.1.17.0, the MME may further report the UE location estimation and accuracy to the GMLC/LRF.

If the location information provided via option 1 is insufficient or if the IMS core requires emergency routing information, or if the IMS core is required to validate the location information, or if the IMS core is required to map the location identifier received from the UE into the corresponding geographical location information, the IMS core sends a location request to the LRF.

Proposals:

In the context of IMS emergency call over NB-IoT NTN, it is recommended to apply both of the options for retrieving location information:

Option 1, UE reported location information;

Option 3: IMS Core retrieved location information.

Other options (e.g., retrieving location information via SUPL, LPP) may be supported as supplementary of the above two options.

### 6.X.3	Procedures

The procedures specified in TS 23.401 [5], TS 23.167 [7], TS 23.271 [y] can be reused. The overall procedures of retrieving location information are illustrated in figure 6.X.3-1.

Figure 6.X.3-1: UE location retrieving for emergency call over NB-IoT NTN procedures

1.	The user initiates an emergency service request to establish the IP-CAN bearer over EPS via NB-IoT NTN. The UE reports Coarse Location Information (as specified in TS 23.401 [5]) from the UE to MME in the NAS Security Mode Command procedure.

2.	If the MME is not able to determine the UE location with sufficient accuracy or if the received ULI is not sufficiently reliable, the MME initiates UE location procedure to E-SMLC as specified in clause 9.1.17.0 of TS 23.271 [y], to determine the UE location. If the requested location information and the location accuracy within the QoS can be satisfied based on parameters received from the MME, the E-SMLC may skip step 3 and  send a Location Response immediately.

3.	E-SMLC determines the positioning method and performs position measurement procedures as specified in TS 23.271 [y].

4.	E-SMLC returns location information estimation to the MME.

5.	The MME reports location information estimation to the GMLC, as specified in clause 9.1.17.0 of TS 23.271 [y].

6.	The GMLC sends ACK to the MME. The LRF integrated or associated with the GMLC may use the location information to assist routing of the emergency session to the PSAP/emergency center.

7.	The UE sends an INVITE with an emergency indication to the IMS core. The INVITE should contain any location information that the terminal has. If the location information provided by UE is trusted and sufficient to determine the correct PSAP, the step 8-9 are skipped.

8.	If the location information is insufficient, the IMS core sends a location request to the LRF as specified in clause 7.6.1 of TS 23.167 [7].

9. The LRF returns with location and route information.

10. The IMS Core selects the PSAP based on UE location information and forwards the INVITE to the selected PSAP with location information.

11.	 The emergency call is established between the UE and the PSAP.

### 6.X.4	Impacts to Services, Entities and Interfaces

Editor's note:	This clause captures impacts on existing 3GPP nodes and functional elements.

UE

-	Report its coarse location information to MME via SMC procedure defined in TS 23.401 [5] .

-	Report its location information to the IMS Core during session establishment.

-	Support E-SMLC requested UE Assisted and UE Based Positioning and Assistance Delivery.

MME

- Receives UE	coarse location information using existing procedure.

-	Support EPC Network Induced Location Request using existing procedure.

-	Reports the location information to GMLC/LRF using existing procedure.

GMLC/LRF:

-	Retrieve and store the UE location information using existing procedure.

-	Provide routing and/or correlation info of an UE to IMS Core using existing procedure.

# >>>>END OF CHANGES<<<<