SA WG2 Meeting #170	S2-2506924

25th – 29th August 2025, Goteborg, SE

Source:	Lenovo

Title:	[WT#6, Computing] Key Issue for Computing in 6G

Document for:	Agreement

Agenda Item:	20.6.6

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: This contribution intends to give the justifications for the research of computing in the 6G (among UE, core network and application server), the scope and the key issue description.

Editor's Note: We encourage companies to include a short summary in their abstract of the contributions, mainly to help readers quickly understand the intent and scope of the contribution.

1. Justifications

Editor's Note: This justification section is not included in the TR; it is intended solely to justify the scope of the proposed WT.

This contribution aims to provide the key issue descriptions for the WT#6 about the computing for 6G, together with the justifications. The agreed WT#6 wording in the 6G SID is copied below:

WT#6: Study aspects on support of computing for UE, core network and application server in 6G (e.g. coordination between UE, core network and application server, exposure of computing service in the core network, etc.).

NOTE 7: Application layer mechanism and exposure framework may require coordination with SA6.

1.1 Architecture Motivation

As documented in the 3GPP TR 22.870, there are many use cases for computing enhancements, such as for XRM rendering, AI inference, immersive communication. Such use cases motivate the need for the 6G system to execute compute tasks requested by an application in the UE or by a third-party AF, which avoids the needs of the app provider to deploy an infrastructure for computing services.

For example, the APP running on the UE can offload the computing task to the network, by providing the executable code, input data, and specific execution/computing requirements (e.g., latency, security requirements, required computing resource parameters), based on an SLA with the operator, and get the processed results from the network when the computing is finished. The whole process can be regarded as a new service provided by the 6G system, i.e., computing as a service, which could make the 6G system more intelligent and robust.

1.2 Gap Analysis

Edge computing has been extensively studied during the past releases and the existing 5G architecture for edge computing is designed for UEs to connect to pre-deployed applications. In detail, the uplink data traffic from UE will be selectively routed to the edge application server (EAS) that resides in the local data network, which is closer to the UE, based on the latency requirement provided by the application function, and the potential EAS server information. The EAS server is dedicated for a specific kind of APP, which is as same as the one that sends the uplink traffic from UE (APP client at UE).

However, the current edge computing mechanism does not natively support the dynamic submission of arbitrary compute tasks for execution by the network.

For example, the network is not comprehensively aware of the available computing resources, either within the network, or in the edge, and makes full use of them.

On the other hand, the computing cannot be provided as an on-demand service to the service consumers, which can be UE or a third-party AF, and will not satisfy the ever-increasing computing need from many emerging applications, e.g., gaming, XR, and ChatGPT.

Therefore, architectural enhancements need to be studied to enable the 6G system to offer compute as an on-demand service.

  - Technical impact

The potential technical impacts related to introducing computing in the 6G system, i.e., computing as a service, may include the following:

- Support the functionality of orchestration the computing in the network, which could involve receiving the computing service request, discovery of the appropriate computing resources, handling the computing task and coordinating the communication and computation requirements simultaneously.

**** First Change ****

Annex A.X. WT#6 Scope

Editor's Note: Describe the technical scope of the proposed Work Task. If applicable, suggest logical subdivision of this WT into smaller sub-WT. This clause is part of the TR Annex.

The technical scope for WT#6 should include:

- Study how to support network-provided computing service to be used by authorized consumers.

**** Second Change ****

5.X. Key Issue for WT#6: Architectural enhancements for network-provided computing services

Editor's Note: This clause defines the potential scope of KI(s) and is part of the TR.

The following key issues should be studied to support computing in the 6G:

- How an authorized consumer (e.g., a UE or a third-party AF) can request a computing service from the network.

- What enhancements to existing Network Functions or what new Network Functions and interfaces are needed to control and manage the lifecycle of a network-provided computing service, e.g., authorization, handling and monitoring.

- How to discover and select the appropriate computing node to process a computing task by leveraging network-available information, e.g., UE location, network conditions, the supported computing capability of the computing nodes that reside in both the core network and edge.

- How to configure the computing task, establish the path between the service consumer and the selected computing node, and guarantee the QoS requirements.

- How aspects such as UE mobility, charging, and policy control apply to these network-provided computing services.

NOTE: Coordination with SA5 and SA6 is needed when solid requirements are identified

**** End of Changes ****