SA WG2 Meeting #170	S2-2507545

# Goteborg, SE, Aug 25 - 29, 2025				 					(revision/merge of 6280+6830+6332+6910)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.228 | CR | 1653 | rev | 1 | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correcting the avatar ID list downloading procedure and the network centric avatar rendering procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Ericsson, Samsung, Qualcomm |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RTC_Ph2 |  |  |  |  |  | Date: |  |  | 2025--08-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The description of avatar ID list downloading procedure and UE centric / network centric procedure is not clear. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correction to description in AC.11.3.3 Explain in the network centric procedure, what type of ADC would be needed based on the negotiation result Clarify that the associated URL is used to download Avatar representation of an Avatar ID. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is not explained in the network centric procedure, what type of ADC would be needed based on the negotiation result. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | AC.11.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev1 - step 5/15 of figure - Note 1 : UE can determine - Merged in 6830+6332 - Step 20 of network centric: Based on the type of network rendering mode decided in step 4, either option 1 (MF rendering mode, steps 21a-26a) or option 2 (DC AS rendering mode, steps 21b-26b) is performed in steps 22-26. - Step 25/26 of network centric: 25-26. MF or DC AS sends the animated video stream to UE2 and sends the animated video stream to UE1 if required. - Updated reason for change QC 6910 and Nokia 6280 are handling the same clauses (AC.11.3.1, AC.11.3.2.1, AC.11.3.2.2, AC.11.3.3) and have some clash (major on editorial changes). this paper only handles AC.11.3.3 Network centric procedure. QC paper handles AC.11.3.1, AC.11.3.2.1, AC.11.3.2.2. |  |  |  |  |  |  |  |  |

# >>>>START OF CHANGES<<<<

### AC.11.3.3	Network centric procedure

Figure AC.11.3.3-1: Procedures of network centric IMS Avatar communication

Figure AC.11.3.3-1 depicts a typical call flow procedure of network centric IMS avatar communication. The main steps in the call flow are as follows:

1.	IMS session and bootstrap data channel have been established. UE1 downloads avatar communication application. The avatar ID list is downloaded as described in clause AC.11.1. Then UE1 chooses an avatar ID and runs the avatar communication application.

2.	UE1 decides to request network to perform avatar animation based on its status such as power, signal, computing power, internal storage, etc.

3.	UE1 initiates application data channel establishment procedure. Application data channel(s) are established between UE1 and DC AS and UE2 and DC AS.

4.	The avatar animation negotiation is finished between the DC AS and UE1 via the application data channel established in step 3.

Editor's note:	The avatar animation negotiation procedure and parameters need to be further defined in SA WG4 and cooperation with SA WG4 is needed.

5.	If the negotiation result in step 4 is that the network will perform the rendering, the UE1 initiates new P2A2P or P2P application data channels, which are specifically used for avatar rendering-related data transmission between the UE1/UE2 and the network.

NOTE 1:	Based on the negotiation result in step 4, the UE1 decides what type of application data channel would be needed for network centric procedure. For example, if the negotiation result is MF rendering mode, the UE1 can determine that a P2P application channel is needed; if the negotiation result is DC AS rendering mode, the UE1 can determine that a P2A2P application channel is needed.

6-7.	During the application data channel establishment procedure, the IMS AS notifies the DCSF via Nimsas_SessionEventControl_Notify of the media change request event. The DCSF checks that the app id in the ADC establishment request belongs to Avatar communication DC app, subsequently, then the DCSF fetches DC AS URL from DCSF service specific data or configured locally in DCSF.

8-9.	The DCSF retrieves the avatar media specification data from DC AS using DC AS URL. The DCSF also receives additional instruction on what type of application data channel should be used and what type of rendering mode (e.g. network centric with MF rendering, network centric with DC AS rendering) applies for network-centric rendering initiated in step 5, as well as the Resource URL.

NOTE 2:	The Service operation of step 8-9 is not specified in this Release.

10-11.	The DCSF instructs the MF via IMS AS how to establish the data channel and corresponding avatar media processing specification, via Nimsas_MediaControl_MediaInstruction with the parameters set according to the decisions in step 8-9. If the rendering mode is network centric with MF rendering, IMS AS selects a MF that support network centric with MF rendering based on local configuration or via NRF.

12.	The IMS AS sends the MediaControl Instruction response to the DCSF.

13-18.	Steps 11-23 of clause AC.7.2.3, or steps 7-17 of clause AC.7.2.1 applies, depends on type of application data channel decided in step 5.

19.	The application data channel(s) are established. In the network centric with MF rendering mode, the application data channel terminated at MF. In the network centric with DC AS rendering mode, the MF proxies the media transparently to the DC AS.

20.	The IMS AS initiates the media re-negotiation between UE1, UE2 and MF, which anchors UE1 and UE2's audio/video to MF.

Based on the type of network rendering mode decided in step 4, either option 1 (MF rendering mode, steps 21a-26a) or option 2 (DC AS rendering mode, steps 21b-26b) is performed in steps 22-26.

21.	The MF or DC AS downloads UE1's avatar representation using the Resource URL in the avatar media specification data sent from DCSF in step 10.

22.	 The UE1 sends avatar metadata (e.g. facial expression) through the ADC established in step 19 and/or audio/video through RTP channels established in step 20.

23.	Optionally, the UE2 may send avatar metadata (e.g. facial expression) through the ADC established in step 19 and/or audio/video through RTP channels established in step 20.

24.	MF or DC AS perform avatar animation based on the avatar metadata received from application data channel and/or audio/video. MF should support different media processing according to the avatar type, e.g., animation with no rendering for 2D avatar, animation and rendering for 3D avatar.

25-26.	MF or DC AS sends the animated video stream to UE2 and sends the animated video stream to UE1 if required.

The above procedures also apply for the transition from audio/video communication between two users to avatar communication. In an ongoing audio/video communication, one of UEs decides to switch the communication to avatar mode, it will interact with the IMS Core and peer UE to establish application data channel for the avatar communication. Then two UEs will exchange avatar media; and the original video media between two UEs may be stopped.

# >>>>END OF CHANGES<<<<