3GPP TSG-WG SA2 Meeting #170	S2-2507242

Goteburg, Sweden, Aug 25 – 29, 2025

Source:	China Mobile

Title:	Update on KI#6 Solution 20 Configuration Parameter Provisioning for Sensing Entities

Document for:	Approval

Agenda Item: 	20.2.1

Work Item / Release:	FS_Sensing_ARC/Rel-20

Abstract of the contribution: This paper proposes an update to KI#6 on Configuration Parameter Provisioning of Sensing Entities for the FS_Sensing_ARC TR 23.700-14.

1	Discussion

There are four cases for configuration parameter provisioning towards sensing entities:

- The first case may be handled by the sensing operation server, which could belong to the operators or have an SLA with the operators.

- The second situation may be caused by multiple sensing services.

- The OAM handles the third case within the operator’s domain.

- The SCF may carry out the fourth case based on the sensing result.

2	Proposal

It is proposed to include the following changes in TR 23.700-14 V0.2.0.

* * * * Start of Changes * * * *

## 6.20	Solution #20: Solution for Configuration Parameter Provisioning of Sensing Entities

### 6.20.1	High level principles

Different from sensing service request to trigger sensing service operation, configuration parameter provisioning aims to provision general configuration parameters towards the sensing entities, not for individual sensing service operation.

There may be four cases for configuration parameter provisioning towards sensing entities.

- The first case may be conducted by sensing operation server, which could belong to the operators or have SLA with the operators.

- The second case may be triggered by multiple sensing services.

- The third case is conducted by the OAM within the operator's domain.

- The SCF may perform the fourth case based on the sensing result.

The high level principles of the solution are:

1.	For the untrusted sensing operation server, it sends configuration parameter provisioning request to the NEF. After authentication/authorization was made, the NEF forwards configuration parameter provisioning request to the SCF(s). For the trusted sensing operation server, it sends configuration parameter provisioning request directly to the SCF(s).

2.	The SCF determines the targeted sensing entity(s) and performs the configuration parameter provisioning on sensing entity(s).

NOTE:	The SF (Sensing Function) splits into SCF (Sensing Control Function) and the SPF (Sensing Processing Function.

### 6.20.2	Description

This solution is to resolves Key Issue #6 about the configuration parameter provisioning of sensing entities.

### 6.20.3	Procedures

#### 6.20.3.1	Sensing operation server initiated Configuration Parameter Provisioning procedure

Figure 6.20.3.1: Configuration Parameter Provisioning procedure

1a.	The untrusted sensing operation server sends the configuration parameter provisioning request to the NEF with the following parameters: AF ID, sensing related parameters, external target area information and so on.

1b.	The trusted sensing operation server sends the configuration parameter provisioning request to the SCF with the following parameters: AF ID, sensing related parameters, target area information and so on.

Editor's note:	What are the provisioned configuration parameters is FFS.

NOTE:	Sensing operation server is application function.

2a.	The NEF authenticates and authorizes the untrusted sensing operation server to allow to perform the operation.

3a.	The NEF converts the external target area information to internal target area information.

4a.	The NEF obtains the SCF based on internal target area information by NRF query or local configuration. Then the NEF forwards the request to the SCF.

5.	The SCF determines the targeted sensing entity(s) based on internal target area information.

6.	The SCF performs the sensing configuration parameters provisioning on sensing entity(s).

7a.	The SCF notifies the NEF with configuration parameter provisioning response.

7b.	The SCF notifies the untrusted sensing operation server with configuration parameter provisioning response.

8a.	The NEF forwards configuration parameter provisioning response to the trusted sensing operation server.

#### 6.20.3.2	Multiple sensing services initiate the Configuration Parameter Provisioning procedure

- Sensing entities are performing sensing services.

- The Sensing Operation Server sends the sensing service request to the SCF either directly or indirectly via the NEF.

- The SCF verifies and authorizes the sensing service request and selects the target sensing entity(s) as described in KI#3.

- The SCF can detect the concurrent sensing services and properly deal with the concurrent sensing service(s) by merging the concurrent sensing services or triggering the configuration parameters update (e.g., sensing service Quality of Service).

NOTE:  The SCF may merge concurrent sensing services based on sensing entities' status, ongoing sensing quality of service, target area, sensing service type, requested sensing quality of service, AF ID, or other factors.

- Configuration parameters update between the SCF and sensing entity(s).

- The SCF sends the sensing service response to the sensing operation server.

#### 6.20.3.3	OAM initiates the Configuration Parameter Provisioning procedure

The profile of the UE sensing entity(s) will be managed in UDM/UDR. The profile of the RAN sensing entity(s) will be managed by the SCF.

OAM initiates configuration parameter provisioning on RAN sensing Entity(s)

Figure 6.20.3.3.1: OAM initiates Configuration Parameter Provisioning on RAN Sensing Entity(s)

- RAN Sensing Entity(s) synchronize their sensing service-related configurations either directly with the SCF or indirectly via AMF during the NGAP connection setup procedure.

NOTE:  The SCF may use a subscribe/notification mechanism to the AMF to obtain RAN sensing Entity(s) configurations in indirect connection mode.

- OAM starts the Configuration Parameter Provisioning process on RAN sensing Entity(s) on the SCF.

- The SCF verifies and authorizes the configuration parameter provisioning process, and selects the target RAN sensing entity(s).

- The SCF sends the configuration parameter provisioning request to the RAN sensing entity(s) either directly or indirectly via AMF.

- RAN sensing entity(s) update the configuration parameters.

- The RAN sensing entity(s) sends the configuration parameter provisioning response to the SCF either directly or indirectly through the AMF.

- The SCF sends the configuration parameter provisioning response to the OAM.

- The updated configuration parameters may affect the ongoing sensing service. The SCF will initiate the corresponding procedure resulting from the provisioning of the configuration parameters.

OAM initiates the configuration parameters provisioning on the UE sensing Entity(s)

Figure 6.20.3.3.2: OAM initiates Configuration Parameter Provisioning on UE Sensing Entity(s)

- UE Sensing Entity(s) initiates the registration process with sensing-related configuration parameters to the core network. The UDM/UDR will store and manage the profile of the UE sensing entity(s).

- The SCF uses a subscriber/notification mechanism to the AMF to obtain the UE sensing entity(s) configuration parameters if needed.

- OAM starts the Configuration Parameter Provisioning process on the UE sensing Entity(s) on the UDM/UDR.

- The UDM/UDR verifies and authorizes the configuration parameter provisioning process, selects the target UE sensing entity(s), and determines the corresponding serving AMF.

- The UDM/UDR sends the configuration parameter provisioning request for the UE sensing entity(s) to their corresponding serving AMF.

- Configuration parameter provisioning procedure between the AMF and the UE sensing entity(s).

- The AMF sends the configuration parameter provisioning response to the UDM/UDR.

- The UDM/UDR sends the configuration parameter provisioning response to the OAM.

- If the SCF subscribes to the configuration parameter update event on the UE sensing entity, the AMF shall be triggered to send a notification message informing the SCF of the update.

- The updated configuration parameters may affect the ongoing sensing service. The SCF will initiate the corresponding procedure resulting from the provisioning of the configuration parameters.

#### 6.20.3.4	Sensing Result initiates Configuration Parameter Provisioning procedure

In some scenarios, the configuration parameter provisioning procedure may be triggered by the sensing result. The SCF will initiate the configuration parameter provisioning procedure (e.g., sensing service quality of service) to the sensing entity(s) based on the sensing result.

Figure 6.20.3.3: the SCF initiates Configuration Parameter Provisioning based on sensing result

- The sensing entity(s) send the sensing data to the SPF.

- The SPF calculates the sensing result based on the sensing data

- The SPF will notify the SCF of the sensing result. The SCF decides whether to initiate the configuration parameter provisioning procedure to the sensing entity based on that result.

- The SCF sends configuration parameter provisioning to the sensing entity(s).

NOTE:  Steps 1 to 4 may be executed repeatedly.

### 6.20.4	Impacts on services, entities, and interfaces

This solution impacts the following entities.

NEF:

-	Supports the selection of the SCF based on internal target area information.

UDM/UDR:

-	Supports configuration parameter provisioning procedures

SCF:

-	Supports configuration parameter provisioning procedures

-	Supports the concurrent sensing services.

-  Determine whether to trigger configuration parameter provisioning based on the sensing result or sensing entity status

-  Support the subscribe/notification mechanism to AMF to obtain the sensing entity configuration information

Sensing Entity:

-	Supports sensing configuration parameter provisioning procedures.

* * * * End of Changes * * * *