3GPP TSG-SA WG2#170	S2-2507327

Goteburg, Sweden, 25 - 29 August 2025	( revision of S2-250xxxx)

Source:	Samsung

Title:	TR 23.700-56: FS_NG_RTC_Ph3_ARC-WT#2 New key issue

Document for:	Approval

Agenda Item:	20.7.1

Work Item / Release:	FS_NG_RTC_Ph3_ARC / Rel-20

Abstract of the contribution: This contribution proposed the key issues for TS 23.700-56 for study on system architecture for next generation real time communication services Phase 3 SID.

# 1.	Discussion

According to the approved study item on Study on Stage 2 for system architecture for next generation real time communication services Phase 3 (SP-250855), there follow general WTs below need to be reflected as the key issues in the TR:

(WT#2.1) It has been discussed in Rel-19 whether to enhance NEF services to expose IMS capability for MMTel service with voice/video media besides DC media and has been decided to not support it with the assumption to reuse OMA APIs. However, exposing DC and voice/video media with different sets of APIs brings issues of insufficiency and market fragmentation, and creating some gaps to support some use cases, e.g. customized ringback tone.,

In event exposure framework defined in Rel-19 only DC related events detected by IMS AS are supported. But there are some specific DC events like downloading particular application using existing bootstrap data channel detected by DCSF are missing. Considering the cases requiring non-DC events, e.g. a third part AS wants to subscribe to any UE calling a 1800 number, event exposure framework need to be enhanced to support subscription and notification of non-DC events, otherwise the third party AS will never know subscriber specific and non-subscriber specific information related to the subscriber whose UE is not DC capable.

Therefore, this contribution proposed the related key issues, with additional considerations for these issues.

# 2.	Text proposal

It is proposed to agree the following changes vs. TS 23.700-56:

# >>>>BEGINNING OF CHANGES<<<<

## 5.X	Key Issue #X: Capability exposure enhancement to support IMS MMTEL and DC services

### 5.X.1	Description

The key issue is to address capability exposure enhancement to support IMS DC services supported by DCSF

-	discovery and selection of DCSF

-	identification of list of events supported by DCSF

-	service capability exposure by DCSF using which a NF and third party AF can subscribe and get notification.

Additionally, to support MMTEL services

-	identification of list of IMS non-DC events

-	service capability exposure by NF to support IMS non-DC events

# >>>>END OF CHANGES<<<<