SA WG2 Meeting #170	S2-2508003

# 25 - 29 August 2025, Goteborg, Sweden

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 1479 | rev | 2 | Current version: | 18.10.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections on ADRF related services |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph3 |  |  |  |  |  | Date: |  |  | 2025-3-4 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | According to the call flows described in clause 6.2B.5, when the MTLF provides the ML model file address to the ADRF, the ADRF downloads and stores the ML model using the address. This means that the ADRF does not store the ML model file address provided by the MTLF. However, clause 10.3.2 contradicts this by stating that the ADRF can store the ML model file address received from the MTLF. This is inconsistent with the behaviour of ADRF outlined in 6.2B.5. Furthermore, clause 10.3.3 incorrectly describe the ADRF delete the stored ML model file address, which again conflicts with 6.2B.5. Technically, the ML Model address provided by MTLF may be expired in a short period(e.g. 10 min), it is meaningless for ADRF store the address provided by MTLF. As defined in 6.2A.2 of TS 23.288: ML Model Information, which includes: - the ML Model file address; or - ADRF (Set) ID. Clause 6.2B.6 mentioned the ADRF delete ML Model information based on the request, but ADRF does not have stored ML Model file address or ADRF(set) ID. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Change ‘related ML Model information’ to ‘related information’ in clause 6.2B.6. Remove ‘ML Model file address’ descriptions in clause 10.3.2, 10.3.3. Editorial corrections. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Confusion exists, Misalignment between spec may confuse stage 3 colleague. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2B.6, 10.3.2, 10.3.3, 10.3.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Change * * *

6.2B.6	ML Model removal from ADRF

The procedure depicted in figure 6.2B.6-1 is used by NWDAF containing MTLF to delete ML Model from an ADRF.

Figure 6.2B.6-1: ML Model removal from ADRF

1.	NWDAF containing MTLF requests to delete the ML Model(s) previously stored in the ADRF using Nadrf_MLModelManagement_Delete request service operation.

2.	The ADRF deletes both the stored ML Model(s) and ML Model related information.

3.	The ADRF indicates the result (i.e. ML Model deleted, ML Model not found, ML Model found but not deleted) using Nadrf_MLModelManagement_Delete response service operation.

* * * Next Change * * *

10.3.2	Nadrf_MLModelManagement_StorageRequest service operation

Service operation name: Nadrf_MLModelManagement_StorageRequest

Description: The consumer NF uses this service operation to request the ADRF to store or update ML Model(s). ML Model(s) or ML Model address(es) stored in NWDAF containing MTLF are provided to the ADRF in the request message.

Inputs, Required: NF instance ID of the NWDAF containing MTLF and set of:

-	(Updated) ML Model(s); or

-	the tuple (unique ML Model identifier and address of (updated) ML Model file and (updated) Storage size required for each of the ML Models).

The ADRF downloads ML Model file using the ML Model addresses provided by the NWDAF containing MTLF. How the ADRF locally stores the ML Model is left for implementation.

NOTE 1:	The ADRF does not store the ML Model file address(es) received from NWDAF containing MTLF.

Inputs, Optional: Allowed NF instance list for the ML Model identifier, Storage Transaction Identifier.

If Storage Transaction Identifier is included in the input, then ADRF updates the ML Model(s) corresponding to the Storage Transaction Identifier with the updated ML model(s) or updated ML Model file address(es) provided by consumer NF. A new Storage Transaction Identifier may be generated and sent to consumer NF by ADRF after the ML Model update process.

Outputs Required:

-	ML Model storage or ML Model Update Result Indication;

-	[Conditional] one or more tuples of unique ML Model identifier and (updated) ML Model address of Model file stored in ADRF.

NOTE 2:	The definition of ML file address such as e.g. name, location and access method is up to stage 3.

Outputs, Optional: Storage Transaction Identifier.

* * * Next Change * * *

10.3.3	Nadrf_MLModelManagement_Delete service operation

Service operation name: Nadrf_MLModelManagement_Delete

Description: This service operation instructs the ADRF to delete stored ML Model file(s) or ML Model address(es).

Inputs, Required:

-	Storage Transaction Identifier; or

-	one or more unique ML Model identifier(s).

When a Storage Transaction Identifier is given, ADRF shall delete all the ML models that stored in the corresponding storage transaction.

Inputs, Optional: None.

Outputs, Required: One of the following:

-	Operation execution result indication (i.e. ML Model deleted, ML Model not found, ML Model found but not deleted), if for all of the involved ML Model(s) have the same execution result.

-	One or more tuples (each includes the unique ML Model identifier and corresponding operation execution result indication).

Outputs, Optional: None.

* * * Next Change * * *

10.3.4	Nadrf_MLModelManagement_RetrievalRequest service operation

Service operation name: Nadrf_MLModelManagement_RetrievalRequest

Description: The consumer NF uses this service operation to request the ADRF to get the ML Model(s) stored in ADRF.

Inputs, Required:

-	Storage Transaction Identifier; or

-	one or more tuples of unique ML Model identifier(s).

Inputs, Optional: None.

Outputs Required: Result Indication.

-	[Conditional] one or more tuples of unique ML Model identifiers and address of ML Model file stored in ADRF.

NOTE:	The definition of address of ML Model file such as e.g. name, location and access method is up to stage 3.

Outputs, Optional: None.

* * * End of Changes * * *