SA WG2 Meeting #170	S2-2508110

25 – 29 Aug, 2025, Goteborg, SE

Source:	NTT DOCOMO (pen holder)

Title:	[General aspects] Summary of Architectural Assumptions

Document for:	Approval

Agenda Item:	20.6.0

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: This contribution summarizes all contributions on Architectural Assumptions

**** First Change (all text new) ****

## 3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

Definition format (Normal)

<defined term>: <definition>.

Core Network (CN): In this study, it refers to the Core Network for the 6G System.

6G RAN: In this study, it refers to the radio access network for 6G radio access technology (6GR) being studied by 3GPP RAN WGs [ref: RP-251881].

6G System (6GS): 3GPP System consisting of 6G RAN, non-3GPP access, Core Network and UE.

**** Next Change (all text new) ****

## 4.1	Architectural Assumptions

Editor's Note: This clause documents the common architecture assumptions identified for the study.

The following is assumed for 6GS architecture:

1.	The framework of SBA specified for 5GC is assumed as a starting point for discussion.

[OPTION1]

2.	For 3GPP access, the scope of the study is single technology framework based on a stand-alone architecture.

[OPTION2]

2.  The study assumes a standalone architecture [ref: RP-251881].

[ENDOFOPTIONS]

NOTE: approaches beyond stand-alone architecture will be considered in coordination with TSG RAN [ref: RP-251881].

3.	The 6G system is assumed to maintain the existing RAN and CN functionality split as supported in 5GS, and avoid duplication of functionality in 6G RAN and CN.

4.	Support of multi-media services such as voice, video (essential and regulatory services) will be using IMS.

**** End of Changes ****