SA WG2 Meeting #171	S2-2508332

13 – 17 Oct, 2025, Wuhan, China                                                                    was S2-25xxxxx

Source:	China Unicom, CATT, China Telecom, vivo

Title:	[WT#1.2, Network Sharing] WT(s) and KI(s) on 6G network sharing

Document for:	Agreement

Agenda Item:	20.6.1.2

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: This pCR gives the WT(s) and KI(s) on 6G network sharing based on the SA2#170 progress.

- Justifications

In the SA2#170 meeting, the initial WT(s) on 6G Network Sharing has been approved and captured in TR 23.801-01:

Study on how to support network sharing in 6G, including the following aspects:

- How to support the following network sharing architectures in 6G: Multi-Operator Core Network in 6G and Indirect Network Sharing in 6G.

Editor’s Note: It is FFS whether other aspects of network sharing in 6G can be added in this clause.

There is a remaining EN to capture the other aspects (e.g. extension of the scope and type of network sharing resources, the event triggered network sharing, non-3GPP aspect, etc) which has not reached a consensus during the SA2#170.

This pCR focuses on the aspects of the extension of the scope and type of network sharing resources, especially considering the network sharing on radio access network with sensing capability through analysis of operator requirements and potential architectural impacts. And this pCR gives the WT(s) and KI(s) on 6G network sharing based on the SA2#170 progress.

- 1.1 Network sharing on radio access network with sensing capability

- 1.1.1 Operator requirements

Given the multitude of use cases for services, some operators, based on their business model, may deploy one or more network capabilities (e.g., computing capability, or sensing capability) in some areas to serve their services in 6G, but not in all areas. However, through multi-operator sharing collaboration, the service coverage of specific network capability can be expanded, for instance, this could be used to provide positioning and velocity sensing services for drones, smart transportation and refrigerated transport. This particularly occurs in rural areas, in long-distance logistics and in traffic management, for example, in order to reduce the cost of deployments.

Figure 1 Network Sharing on radio access network with sensing capability

For point-to-point delivery by logistics UAVs from area A to area B, the sensing service can provide logistics customers with real-time positioning and velocity data assistance through operator sensing capability based on the operator’s policy.

As shown in Figure 1, Mobile Network Operator #A (OP#A) can provide sensing service by deploying radio access network with sensing capability (e.g., RAN node as sensing transmitter/receiver) in area A. The coverage of sensing service in OP #A is area A.

Mobile Network Operator #B (OP#B) can provide sensing service by deploying radio access network with sensing capability in area B. The coverage of sensing service in OP#B is area B.

To expand the sensing service coverage for OP#A and OP#B, OP#A and #B have the sensing network sharing agreement. This will extend their sensing service coverage to area A plus area B.

For the specific sensing service (e.g. UAV tracking), OP#A provides sensing service for logistics UAV tracking for collection of sensing data related to UAV using the radio access network with sensing capability of OP#A (e.g., UAV’ position, velocity and environment related data), when the UAV enters the sensing area A. When the UAV enters sensing area B, based on sensing network sharing agreement between operators, the OP#A can use the shared radio access network with sensing capability to collect the sensing data which satisfies the service requirement of OP#A’s customer (e.g. logistics UAV tracking) as Figure 1, while these collected data will not be used by other operators. Logistics customers do not need to be aware of which radio access network with sensing capability are used to provide the sensing service.

NOTE 1:  In the above case, the shared radio access network with sensing capability can be used as OP#A’s own radio access network with sensing capability.

NOTE 2:  The above case has been captured in SA1 TR 22.870.

- 1.1.2 Potential architectural impact

In order to satisfy the above requirements, the network sharing on radio access network with sensing capability focuses on gNB based sensing sharing (i.e. the shared Base Stations are as the sensing transmitter/receiver).

Based on the progress of the sensing topic (WT#4 Sensing), the network sharing related aspects can study which network sharing architectures are applicable for the network sharing on radio access network with sensing capability, e.g., whether the 6G MOCN and/or 6G INS can be applicable (directly or with few enhancements) or potential new network sharing architecture is suitable.

Summary: Based on the progress of the sensing topic (WT#4 Sensing), the 6G network sharing related aspects should study which network sharing architectures are applicable for the network sharing on radio access network with sensing capability.

- Proposals

Based on the progress of SA2#170, this pCR solves the EN to give the additional WT. On top of the WTs, the corresponding KIs are proposed as well.

The following should be captured in TR 23.801-01.

**** First Change ****

A.1.2.1	Network Sharing in the 6G system

Study on how to support network sharing in 6G, including the following aspects:

- How to support the following network sharing architectures in 6G: Multi-Operator Core Network in 6G and Indirect Network Sharing in 6G.

- How to support the network sharing on radio access network with sensing capability in 6G:

  - Study whether and how the MOCN in 6G and/or INS in 6G can be applicable (directly or with few enhancements) or any new network sharing architecture is suitable for the network sharing on radio access network with sensing capability.

NOTE 1:  For the network sharing on radio access network with sensing capability, it needs to consider the progress of WT#4 Sensing.

**** Second Change (All new text)****

- X	Key Issue #X: Network Sharing in the 6G system

Study on how to support network sharing in 6G, including the following aspects:

- 1) How to support the following network sharing architectures in 6G: Multi-Operator Core Network in 6G and Indirect Network Sharing in 6G.

- 2) How to support the network sharing on radio access network with sensing capability in 6G:

- Study whether and how the MOCN in 6G and/or INS in 6G can be applicable (directly or with few 	enhancements) or any new network sharing architecture is suitable for the network sharing on radio access 	network with sensing capability.

NOTE 1:  For the network sharing on radio access network with sensing capability, it needs to consider the progress of WT#4 Sensing.

**** End of Changes ****