3GPP TSG-SA WG2 Meeting #171                                                                   S2-2508354

Wuhan, China, October 13 – 17, 2025

Source:	vivo

Title:	[WT#2, Migration and Interworking] Migration and Interworking

Document for:	Approval

Agenda Item:	20.6.2

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: this contribution introduces the work task and key issues for WT#2 (migration and interworking) based on the baseline tdoc S2-2508129.

1. Justifications

During SA2#170 meeting, the baseline paper S2-2508129 contains following contents:

WT#2: Study migration and interworking, including

1	How to support migration to 6GS.

2	How to support interworking with 5GS (including when using 5G NTN/satellite access).

3	Whether and how to support interworking with EPS (including when using 4G NTN/satellite access).

NOTE 1:	Interworking with 2G/3G are not considered in this study. Interworking between 6GS and 5GS is assumed to work even if the UE has previously registered in 2G, 3G or 4G.

NOTE 2: The detailed migration study scope will be coordinated and aligned with RAN.

NOTE 3: The Work Task should consider roaming aspects.

Based on the baseline paper, we have following proposals:

- Interworking architecture:

In 5GS, architecture for interworking between 5GS and EPC/E-UTRAN is specified as figure 1:

Figure 1: Non-roaming architecture for interworking between 5GS and EPC/E-UTRAN

The key features of interworking between 5GS and EPC/E-UTRAN as blow:

- A N26 interface between 5G AMF and MME is specified to transfer UE context during interworking.

- The combined NF, i.e. SMF+PGW-C and UPF+PGW-U, dedicated for interworking between 5GS and EPC are specified to guarantee IP continuity during interworking.

For interworking between 6GS and 5GS, the same key features may also need to be considered:

Whether and how to specified a N26 like interface between 5GC and 6G CN

To support interworking between 6G and 5G, the UE contexts may also need to be transferred between 6G CN(e.g. 6G AMF) and 5GC(e.g. 5G AMF), so the interface is needed.

But whether the interface is a N26 like interface or service-based interface needs to be studied considering both 5GS and 6GS support SBA.

Proposal 1: Study whether the interface between 6G CN and 5GC is needed, and if needed define interface is a 5G N26 like interface or service-based interface.

Whether and how to introduce combined NF

For User Plane (UP), to support IP continuity from 6GS to 5GS for UE, and considering the UE may need to further transfer from 5GS to EPS, the 6G UPF may need to combined with 5G UPF+PGW-U i.e., the combined 6G UPF+5G UPF+PGW-U needs to be introduced.

For Control Plane(CP), two options exist：

- Option 1: introduce a combined NF: 6G SMF+5G SMF+PGW-C

- Option 2: separate 6G SMF and 5G SMF+PGW-C,

both 6G SMF and 5G SMF+PGW-C can control 6G UPF+5G UPF+PGW-U

Option 1 may lead to the combined CP NF complex and not easy to support new 6G features.

Option 2 may need to specify the interface between 6G SMF and 5G SMF+PGW-C, but much easier to support new 6G features.

Proposal 2: Whether the 6G SMF needs also to be combined into 5G SMF+PGW-C needs to be studied.

- Interworking scenario:

When a UE has a service that requires IP continuity in 6GS (e.g., native IMS voice over PS session in 6GS) and the UE hands over from 6GS to 5GS due to a coverage problem, the network should guarantee that the UE can further hand over from 5GS to EPS if the UE loses 5G coverage.

Proposal 3: the scenario of interworking from 6GS to 5GS should be studied, and if the UE has been transferred from 6GS to 5GS, the UE can be further transferred to EPS.

- The relationship between WT 1.4 and WT 2

In 5GS, the EPS fallback is used to support IMS voice when the PLMN does not support native IMS voice over PS session in NR(VoNR). The “fallback” is based on inter RAT interworking procedure.

For 6GS, the interworking may also be used to support IMS voice if the PLMN does not support native IMS voice over PS session in 6GS(Vo6G). And considering some operators may not support VoNR, the interworking from 6GS to EPS may also be needed.

Proposal 4: the interworking may be used to support “fallback” procedure for IMS voice service.

2. Proposal:

It is proposed to add following changes to TR 23.801-01 V0.1.0.

**** First Change (based on paper S2-2508129) ****

Annex A.X. WT#2 Migration and Interworking Scope

Editor's Note: Describe the technical scope of the proposed Work Task. If applicable, suggest logical subdivision of this WT into smaller sub-WT. This clause is part of the TR Annex.

WT#2: Study migration and interworking, including

1	How to support migration to 6GS.

2	How to support interworking with 5GS (including when using 5G NTN/satellite access).

- Study whether the interface between 6G CN and 5GC is needed, and if needed define interface is a 5G N26 like interface or service-based interface.

- Study whether combined NFs are needed, such as whether to combine 6G SMF with 5G SMF+PGW-C, whether to combine 6G UPF with 5G UPF+PGW-U.

-    Study when the UE transfers from 6GS to 5GS, how the UE can be further transferred to EPS.

3	Whether and how to support interworking with EPS (including when using 4G NTN/satellite access).

NOTE 1:	Interworking with 2G/3G are not considered in this study. Interworking between 6GS and 5GS is assumed to work even if the UE has previously registered in 2G, 3G or 4G.

NOTE 2: The detailed migration study scope will be coordinated and aligned with RAN.

NOTE 3: The Work Task should consider roaming aspects.

NOTE X: The interworking from 6GS to 5GS or from 6GS to EPS may be used to perform “fallback” procedure for IMS voice service when native IMS voice over PS session in 6GS is not support for an operator.

**** Next Change****

5.X. Key Issue #X: Migration and Interworking

Study migration and interworking, including

1	How to support migration to 6GS.

2	How to support interworking with 5GS (including when using 5G NTN/satellite access).

- Study whether the interface between 6G CN and 5GC is needed, and if needed define interface is a 5G N26 like interface or service-based interface.

- Study whether combined NFs are needed, such as whether to combine 6G SMF with 5G SMF+PGW-C, whether to combine 6G UPF with 5G UPF+PGW-U.

-    Study when the UE transfers from 6GS to 5GS, whether and how the UE can further transfer to EPS.

3	Whether and how to support interworking with EPS (including when using 4G NTN/satellite access).

NOTE 1:	Interworking with 2G/3G are not considered in this study. Interworking between 6GS and 5GS is assumed to work even if the UE has previously registered in 2G, 3G or 4G.

NOTE 2: The detailed migration study scope will be coordinated and aligned with RAN.

NOTE 3: The Work Task should consider roaming aspects.

NOTE X: The interworking from 6GS to 5GS or from 6GS to EPS may be used to perform “fallback” procedure for IMS voice service when native IMS voice over PS session in 6GS is not support for an operator.

**** End of Changes ****