3GPP TSG-SA2 Meeting #171	S2-2508362

Wuhan, CN, Oct 13 – 17, 2025                                          (was)

Source:	Nokia

Title:	Proposed agreement of principles for Key Issues 1

Document for:	Approval

Agenda Item:	20.2.1

Work Item / Release:	FS_Sensing_ARC / Rel-20

Abstract: This document proposes to agree principles for KI#1

# 1.	Introduction

This document discusses aspects related to several KIs and proposes principles for conclusions by considering the scope, architectural assumptions and the solutions captured in the TR.

## 1.1 Discussion for KI#1:

The Sensing Function (SF) implements receiving of the sensing request as configurations from the AF (Sensing Client - SC) with or without NEF and providing the corresponding translated configurations to the Sensing Entity (SE); SF is responsible for collecting the sensing data from the SE and provide the generated sensing result to the SC for the associated request.

Based on the agreed scope and architectural assumptions, the considered use case for Release 20 5GA is for detection and tracking of aerial objects (e.g., drone) which could be realized on re-using of the existing eco system (e.g., BS) and its interface with the Core Network. There is no motivation or a need for SA2 to bring in major architectural changes and requirements on gNB in this stage of 5GA (where a new generation work has started) for receiving the configurations from SF and communicating the sensing data to the SF. For the use case described, existing mechanisms which are proven already in features likes positioning are proposed with limited changes. Bringing major architectural changes would bring complexities and challenges to commercialize 5GA Sensing, which involves bringing additional requirement on NG-RAN to interface to the CN, increased interoperability testing requirements, additional resources.

Proposal for KI#1: It is proposed to capture the following principles as conclusions for KI#1.

- Introduce new NF, Sensing Function (SF) in the overall architecture.

- SF interfaces with NEF, AF and AMF.

- AMF acts a transport for sensing activities i.e., for relaying the sensing configurations to the SE from the SF and for relaying the sensing data to the SF from the SE. AMF does not play any active role apart from exposing services to transport configurations and sensing data between gNB and SF.

- SF implements logical functions, Sensing Control function (SeCF) and Sensing Processing function (SePF) with interfaces not being specified nor standardized.

# 2. Text Proposal

It is proposed to capture the following changes in TR 23.700-14.

# * * * * First change (all new text) * * * *

### 7.1.1	Agreed Principles for KI#1

Editor's note:	This clause will include the principles that are agreed as work progresses for the specific KI#Y. This may be populated directly or e.g. also when a topic in clause 7.2.Y gets resolved and a principle is agreed.

The following principles are agreed for KI#1:

- Introduce new NF, Sensing Function (SF) in the overall architecture.

- SF interfaces with NEF, AF and AMF.

- AMF acts a transport for sensing activities i.e., for relaying the sensing configurations to the SE from the SF and for relaying the sensing data to the SF from the SE. AMF does not play any active role apart from exposing services to transport configurations and sensing data between gNB and SF.

- SF implements logical functions, Sensing Control function (SeCF) and Sensing Processing function (SePF) with interfaces not being specified nor standardized.

# * * * * End of change * * * *