SA WG2 Meeting #171	S2-2508369

# 13 - 17 October, 2025, Wuhan, China								        (revision of S2-2506516)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 1500 | rev | 1 | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification made on VFL description in R19 TS 23.288 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2025-9-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Clarification are made on VFL description in R19 TS 23.288, the main changes are as follows: To clarify that when NWDAF is acting as VFL server, the VFL Clients can be NWDAF or AF. VFL Server supports to monitor the ML Model metric (e.g. ML Model Accuracy) of the VFL training and inference. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarification are made on VFL description in R19 TS 23.288, the main changes are as follows: When NWDAF is acting as VFL server, the VFL Clients can be NWDAF or AF. VFL Server supports to monitor the ML Model metric (e.g. ML Model Accuracy) of the VFL training and inference. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The VFL description is unclear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Change * * *

5.4	Vertical Federated Learning (VFL)

Vertical Federated learning is a machine learning technique working without exchanging/sharing of local data set, while maintaining some level of coordination amongst VFL participants, i.e., VFL Server and VFL Client, when training and inference are performed on local ML Models, wherein the local data set in different VFL Participant for local model training have different feature spaces for the same samples (e.g. UE IDs). Vertical Federated Learning may involve multiple NWDAFs and AFs.

For Vertical Federated Learning, there may be one NWDAF or one AF acting as a VFL server and one or multiple NWDAF(s) and/or one or multiple AF(s) acting as VFL Client(s). Vertical Federated Learning is available among NWDAFs or between NWDAF(s) and AF(s) within a single PLMN or between an AF and NWDAF(s) in a single PLMN. When AF is acting as VFL Server, NWDAF(s) is VFL Client(s). When NWDAF is acting as VFL server, the VFL Clients can be NWDAF or AF.

The main functionalities of VFL server and VFL client include:

VFL server:

-	An NWDAF or trusted AF acting as VFL server discovers and selects VFL client(s) (NWDAF(s) and/or AF(s)) to participate in a VFL procedure.

NOTE 1:	When an untrusted AF is acting as VFL server, NEF discovers and selects candidate VFL client NWDAFs, then the AF determines final set of VFL clients.

-	It requests VFL clients to do local ML model training for an Analytic ID, it assigns VFL correlation ID, and it requests to report intermediate training results.

-	It optionally locally trains ML Model with the available local data set.

-	It combines intermediate training results from VFL client(s) and VFL server and computes intermediate model training information (e.g. gradient information, loss information) using label for updating its own local ML Model and sends the intermediate model training information to VFL clients for updating the ML Models of VFL clients that involved in the VFL training process. VFL server may send and receive separate message for each VFL client.

NOTE 2:	If VFL server does not have a local ML Model, it only combines intermediate training results from VFL client(s) and computes intermediate model training information which will be sent out the VFL Clients.

NOTE 3:	NEF forwards the message from/to NWDAF to/from untrusted AF.

-	It determines to terminate the VFL training process.

-	It stores VFL correlation ID and locally trained ML Model after VFL training process.

-	It initiates the VFL inference process using VFL correlation ID.

-	It combines local inference result from VFL clients and generates the final VFL inference result.

-	It may send the final VFL inference result to the consumer.

-	It supports to monitor the ML Model metric (e.g. ML Model accuracy)of the VFL training and inference.

VFL client:

-	It locally trains ML Model with the available local data set, which includes the data that may not be allowed to be shared with other VFL clients or VFL server due to e.g. data privacy, data security, data access rights.

-	It computes the intermediate training results for their local ML Models involved in the VFL training and provide reports with the intermediate training results to the AF or NWDAF acting as VFL server.

-	It updates their local ML model based on the intermediate model training information received by the VFL server.

-	It stores VFL correlation ID and locally trained ML model after VFL training process.

-	It performs inference based on the local model and local data and provides local inference results to VFL server.

Vertical Federated Learning includes the following procedures:

-	Registration of the NF profile including a list of VFL related information to NRF. Registration of the NWDAF profile to NRF is described in clause 5.2. Registration of the AF profile to NRF is described in clause 5.5. The procedure for registration and discovery of VFL server and VFL client is described in clause 6.2H.2.1.

-	Preparation for VFL including sample alignment to ensure that all the VFL participants have common samples when training ML models as described in clause 6.2H.2.2.

-	Training for VFL as described in clause 6.2H.2.3.

-	Inference for VFL as described in clause 6.2H.2.4.

In this Release, the same NF associated with a VFL Server or VFL Client capability during the VFL training for a VFL correlation ID is also the same NF during the VFL inference.

* * * End of Changes * * *