3GPP TSG-SA2 Meeting #171	S2-2508376

# Wuhan, China, October 13 - October 17

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 6428 | rev | - | Current version: | 19.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Enbaling and Disabling LP-WUS per UE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | InterDigital Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_LPWUS-Core |  |  |  |  |  | Date: |  |  | 2025-10-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In S2-2508182/R2-2506261, RAN2 indicaes the RAN2 made the following 4 agreements: 1. RAN2 assumes NAS signalling is introduced to support enabling/disabling LP-WUS per UE. 2. RAN2 assumes that NAS signalling needs to be extended to enable/disable LP-WUS per UE in IDLE and INACTIVE. Detail signalling is up to SA2 and CT1. 3. RAN2 assumes that CN needs to inform gNB that LP-WUS is enabled/disabled per UE in IDLE and INACTIVE. Details signalling is up to SA2, CT1 and RAN3. 4. RAN2 assumes that without such NAS signalling, UE is allowed to use LP-WUS in IDLE and INACTIVE. TS 24.501 already includes the following statements about the registration procedure: The UE shall use the LP-WUSPS assistance information only if the UE received the Negotiated LP-WUSPS assistance information IE during the last registration procedure for either initial registration or mobility registration update. If the UE did not receive the Negotiated LP-WUSPS assistance information IE during the last registration procedure for either initial registration or mobility registration update, the UE shall delete any existing LP-WUSPS assistance information received from the network. TS 24.501 already includes the following statement about the UE configuration update procedure: If the UE supports the LP-WUSPS assistance information and the AMF needs to remove the LP-WUSPS assistance information from the UE, the AMF shall include the Updated LP-WUSPS assistance information IE with the length of LP-WUSPS assistance information contents field set to zero in the CONFIGURATION UPDATE COMMAND message. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updated the stage-2 description to align with the stage-3 description and the RAN2 agreements. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misalignment with stage-3 and it would not be clear that the the AMF can disable LP-WUSP for a UE in the UE and RAN. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.12a.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

**************************** First Change ****************************

#### 5.4.12a.2	Core Network Assistance for LP-WUSPS

To support the LP-WUSPS, the LP-WUS Paging Subgrouping Support Indication and the LP-WUSPS Assistance Information are used by the AMF and NG-RAN to help determine whether LP-WUSPS applies to the UE and which paging subgroup is used when paging the UE (see TS 38.300 [27]).

In the Registration Request message, the UE includes the LP-WUS Paging Subgrouping Support Indication if the UE supports AMF LP-WUSPS Assistance Information for LP-WUS Paging Subgrouping. If the UE includes the LP-WUS Paging Subgrouping Support Indication, the UE may also include the paging probability information to assist the AMF. An AMF supporting LP-WUSPS stores the UE provided LP-WUS Paging Subgrouping Support Indication and the AMF determined AMF LP-WUSPS Assistance Information in the UE context. The AMF may use local configuration, the UE's paging probability information if provided, information provided by the RAN (e.g. any of the Information On Recommended Cells And RAN nodes For Paging), and/or previous statistical information for the UE to determine AMF LP-WUSPS Assistance Information. The AMF LP-WUSPS Assistance Information includes the Paging Subgroup ID.

NOTE 1:	To minimise MT voice call setup latency, the AMF could allocate Paging Subgroup IDs for LP-WUS taking into account whether or not the UE is likely to receive IMS voice over PS session calls.

NOTE 2:	To avoid MT traffic for more mobile UEs causing more stationary UEs to be woken up, the AMF could allocate Paging Subgroup IDs for LP-WUS taking into account the UE's mobility pattern.

If the AMF has determined AMF LP-WUSPS Assistance Information for the UE and wants to enable LP-WUSPS for the UE, the AMF stores it in the UE context in AMF and provides it to the UE in every Registration Accept message.

If the AMF has determined AMF LP-WUSPS Assistance Information and wants to enable LP-WUSPS for the UE, the AMF shall provide it to NG RAN when paging the UE. In addition, in order to support LP-WUSPS for UEs in RRC_INACTIVE mode, the AMF shall provide the AMF LP-WUSPS Assistance Information to NG-RAN as part of the RRC Inactive Assistance Information.

When the NG-RAN is using AMF allocated subgroups for LP-WUSPS, both the UE and NG-RAN use the AMF LP-WUSPS Assistance Information to determine the paging subgroup to apply as defined in TS 38.300 [27].

The AMF may use the UE Configuration Update procedure (as described in clause 4.2.4 of TS 23.502 [3]) and N2 UE Context Modification procedure (as described in clause 8.3.4 of TS 38.413 [34]) to update the AMF LP-WUSPS Assistance Information in the UE and NG-RAN.

The AMF may use the Registration Accept message (as described in clause 4.2.2 of TS 23.502 [3]) or the UE Configuration Update procedure (as described in clause 4.2.4 of TS 23.502 [3]) and N2 UE Context Modification procedure (as described in clause 8.3.4 of TS 38.413 [34]) to disable LP-WUSPS for the UE.

NOTE 3:	How the AMF decides whether the Low Power Wake UP Signal should be disabled or enabled for a UE is an implementation decision (e.g. based on the UE’s Allowed NSSAI).