3GPP SA WG2#171	                                                   S2-2508397

Wuhan, China, 13-17 October, 2025

Source:	China Telecom

Title:	23.700-30: New Solution of AIoT Devices registration procedure for KI#2

Document for:	Approval

Agenda Item:	20.5.1

Work Item / Release:	FS_AmbientIoT_Ph2_ARC / Rel-20

Abstract: This is a new solution proposal related to KI#2 in the R20 Ambient IoT TR 23.700-30 V0.1.0.

1. Introduction/Discussion

Based on TR 23.700-30 V0.1.0, the following KI#2 shall be studied:

Key Issue #2: Support of DO-A Capable AIoT Devices

This key issue will study the system architecture to support DO-A capable Ambient IoT Devices in Topology 1 and Topology 2.

The following aspects will be studied:

-	How the AIoT Device informs the network of its presence autonomously (e.g., an AIoT Device initiated registration-like procedure) and what are the triggers for the DO-A capable device to inform the network of its presence.

-	Whether and how to consider power consumption of DO-A Capable AIoT Devices.

-	How an AIoT Device sends data to the AIOTF autonomously.

-	Support for routing the data received by AIOTF from an AIoT Device to an AF.

-	Whether and how to enhance the Inventory and Command procedures defined in TS 23.369 [3] to support DO-A capable AIoT Devices.

-	Naiotf, Namf and Nnef interface enhancements to support DO-A capable AIoT Device .

NOTE 1:	The conclusions from Key Issue #1 are the basis for supporting DO-A capable AIoT Devices in topology 2 in this key issue.

NOTE 2:	Coordination with RAN WGs is required.

This pCR provides a new solution for corresponding to this KI#2 bullet 1 : How the AIoT Device informs the network of its presence autonomously (e.g., an AIoT Device initiated registration-like procedure) and what are the triggers for the DO-A capable device to inform the network of its presence.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-30.

# * * * * First change (all new texts)* * * *

6 Solutions

6.X	AIoT Devices Registration Procedure for KI#2

6.X.0 High-level solution Principles

This solution is for Key Issue #2, corresponding to bullet 1: How the AIoT Device informs the network of its presence autonomously (e.g., an AIoT Device initiated registration-like procedure) and what are the triggers for the DO-A capable device to inform the network of its presence.It includes AIoT device registration-like procedure, and when will peform AIoT device registration-like procedure.

6.X.1	Description

The main points of this solution are as following:

- The AIoT device performs AIoT device registration-like procedure by it initiated or the network/AF initiated or RAN Reader/UE Reader initiated.

- The AIoT device sends Registration-like Request to AIoTF via the RAN Reader or UE Reader. The Registration-like Request includes AIoT device identification, security data, and may includes AIoT device location if possible, etc.

- The AIoT device shall initiate the registration-like procedure by sending a REGISTRATION REQUEST message to the AIoTF,

  - When the AIoT device enters into new aeras, such as new reader,etc；or

  - When the AIoT device’s timer X expired, if had；or

  - When the stored policy from netwrok about initiate the registration procedure is applied; or

  - When the upper layer or associated device(e.g. UE reader/RAN reader) initial the AIoT device to send registration procedure; or

  - When the AIoT device receives AIoT service(e.g. Inventory Service, Command read/write Service, new Service, etc) which idicate to initiate the registration procedure; or

  - When the AioT device receives system informaton or R2D message which idicate to initiate the registration procedure.

- The AIoT Device may have pre-configurated network resource/assistance information(e.g. frequence, Qos, Resource Expiration Date,area, etc) for autonomously originating registration-like procedure in device or or UISM if had; or support to receive the network resource information/assistance information (e.g. frequence, Qos, Resource Expiration Date,etc) from the network, using by AIoT write commands, or network system information, etc.

6.X.2	Procedures

The following figure presents a AIoT device registration-like procedure for Topology 1 and Topology 2.

Figure 6.x.2-1: AIoT device registration-like procedure

0-a. DO-A Capable AIoT Devices may be Pre-configurated network resource information for AIoT device registration-like procedure in device or or UISM if had. Or the AIoT device may receive the network resource information/assistance information (e.g. frequence, Qos, Resource Expiration Date,etc) from the network, using by AIoT write commands, or network system information, etc.

0-b. DO-A Capable AIoT Devices may be Pre-configurated policy information for AIoT device registration-like procedure in device or or UISM if had. Or the AIoT Devices may receive and store policy information about initiate the registration procedure from netwrok via AIoT service(e.g. AIoT write command).

NOTE 1:	How information is configured into AIoT device is out of scope of 3GPP.

- The AIoT device shall initiate the registration-like procedure,

  - When the AIoT device enters into new aeras, such as new reader,etc；or

  - When the AIoT device’s timer X expired, if had；or

  - When the stored policy from netwrok about initiate the registration procedure is applied; or

  - When the upper layer or associated device(e.g. UE reader/RAN reader) initial the AIoT device to send registration procedure; or

  - When the AIoT device receives AIoT service(e.g. Inventory Service, Command read/write Service, new Service, etc) which idicate to initiate the registration procedure; or

  - When the AIoT device receives system informaton or R2D message which idicate to initiate the registration procedure.

- The AIoT device sends Registration-like Request to AIoTF via the RAN Reader or UE Reader. The Registration-like Request includes AIoT device identification, security data, and may includes AIoT device location if possible, etc.

- AIoTF receives the Registration-like Request from AIoT device, and will check the authorization of AIoT devices from AIoT device profile data in ADM, and registrate for AIoT device, such as update the registration state or update the AIoT location.

- The AIoTF may send Registration-like result(e.g. Registration success or fail,etc) to AIoT device..

- The AIoTF may send Registration-like result(e.g. Registration success or fail, AIoT device location,etc) to AF which the AIoT device corresponding to.

6.X.3	Impacts on services, entities and interfaces

NEF:

- Supports AF to receive Registration-like result (e.g. Registration success or fail, AIoT device location,etc);

AIoTF:

- Supports to receives the Registration-like Request from AIoT device;

- Supports to check the authorization of AIoT devices and registrate for AIoT device, such as update the registration state or update the AIoT location;

- Supports to send policy about initiate the registration procedure to AIoT device;

- Support to initial AIoT device Registration-like procedure if needed;

RAN Reader or UE Reader:

- Supports to receives the Registration-like Request AIoT device from and route for AIoTF;

- Support to initial AIoT device Registration-like procedure based on policy about initiate the registration procedure to AIoT device, if needed；

AIoT Device：

- Pre-configured or stores network resource/assistance information(e.g. frequence, Resource Expiration Date, etc) AIoT device registration-like procedure;

- Performing AIoT device registration-like procedure.

# * * * * End of changes * * * *