SA WG2 Meeting #171	S2-2508436

13 - 17 October, 2025, Wuhan, China	(revision of S2-25xxxxx)

Source:	ZTE

Title:	KI#1, New Sol: Service Operation Aggregation for Topology 1 and Topology 2

Document for:	Approval

Agenda Item:	20.5.1

Work Item / Release:	FS_AmbientIoT_Ph2_ARC / Rel-20

Abstract of the contribution: The paper adds new solution for supporting service operation aggregation for Topology 1 and Topology 2.

1. Discussion

In TR 23.700-30, the following key issue is to be studied:

5.1	Key Issue #1: support AIoT services under the RRC-based option for UE Reader connectivity

NOTE 1:	Only CM-CONNECTED with RRC_CONNECTED UE Reader is in the scope.

Only the following aspects will be included:

-	How to perform and revoke authorization of UE Readers.

NOTE 2:	Rel-19 interim conclusion in TR 23.700-13 [7] should be used as basis for UE Reader authorization.

-	Potential impact from how the NG-RAN node and UE reader selection can be performed.

NOTE 3:	The aspect of NG-RAN node and UE reader selection requires coordination with RAN WG3.

-	Whether and how the AIOTF can provide the information of the UE readers to the NG-RAN node to assist NG-RAN for UE reader selection.

NOTE 4:	Rel-19 interim conclusion in TR 23.700-13 [7] should be used as basis for this key issue. No Rel-19 AIoT Device impact from this key issue is expected.

NOTE 5:	Conclusions of this KI will be coordinated with RAN WGs.

The solution in this paper has been submitted and discussed in Rel-19 study phase; and it is re-submitted to address the KI#1 in Rel-20 which brings the scenario for hybrid deployment of Topology 1 and 2.

The paper is proposed to introduce a new solution to aggregate the service operation for Topology 1 with NG-RAN Reader and Topology 2 with UE Reader.

-	Topology 1: BS <--> Ambient IoT Device;

-	Topology 2: BS <--> intermediate node <--> Ambient IoT Device: Only a UE can act as an intermediate node which is under the network control.

In the scenario which both Topology 1 and Topology 2 are deployed; and Topology 1 with NG-RAN Reader is main deployment, and Topology 2 with UE Reader is supplementary deployment in case the devices cannot be accessed by RAN Reader; when AF sends service operation to the network, how to aggregate the operation for devices in both Topology 1 and Topology 2 needs to be studied.

2. Proposal

It is proposed to capture the following changes into TR 23.700-30.

* * * * Start of Changes * * * *

6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |
| --- | --- | --- |
| Solutions | Key Issue #1 | <Key Issue #2> |
| #1 |  | X |
| #2 |  | X |
| #3 |  | X |
| #4 |  | X |
| #X |  | X |

* * * * Second Change (All new text) * * * *

6.X	Solution #X: Service Operation Aggregation for Topology 1 and Topology 2

6.X.0	High-level solution Principles

In this solution, when receiving the service operation from AF, the network will firstly interact with NG-RAN Readers to execute service operation to target devices; and then determines whether the operation for all target devices is completed based on the device operation results: if not completed the network will interact with UE Readers to execute service operation to the rest devices. When the service operation to all target devices is finished, the network aggregates the service operation results from NG-RAN Readers and UE Readers, and sends the results to AF.

6.X.1	Description

This solution applies for the scenario in which both Topology 1 and Topology 2 are deployed; and Topology 1 with NG-RAN Reader is main deployment, and Topology 2 with UE Reader is supplementary deployment in case the devices cannot be accessed by NG-RAN Reader. The mixed deployment of both NG-RAN Reader and UE Reader can expand the management coverage of AIoT devices. When AF sends service operation to the network, the request message includes the following information: service type, i.e. Inventory, Command; where the Command specifically includes Read, Write, Disable, etc. operations; the target device of the service request can be the Device ID of a single device, a group of Devices with filtering information, or all devices in a specific area. The network will first interact with NG-RAN Readers to execute service operation to target devices; and then determines whether the operation for all target devices is completed based on the device operation results: if not completed the network will interact with UE Readers to execute service operation to the rest devices. Alternatively, the network can select different type of Readers based on the policy (e.g., different priority): either NG-RAN Reader or UE Reader could have high priority for certain service operation; or these two types of Reader have same priority which means they will be operated as same time. When the service operation to all target devices is finished, the network aggregates the service operation results from NG-RAN Readers and UE Readers, and sends the results to AF.

In this solution, the AIOTF is used to interact with NG-RAN Readers and UE Readers to execute service operation to devices targeted by service request, and aggregate the service operation results from NG-RAN Readers and UE Readers.

Figure 6.X.1.2-1 depicts the reference architecture of service operation aggregation for Topology 1 and Topology 2.

Figure 6.X.1-1 Reference Architecture

In the architecture, the AIOTF is used to interact with NG-RAN Readers and UE Readers to execute service operation to devices targeted by service request, and aggregate the service operation results from RAN Readers and UE Readers. The architecture with a single AIOTF addresses the scenario of limited area coverage of devices; for large area coverage, multiple AIOTF serving large numbers of multiple Readers ( UE or NG-RAN Readers) may work together to handle the AF service request for the devices in large area. Other network elements include ADM, AUSF, NRF, NEF, and AF; these network elements fulfil the existing functions in 5G core network, and are enhanced to support AIoT services in this solution.

6.X.2	Procedures

Figure 6.X.2-1 depicts a flow diagram for service operation aggregation for Topology 1 and Topology 2.

Figure 6.X.2.1-1: Initiating message and data transfer procedure

The steps in the service operation aggregation for Topology 1and Topology 2 procedure are as follows:

0.a. NG-RAN Reader reports its AIoT capability and serving area to AIOTF; AIOTF registers to NRF with the information of its serving NG-RAN Readers.

0.b. UE Reader registers to the core network indicating its AIoT capability.

1. AF sends a service request for AIoT devices to NEF. The request message includes the following elements: service type, i.e. Inventory, Command; where the Command specifically includes Read, Write, Disable, etc. operations; the device information of the service request can be the Device ID of a single device, a group of Device IDs, multiple Device IDs with MASK filtering conditions, or devices in a specific area.

2. NEF verifies the service request from AF and queries NRF for the relevant AIOTF information that serves the devices according to the device information in the request.

3. NEF sends the service request for AIoT devices to AIOTF.

4. AIOTF interacts with UDM to verify whether the service request is allowed; if allowed, AIOTF discovers and selects NG-RAN Readers serving devices according to the device information targeted by the service request.

Alternatively, AIOTF can determine the priority of two types of Readers based on the policy: either NG-RAN Reader or UE Reader could have high priority for service operation; or these two types of Readers have same priority which means they will be operated as same time. The following steps assume the NG-RAN Reader has high priority for service operation.

5. AIOTF sends the service request for AIoT devices to the selected NG-RAN Reader.

6. NG-RAN Reader interacts with multiple AIoT devices to complete the service operation.

7. NG-RAN Reader returns the service operation results of the AIoT devices to AIOTF.

The above steps 5~7 will repeat for multiple selected NG-RAN Readers.

8. The service request target is for one or multiple specific devices, AIOTF determines whether the operation for all devices in the service request is completed based on the device operation results sent from NG-RAN Readers in steps 5~7: if completed, executes step 9.a.

If AIOTF determines that the operation for all devices in the service request is not completed, then executes steps 9.b~12.b.

9.a. AIOTF returns the service operation results of AIoT devices to AF through NEF.

9.b. AIOTF sends the service request of AIoT devices to UE Reader through the NG-RAN serving UE Reader. The NG-RAN serving UE Reader is not the same NG-RAN as NG-RAN Reader.

10.b. UE Reader interacts with AIoT devices to complete the service operation.

11.b. UE Reader returns the service operation results of AIoT devices to AIOTF.

The above steps 9b~11b will repeat for multiple selected UE Readers.

12.b. AIOTF aggregates the service operation results from NG-RAN Readers and UE Readers, and returns the results to AF through NEF.

6.X.3	Impacts on Services, Entities and Interfaces

AIOTF:

-  Interact with UE Readers and RAN Readers for service operation.

-  Aggregate service operation result from both UE Readers and RAN Readers.

-  Send aggregated service result to AF.

NRF:

-  Store and manage AIOTF information.

NEF:

-  Receive AIoT service requests from AF and send back aggregated service operation result to AF.

-  Interact with AIOTF for AIoT service operation.

Reader (NG-RAN/UE):

-  Receive AIoT service requests from AIOTF and send back service operation result to AIOTF.

-  Interact with AIoT devices to execute service operation.

* * * * End of Changes * * * *