3GPP TSG-SA WG2 Meeting #171	S2-2508454

October 13-17, 2025 Wuhan, China

Source: 	OPPO

Title: 	Interim conclusion for KI#1 in TR 23.700-19

Document for: 	Approval

Agenda Item: 	20.1.1

Work Item / Release:	FS_5GSAT_Ph4_ARC / Rel-20

Abstract of the contribution: This contribution proposes an interim conclusion for KI#1 in TR 23.700-19 (FS_5GSAT_Ph4_ARC).

1	Discussion

In TR 23.700-19 v0.3.0, 11 solutions are agreed for KI#1. Regarding interim conclusion, we provide the proposals based on the considerations as below.

Consideration for PDN connection:

- For NB-IoT, EPS Attach without PDN connection request has been supported and should be used, which helps only establishing IMS PDN connection for GEO voice case. In addition, the PDN connection mandatory has default bearer. Thus, establishing one PDN connection (IMS APN) for UE using GEO voice helps saving the total number of EPS bearers in order to fulfil the upper limits of DRB numbers for EPS NB-IoT (currently 2).

- Regarding IMS signalling transfer, in regular mechanism, when the IMS PDN connection established, the QCI=5 default bearer (non-GBR) via UP will be established for IMS signalling. It should reuse QCI=5 EPS bearer for IMS signalling transfer in GEO voice case.

- Regarding IMS voice media transfer, regularly the QCI=1 dedicated bearer (GBR) is dynamically established during call setup and removed when the call is finished. It should use a dedicated bearer for IMS media for GEO voice as well, however 1) whether a new index/QoS is defined belongs to normative work, and 2) whether a pre-established or dynamically established dedicated bearer for voice media will be decided in normative phase.

Proposal-1: UE using GEO voice only supports one PDN connection (IMS APN), which is established after performing EPS attach without PDN connection request.

Proposal-2: In IMS PDN connection, default bearer with QCI=5 via UP is reused for IMS signalling transfer, and dedicated bearer via UP is used for IMS voice media with a new index/QoS needs to be defined in normative phase.

Consideration for PDN type (IP/Non-IP):

- Typically, in market, the native IMS signalling and voice media are transferred over RTP/UDP/IP. The most secure way is to reuse IP type PDN connection to avoid a lot of unforeseen complexity and challenges on implementation. This implementation impacts exist on both NAS layer and IMS layer.

- The overhead of IP header is quite limited. A typical example is 40 Byte for IP header and 8 Byte for UDP header. Compared to a conventional SIP message (e.g., 1200 Byte for INVITE), the benefit is very limited and helps little to fulfil the call setup time requirement (30s).

NOTE: assuming 1Kbps data rate via GEO, the time saved by non-IP message is 48 Byte/1Kbps=0.384s for each SIP message only.

-   RAN has supported RoHC, which helps reduce the IP header to [3] Byte (or even smaller). Then the overhead is ignorable.

-	Compared to removing IP header, other methods such as B2BUA, compressed SIP message, or Binary (only impacting IMS layer) can achieve much benefits to reduce the call setup time into less than 30s.

Observation: Compared to other methods (B2BUA, Text/Binary based message), the Non-IP optimization cannot be used independently because it has little benefits and still need other optimization to achieve the ideal call setup time (30s). On the other hand, one of the other methods can achieve the call setup time goal (30s). Instead of Non-IP, RoHC has been widely used.

Proposal 3: IP type PDN connection shall be mandatorily supported for GEO voice. Whether the Non-IP is considered depends on if it can achieve the 30s call setup time by using other methods (pending on KI#2 conclusion) and if the RoHC can be applied for GEO voice (pending on RAN LSreply).

2 Proposal

# * * * * First Change* * *

7	Interim Agreements

7.1	Agreed Principles

7.1.X	Interim principles for Key Issue#1

Proposal-1: UE using GEO voice only supports one PDN connection (IMS APN), which is established after performing EPS attach without PDN connection request.

Proposal-2: In IMS PDN connection, default bearer with QCI=5 via UP is reused for IMS signalling transfer, and dedicated bearer via UP is used for IMS voice media with a new index/QoS needs to be defined in normative phase.

Proposal 3: IP type PDN connection shall be mandatorily supported for GEO voice. Whether the Non-IP is considered depends on if it can achieve the 30s call setup time by using other methods (pending on KI#2 conclusion) and if the RoHC can be applied for GEO voice (pending on RAN LSreply)

# * * * * End of change * * * *