SA WG2 Meeting #171	S2-2508537

13 – 17 October 2025, Wuhan, CN

Source:	Boost Mobile Network, EchoStar, China Mobile, Cisco Systems, KPN N.V., Telecom Italia S.p.A, Verizon

Title:	[General Aspects] Key Issue for 6G Architecture support Stateless NF Services

Document for:	Approval

Agenda Item:	20.6.0

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: This contribution proposes a key issue for the 6G system architecture to support cloud native features of all NF services being stateless.

# 1. Justifications

The 6G Study item objective says the following: “The study should consider at least the following aspects: cloud native, sustainability and energy efficiency, robustness and resiliency, etc.” Supporting cloud native has started in 5G where SBA has been supported in addition to restful interfaces. This contribution proposes a key issue where the 6G system architecture should support the cloud native feature of stateless NF services.

# 2. Discussion

There are several types of benefits of cloud native technology that 6G architecture and deployment can benefit from. Some of the commercial benefits are reduced cost, faster time to market and enhance customer experience. Other benefits that improve the operational efficiency of the 6G architecture deployment are rapid provisioning of resources, increased agility and flexibility, fully automated and secure CI/CD, and In-service up-grade and downgrade. In addition, stateless services is one of the important yet critical features of cloud-native.

Having a 6G core network supporting stateless NF services provides many advantages which enable scalability, fault tolerance and resilience, simplified deployment and management, enhanced performance and efficiency, etc.

#### 2.1 Scalability:

- Having 6G core network supporting stateless NF services ensures all 6G network functions support being stateless. This is fundamental for horizontal scalability where adding more servers or instances can handle increased load.

- Supporting stateless NF services means no session dependency and each request is independent and can be handled by any available instance where enhanced and efficient load balancing can be achieved.

- In addition, stateless NF services do not require server-side data session storage, thus frees up memory and CPU resources.

#### 2.2 Fault tolerance and resilience

- Supporting stateless NF services allows failure isolation where the failure on one server does not impact other servers.

- In case of failure, requests can be seamlessly routed to other available instances where service is not interrupted.

- Finally, it eliminates the need for complex state maintenance and replication and thus it provides easier recovery and failure management.

#### 2.3. Simplified deployment and management

- Stateless services have simpler configurations based on predefined rules or policies and don't need to maintain connection states.

- Stateless services can be deployed and updated independently allowing quick adoption for new features based on faster response to new or modified requirements.

- Debugging and troubleshooting are simplified as each transaction is treated independently, making it easier to pinpoint the source of issue.

# 3. Conclusion

Although, services and network functions being stateless is not new to 3GPP standardization and systems, there is an increased need for 6G core network architecture to support stateless NF services and/or NF applications. This is driven by the need to have a highly scalable, resilient to failures, and easy to deploy and manage 6G core network.

Stateless 6G core network NF services and network functions address these demands by decoupling the session state from the application logic. This allows for a greater flexibility and efficiency in 3GPP mobile network environments

A key issue is needed to study how to ensure that all 6G core network NF services support being stateless.

# 4. Proposal

It is proposed to add the following key issue to the TR 23.801 “Study on Architecture for 6G System”.

* * * Start of changes * * * *

# 5	Key Issues

Editor's Note: This clause identifies key issues to be addressed.

## 5.X	Key Issue #X: 6G Core Network Architecture support stateless NF services

As part of 6G Architecture study, this key issue is focused on how the 6G core network architecture support stateless NF services.

NOTE 1: Achieving stateless NF services can be through decoupling the session state from the application logic.

NOTE 2: Having stateless NF service and/or NF applications is necessary for efficient local and geo high availability.

* * * End of changes * * * *