3GPP TSG-WG SA2 Meeting #171	S2-2508709

13th – 17th October 2025, Wuhan, CN	(revision of XXXXXX)

Source:	China Telecom

Title:	New Solution: Support of DO-A capable AIoT Device for AIOTF selection and routing to AF

Document for:	Approval

Agenda Item: 	20.5.1

Work Item / Release:	FS_AmbientIoT_Ph2_ARC / Rel-20

Abstract of the contribution: This paper proposes the solution on how to Support DO-A capable AIoT Device selecting or routing to the Serving AIOTF.

# 1	Introduction

Based on the Key Issue 2 (TR 23.700-030), the following work be studied:

Key Issue #2: Support of DO-A Capable AIoT Devices

This key issue will study the system architecture to support DO-A capable Ambient IoT Devices in Topology 1 and Topology 2.

The following aspects will be studied:

-	How the AIoT Device informs the network of its presence autonomously (e.g., an AIoT Device initiated registration-like procedure) and what are the triggers for the DO-A capable device to inform the network of its presence.

-	Whether and how to consider power consumption of DO-A Capable AIoT Devices.

-	How an AIoT Device sends data to the AIOTF autonomously.

-	Support for routing the data received by AIOTF from an AIoT Device to an AF.

-	Whether and how to enhance the Inventory and Command procedures defined in TS 23.369 [3] to support DO-A capable AIoT Devices.

-	Naiotf, Namf and Nnef interface enhancements to support DO-A capable AIoT Device .

NOTE 1:	The conclusions from Key Issue #1 are the basis for supporting DO-A capable AIoT Devices in topology 2 in this key issue.

NOTE 2:	Coordination with RAN WGs is required.

Motivation of this solution:

In the issue ‘How an AIoT Device sends data to the AIOTF autonomously’, there may be a sub-issue to discuss, that how the NG-RAN/Reader discovers and selects the serving AIOTF when receiving the AIoT DO-A message, since the AIoT Device may not have the serving AIOTF information. Different from a non-DO-A device that the AIOTF will provide the information of itself to the NG-RAN/Reader. Under the DO-A mechanism, the AIoT device and NG-RAN/Reader may not have this information when AIoT device initiates the DO-A traffic for the first time.

Different from the 3GPP UE, which can provide the AMF info when a UE sends a request to the network, AIoT device may not have this capability. So the key is to make some enhancements on the Reader to help AIoT device discover and select the AIOTF.

Furthermore, there are so many methods and possible cases to select the serving AIOTF. This should depend on the real scenarios or service requests. Refer to the ‘UE-AMF’ mechanism, the AIOTF selection may have 2 methods: relocation or redirection. For example, when the AIoT Device is in an outdoor scenario or has high mobility, selecting the closest AIOTF may be more appropriate, i.e., relocation. If the AIoT Device has higher data privacy requirements or a specific service agreement, routing the AIoT request to the last known AIOTF may be better, i.e., redirection.

Besides, regarding another bullet in the key issue description about “ Support for routing the data received by AIOTF from an AIoT Device to an AF”, it is necessary for the AIOTF to know which AF shall it forward the traffic from the AIOT device to, especially for the DO-A when the data is coming firstly from the AIoT device.

Based on these considerations, we propose this solution to support DO-A capable AIoT Device for initial AIOTF selection and routing, and also the routing from AIOTF to the AF.

# 2. Text Proposal

It is proposed to capture the following changes to TR 23.700-30.

# * * * * First change (All new text) * * * *

## 6.X	Solution #X: Support of DO-A capable AIoT Device for initial AIOTF selection and routing

### 6.X.0	High-level solution principles

This solution addresses Key Issue #2 “Support for routing the data received by AIOTF from an AIoT Device to AF”. The key technical principles proposed in this solution are summarized below:

- The AIOTF selection has 2 different principles with 4 potential cases:

  - The serving AIOTF is provided:

    - The AIoT device can provide serving AIOTF info e.g., the AIoT temporary ID may contain this info.

    - The AIoT service context in NG-RAN/Reader may include serving AIOTF info.

  - The serving AIOTF is unknown for NG-RAN/Reader:

    - The default AIOTF can be the serving AIOTF in relocation determination.

    - The default AIOTF can help routing the request to the serving AIOTF in redirection determination

- The NG-RAN/Reader can be pre-configured with default AIOTF info.

- The AIOTF can route the DO-A data traffic received from the Device to the target AF, based on the subscribed AF information for the device, which is retrieved from the ADM.

### 6.X.1	Description

This solution aims for the case of how to discover and select the serving AIOTF when the DO-A capable AIoT Device sends the request/message to the network. As DO-A capable AIoT Device may already have been registered in the network, the ADM should have the AIoT subscription information based on SLA (e.g., AIoT device profile data, AF authorization data) which may contain the related AIOTF information, i.e., last known AIOTF information for the device. But the NG-RAN/Reader, different from the ADM, may or may not have this info, especially when the AIoT Device is mobile.

Furthermore, considering the capability of the AIoT Device and various scenarios of AIoT, such as warehouse, supply chain, and individual tags, different scenario may have different reqirements, resulting in different preferences to discover and select the serving AIOTF such as mentioned before (e.g., AIOTF relocation and AIOTF redirection).

Based on these considerations, there are a few assumptions for this solution:

Assumption:

- The ADM should further include the AIOT device service agreement and the subscribed AF information.

- The last known AIOTF should have the service context that targets at the AIoT device(s).

- The NG-RAN/ Reader may or may not have the context or the serving AIOTF info.

- The AIoT Device may or may not have the serving AIOTF info.

Based on these assumptions, for 4 potential cases, when the AIoT device or service context in NG-RAN/Reader have the serving AIOTF info, the NG-RAN/Reader only need to check this info and route the request to this AIOTF. However, when the serving AIOTF is unknown, the key is 1) how to perform the AIOTF selection and 2) how the initial AIOTF routes to or is confirmed as the serving AIOTF.

For the 1) issue, this solution proposes a pre-configuration on the NG-RAN/ Reader, which contains the default AIOTF info for the initial AIOTF selection. The pre-configuration is generated from the network, usually according to the deployment of the Reader or the AIoT service from the service provider.

For the 2) issue, this solution proposes two methods based on different scenarios and considerations. In general, the default AIOTF will obtain the AIoT subscription information and context, including the last known AIOTF, and it will determine to relocate or redirect the serving AIOTF to execute the DO-A capable AIoT request.

To better understand this solution, here are the explanations and definitions of the AIOTF wording:

- Default AIOTF: The default AIOTF is the prior selection for the NG-RAN/ Reader when the serving AIOTF info is unknown.

- Last known AIOTF: The last known AIOTF info is that the former AIOTF executed the last AIoT request for the AIoT device.

- Serving AIOTF: the serving AIOTF is the AIOTF to execute the current DO-A request. When the serving AIOTF is unknown for NG-RAN/Reader at first, both the default AIOTF and the last known AIOTF can be the serving AIOTF, based on the determination of the default AIOTF.

Here are the descriptions of the relocation and redirection:

Relocation: The default AIOTF determines itself to be the serving AIOTF; the default AIOTF may retrieve the context from the last known AIOTF, that includes the service context which targets at the AIoT device. Then the default AIOTF will request ADM to update the last known AIOTF information, and the default AIOTF will become the serving AIOTF, executing the AIoT request.

Redirection: The default AIOTF determines that the last known AIOTF is the serving AIOTF; the default AIOTF will provide the last known AIOTF info to the NG-RAN/ Reader. Then the NG-RAN/ Reader will transmit the AIoT request to the last known AIOTF to execute this request. In this case, the last known AIOTF may hold security context for AIoT device, which can not be transmitted. This may lead to redirection rather than relocation in the AIOTF selection.

### 6.X.2	Procedures

#### 6.X.2.1	AIOTF selection procedure with serving AIOTF info provision

Figure 6.X.2.1-1. AIOTF selection procedure with serving AIOTF info provision

In this solution, the AIoT device or NG-RAN/Reader should already have the serving AIOTF information. For an AIoT device, the serving AIOTF info may be included in the initial request or in the Temporary ID.

Step 0: AIoT device receives radio resource allocated from the NG-RAN/ Reader.

Editor’s Note: How the NG-RAN/Reader allocates the radio resource is to be decided by RAN WGs.

Step 1: When the AIoT device is triggered by itself, it initiates an initial DO-A (Device Oriented-Autonomous) request to the Reader.

Step 2: The NG-RAN/Reader retrieve the serving AIOTF info by checking the AIOTF info from the AIoT request (e.g., the AIoT Device Temporary ID) and the AIoT serving context in the NG-RAN/Reader .

Editor’s Note: Whether and how the AIoT temporary ID contains the serving AIOTF info is to be decided by SA3.

NOTE 1: For the information check. The suggestion is that if the Reader is a BS Reader, the checking is executed by the high-level RAN node/unit; if the Reader is a UE Reader, the checking is executed by the Reader itself.

Step 3: The NG-RAN/Reader obtains the serving AIOTF information and sends the UL request to the serving AIOTF. The UL request should contain the AIoT device ID and may include a service request (e.g., registration request, subsequent data transmission indication, etc.)

NOTE: The indirect connectivity where AMF is between the AIOTF and NG-RAN will be investigated in other solution.

Step 4: The AIOTF performs the AIoT device authentication and authorization procedure, coordinating with ADM. The AIOTF and ADM update the context.

NOTE 2: How the authorization and authentication is performed is based on SA3.

Step 5: The AIOTF sends the DL message to the NG-RAN/Reader; the message may contain the service response for the AIoT device (e.g., subsequent data transmission approved, registration accept, etc.)

Step 6: The NG-RAN/Reader sends the DO-A response to the AIoT device.

Step 7: After the DO-A AIoT device accesses the network, it can send service requests (e.g., AIoT data transmission )

Step 8: AIOTF checks the subscribed AF information (e.g., AF ID) for the device towards the ADM, using e.g., device ID as the key. The subscribed AF is the AF allowed to receive the AIoT data. It is assumed that the ADM is enhanced with subscribed AF information for the device data exposure/report.

Step 9: Based on the subscribed AF information, the AIOTF sends the uplink data received from the AIoT device to the AF, either directly, or via NEF, that contains device ID, DO-A data.

.

#### 6.X.2.2	AIOTF selection and relocation/redirection procedure

Figure 6.X.2.2-1. AIOTF selection and relocation/redirection procedure

In this solution, the AIoT device and NG-RAN/Reader do not have the serving AIOTF info, so the network should select an AIOTF as the serving AIOTF, dealing with the DO-A capable AIoT request.

Step 0a: Pre-configure the default AIOTF information on the NG-RAN or Reader.

NOTE 3: The information of the default AIOTF can be provided by AIOTF itself, ADM, or the OAM system. It usually depends on the AIoT service deployment or the service provider.

NOTE 4: For the pre-configuration of default AIOTF on whether NG-RAN or Reader. The suggestion is that if the Reader is a BS Reader, the pre-configuration is on the high-level RAN node/unit; if the Reader is a UE Reader, the pre-configuration can on the Reader itself.

Step 0b and 1 are the same as 6.X.2.1

Step 2: The NG-RAN/Reader sends the UL request to the default AIOTF. The UL request should contain the AIoT device ID and may include a service request (e.g., data transmission indication, registration, etc.)

Step 3: The default AIOTF retrieves the last known AIOTF and service-related information from ADM. The last known AIOTF info is in the AIoT profile data, and the service-related information may include the AIoT service agreement, the AIoT subscribed AF. In addition, the AIoT service agreement is the allowed services for AIoT device, e.g. allowed service area. The service-related information is assisted for the AIOTF selection.

Step 4: Based on the information provided by the ADM, the default AIOTF determines to relocate the serving AIOTF, that the default AIOTF becomes the serving AIOTF (option a below), or to redirect to the serving AIOTF, i.e., redirecting the UL AIoT request to the last known AIOTF (option b below).

NOTE 5: How the default AIOTF determines to relocate, or redirect is based on its implementation (e.g., AIOTF determines by the local configuration or AIOTF determines based on the service level agreement (SLA) between the 3rd party AF and the 5GS). In some cases, if the AIoT service-related information indicates some special requirements e.g., the last known AIOTF has security context which is not permitted to transmit, the default AIOTF should determine to redirect rather than relocate.

Relocation procedure:

Step 5a: The default AIOTF (now the serving AIOTF) retrieves the device context from the last known AIOTF according to the device ID, that may include the service context for the device. The service context can include, e.g., transaction ID, AF ID, target area information, and other assistance information associated with the AIoT service as described in TS 23.369.

Step 6a: The last known AIOTF responses to the default AIOTF.

Step 7a: The default AIOTF updates the context of last known AIOTF in ADM, and checks the authentication and authorization of the AIoT device.

Step 8a: The AIOTF sends the DL message to the NG-RAN/Reader; the message may contain the service response (e.g., subsequent data transmission approved, registration accept, etc.)

Redirection procedure:

Step 5b: The default AIOTF sends DL message to the NG-RAN/Reader. The DL message should contain the device ID,   serving AIOTF info, and the indication of AIOTF redirection.

Step 6b: The NG-RAN/Reader re-sends the UL request to the serving AIOTF as indicated in step 5b.

Step 7b: The serving AIOTF, coordinating with ADM, updates the context and checks the authentication and authorization of the AIoT device.

Step 8b: The serving AIOTF sends the DL message to the NG-RAN/Reader; the message may contain the service response (e.g., subsequent data transmission approved, registration accept, etc.)

Step 9-12 are the same as step 6-9 in 6.X.2.1. The routing from the serving AIOTF to the target AF follows the procedure as illustrated in the clause 6.X.2.1 step 8 and step 9.

### 6.X.3	Impacts on Services, Entities and Interfaces

AIoT Device:

- DO-A capable

- Provide serving AIOTF info in some cases.

AIOTF:

- Determine to relocate or redirect the serving AIOTF

- Coordinate with ADM to update last known AIOTF

- Send serving AIOTF info to the NG-RAN or Reader

- Caching the security context for AIoT Device if necessary

- Retrieve the subscribed AF information from the ADM for the AIoT device DO-A UL traffic.

- Retrieve the service context from the last known AIOTF.

ADM:

- Manage the AF and AIOT service agreement

- Store the subscribed AF information for the AIoT device

AIOT Reader/ NG-RAN:

- Pre-configured with default AIOTF info

- Context in NG-RAN/Reader can contains the serving AIOTF info

- Check the AIOT info whether contains the serving AIOTF info

# * * * * End of changes * * * *