SA WG2 Meeting SA2 #171	S2-2508878

13 - 17 October, 2025, Wuhan, China

Source:	China Mobile

Title:	Interim Agreement for KI#1: UE reader authorization and revocation

Document for:	Approval

Agenda Item:	20.5.1

Work Item / Release: FS_AmbientIoT_Ph2_ARC / Rel-20

Abstract of the contribution: This contribution proposes the interim agreements for FS_AmbientIoT_Ph2_ARC / Rel-20.

- Discussion

The AIOTF should only select the UE readers that belong to the AF, which initiates the AIoT service request. For example, the Costco AF can only select its own UE readers but can’t choose the Home Depot UE readers to perform the AIoT service request from the Costco AF.

Therefore, the AF ID should be part of the UE reader’s subscription. Then AIOTF can use the AF ID as validation information.

The AIOTF can verify the UE readers during the selection process based on the AF ID in the UE reader’s subscription and the AF ID, which initiates the AIOTF service request.

Additionally, the UE reader can be limited to a specific location. Therefore, the allowed TAC list or forbidden TAC list can be part of the UE reader’s subscription, too.

Proposal:

Adding AF ID, the allowed TAC list, and the forbidden TAC list information to the UE reader’s subscription.

- Proposal

It is proposed to adopt the following changes to TR 23.700-30.

* * * * First Change (all new text) * * * *

7	Interim agreements

7.1	Agreed Principles

7.1.1	Agreed Principles for KI#1

The afreed principles of UE reader authorization and revocation are as follows:

-	Subscription aspects:

-	The UE subscription in the UDM will be extended with UE Reader subscription information, which consists of the following:

-	information indicating whether the UE is allowed to operate as a UE Reader;

-  AF ID information indicating that the UE reader belongs to the designated AF;

-  Allowing TAC list;

-  Forbidden TAC list.

Editor’s note:	Additional subscription information, e.g., validity information, for the UE Reader is FFS.

-	UE Reader subscription information is available to AMF and AIOTF;

-	If AMF receives, as part of the subscription information, the indication that the UE is authorized to operate as a UE Reader, then AMF informs NG-RAN that the UE is authorized to operate as a UE Reader;

-	If the subscription information has been revoked, the indication that the UE is not authorized to operate as a UE Reader from the UDM, then the AMF shall inform NG-RAN that the UE is not authorized to operate as a UE Reader.

Editor’s note:	Whether and how to enable authorization to the UE is FFS.

* * * * End of the Change * * * *