SA WG2 Meeting #171	S2-2508942

13 – 17 Oct, 2025, Wuhan, China

Source:	InterDigital

Title:	[WT#4] WT proposal for Integrated Sensing and Communication

Document for:	Agreement

Agenda Item:	20.6.4

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: This contribution proposes update of WT proposal for WT#4 Integrated Sensing and Communication for 6G based on the postponed S2-2508043.

# 1. Justifications

Editor's Note: This justification section is not included in the TR; it is intended solely to justify the scope of the proposed WT.

S2-2508043 indicates the aggregated WTs for 6G ISAC. However, several requirements from TS22.137 are still missing:

- Subject to operator’s policy, the 5G System shall be able to provide the 5G wireless sensing service in case of roaming.

- Subject to local regulation, the 5G network shall enable UEs without 5G coverage to use unlicensed spectrum to provide 5G wireless sensing service.

- Subject to regulation, the 5G network shall enable UEs supporting V2X application to perform 5G Wireless sensing when not served by RAN using the allowed ITS spectrum and unlicensed spectrum.

Assuming the UEs will be considered in 6G ISAC with mobility support based on S2-2508043, it is essential to take roaming support into account for the service.

For the last two requirements above, technologies enabling sensing services in out-of-coverage scenarios will be required. However, it is understood that these have dependencies on the availability of V2X or D2D technology within the 6G system. Therefore, it is proposed to consider the use of unlicensed spectrum or ITS spectrum for 6G ISAC, while deferring the actual study of these topics until the relevant technologies are further developed.

**** First Change ****

# Annex A.X. WT#4 Scope

WT#4: Study the integration of Sensing and Communication over 3GPP access, considering the sensing modes to be supported and other sources of sensing data.

The following aspects will be studied:

a)	Architecture and function support for sensing, including:

b)	Service authorization and revocation.

c)	Discovery and (re-)selection of Sensing Entities.

d)	Configuration parameters and policy provisioning to support sensing service, e.g., to the selected Sensing Entities, Sensing function, etc.

e)	Collection, distribution of sensing related data and exposure of sensing result.

f)	Sensing continuity in consideration of mobility of sensing entities and/or target objects.

g)	Support for non-3GPP sensing data and data fusion.

h)	Support of roaming for sensing service when sensing service consumer and/or target sensing objects roams.

i)	Support of sensing operation for coverage-limited scenario.

NOTE X1:	WT#4 needs to consider the work and conclusions of R20 FS_Sensing_ARC on gNB-based sensing.

NOTE X2:	The scope of WT#4 includes sensing modes and their selection, where sensing may involve RAN node and UE.

NOTE X3:	Aspects a), b) and e) require coordination with WT#5. The data framework applicable for collection, distribution and exposure of sensing related data is in scope of WT#5.

NOTE X4:	Coordination with RAN WG for RAN aspects, SA3 for privacy and consent aspects, and SA5 for OAM aspects will be needed.

NOTE X5:	The scope of WT#4 will be aligned with RAN for 6G. SA2 could evaluate the situation in Q1 2026, for possible scoping discussions during Q2 2026.

NOTE X6:	Aspect i) may be studied once V2X or D2D services over 6G are specified.

**** End of Changes ****