SA WG2 Meeting #171	S2-2509158

13 – 17 OCT, 2025, Wuhan, China

Source:	Ofinno

Title:	[WT#1.2.X, User Plane] [WT#1.2.X, QoS] User Plane Architecture and QoS Framework in 6G

Document for:	Approval

Agenda Item:	20.6.1.2

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: This paper proposes the WT scope and introduces a new KI associated with WT#1.2 to support 6G QoS and 6G user plane.

# 0. Background

At SA2 #170, SA2 discussed proposals on updating WTs for FS_6G_ARC. S2-2508121 and S2-2508122 related to QoS and User plane aspects were discussed as baseline documents and finally were postponed.

Following are text proposals in S2-2508121 and S2-2508122.

# Annex A.X. WT#1.2 User Plane Architecture for 6G Scope In order to support 6G user plane for a diverse set of applications and traffic patterns, the following are studied: - Whether and how to enhance application-network collaboration for 6G user plane traffic to enable the UE/AS to provide the network with information that allows the network to adapt to application traffic. Editor’s Note: whether this issue belongs to UP or QoS is FFS. - Whether and how to enhance CP-UP interaction for better interoperability. - Whether and how to extend the service-based architecture for the user plane control interface to UPF. - Whether and how to support flexible user plane (i.e., UPF (re)selection) for different service requirements considering path performance and capability between access network and data network. - Whether and how to enhance the user plane handling for session continuity and mobility between access network and data network (i.e., best path selection considering ULCL and PSA-UPF path metrics, and/or for specific services).

# Annex A.X WT#1.2.X: Evolution of QoS Framework for 6G - Investigate if any new functionality in 6G QoS framework considering the emerging new traffic characteristics and application needs. NOTE 1: The definition of 6GS QoS concepts in the 6G System provider perspective and in the 6G System usage perspective respectively should have clear boundary (enable clear & measurable connectivity service descriptions for SLAs). - Whether and how to enhance QoS control mechanisms to - better support QoS targets than the best effort approach of existing non-GBR and less resource intensive than the existing GBR, Delay Critical-GBR to improve capacity utilization in the network. - to improve QoS for applications with dynamic requirements. - Whether and what to monitor in relation to QoS Framework. - Study whether and how to support the UE and network QoS collaboration to improve awareness in the 6GS of connectivity performance needs. - UE provided information to the network for QoS collaboration - Application and Network QoS collaboration including what information shall be exchanged by means of ‘application/UE and network collaboration’ [see WT#1.2 User Plane]. NOTE 2: Applications typically consist of an Application Client in the UE and application on the server side. - Study how to ensure interworking with 5GS. NOTE 3: The content of the 5GS QoS Framework that will be used as a starting point for discussion on 6G QoS will be determined by moderated discussion process. NOTE 4: Aspects of traffic detection are assumed to be handled by the WT#1.2 User Plane. NOTE 5: Impacts on QoS originating from new services will be considered based on and when relevant input is identified and provided by the relevant WTs. NOTE 6: This WT will coordinate with SA WG4 and RAN WGs.

# 1. Discussion

In the 5GS architecture, based on the principle of separation between the control plane and the user plane for session management, the SMF is a node in charge of signalling exchange with a UE and NFs, and the UPF is in charge of user data exchange with the UE. By making the SMF be focused on overall control and UPF be focused on user data handling, the principle enables independent evolution of the control plane and the user plane, and independent scaling of control plane resources and user plane resources.

However, when the SMF is the only point of contact for SM signalling, several challenges arose:

- SMF overload due to involvement in all signalling related to a PDU session

Signalling related to the PDU session is terminated at the SMF. It may be a logical approach at least for establishment of the PDU session. However, it is questionable whether, after the PDU session is established, all signalling messages should be delivered via the SMF, because this design choice may cause frequent congestion at the SMF.

Many contributions presented during the 6G workshop mentioned that core network functionalities and procedures rely heavily on both the AMF and the SMF, and that the SMF often acts just as an intermediate relay in the signalling path.

For example, during secondary DN authentication, the UE and the UPF exchanges authentication messages via the SMF and the AMF, instead of direct signalling exchange between the UE and the UPF. Then, the UPF and an AS exchange further signalling messages via N6, based on the authentication messages. In this exchange, until the end of the authentication procedure, the role of the AMF and the SMF is just forwarding the authentication messages between the UE and the UPF.

Another example is a support for TSN. TSN-related containers (e.g., PMIC) for DS-TT is exchanged between the UE and the TSCTSF via the SMF. NW-TT in the UPF also exchanges the TSN-related containers, with the TSCTSF via the SMF. Considering that the NW-TT and the DS-TT are two required components for TSN, and that both are managed by the TSCTSF, an offloading of delivery of the containers from the SMF to the UPF may provide substantial gains.

Considering the motivation of WT#1.1 of FS_6G_ARC, the signalling for UP management should be designed to minimize impact on other NFs. Also, the 6G system design should support reducing overall Session management transaction time and reducing processing overhead on NFs in the middle.

- Dependency of PDU session signalling on AMF availability

Session management (e.g., PDU session) related signalling between the UE and the SMF depends entirely on the availability of the signalling path between the UE and the SMF, with the AMF in the middle. If the AMF becomes congested or unavailable, signalling between the UE and the SMF is affected, because messages for a PDU session triggered by the SMF cannot be delivered without involvement of the AMF.

In 5GS, SM signalling for certain scenario (e.g., PS Data off) was allowed, even when the AMF is congested. However, when the AMF is congested, this small exceptional signalling may just add additional load to the AMF.

At the last meeting, many companies mentioned a needs to make the 6G system more resilient. This means, when a temporary unavailability/congestion of the AMF occurs, entire breakdown of system should be avoided. Because AMF is in charge of mobility management, and the SMF is in charge of session management, the impact of AMF’s unavailability on PDU session related signalling should be minimized. This may require that entire reliance of PDU session signalling via the AMF needs to be avoided. Having two independent UE-core network paths for SM signalling may help in providing resiliency.

Maintaining the strict boundary between the role of SMF and the role of UPF may not be desirable in 6G. Having a flexible boundary of roles of the SMF-UPF could allow more robust PDU session management, e.g., by not relying on just one path (e.g., signalling path via the AMF).

- Latency implications of multi-node involvement

A signalling exchange with a UE and a SMF can be triggered by an event detected at the UPF. In this case, to update a parameter for a PDU session, the UPF needs to send a report to the SMF, and the SMF sends new parameters to the UE via the AMF and gNB. Compared to a potential signalling exchange between the UPF and the UE directly, the signalling path involving the AMF has more hops. In this case, the overall transaction time using the signalling path between a UE and the SMF and via the AMF can be longer than the direct signalling path at between the UE and the UPF, and may resulting in higher latency and a slower reaction time.

In coming years, 6G needs to support advanced applications requiring more dynamic adaptation to changing traffic needs. So, for some scenario, e.g., minor QoS adjustments, where actual enforcement occurs in the UE and the UPF, there is a need to investigate whether current 5G architecture always requiring UE–SMF signalling is a viable approach. As low-latency requirements become more critical, faster responsive mechanisms for PDU session handling will be necessary.

- Need for Unified framework for UPF based signalling

In later releases, 5GS adopted a few mechanisms for signalling between the UE and the UPF via a user plane. For example, in ATSSS, availability/measurement/assistance related information can be directly signalled between the UPF and the UE, without SMF involvement. Also, for positioning, a signalling exchange between the LMF and the UE via user plane is introduced in later phase of 5GS. However, because this functionality is introduced in later release of 5G as an add-on feature, there is no general common architecture.

In previous meeting, there were proposals for supporting more tight coupling between a network and an application. Some further proposed to in-band signalling over user plane to dynamically share information of an application between the UE and/or the UPF. This may be another example of how user plane can be used for signalling related to a PDU session, in addition to a basic signalling exchange via a control plane (e.g., via AMF).

If a general framework for signalling between the UPF and the UE can be introduced, all above mechanisms can be supported in a unified UP-based signalling framework. This may help easy introduction of features in the later release, if the signalling for the features needs to be introduced over user plane.

- Load balancing of controlling

Current architecture of 5G is based on separation of control plane functionality and user plane functionality, for a PDU session. Initially, 5GS system was designed by putting all control related functionality in the SMF, and by putting all user plane handling in the SMF.

As discussed above, in later release, 3GPP supported a minimum signalling exchange between the UPF and the UE. This can be starting point to investigate what functionality of current SMF can be delegated to UPF. Especially, considering that the UPF provides services to other entities than the SMF, we need to view the UPF as a node which can handle some basic control functionality, in addition to user plane data handling.

Then, there is a need for a study on which control functionality is more suitable for SMF to handle and/or which control functionality is more suitable for UPF to handle, or which control information can be exchanged between the UE and the UPF, without using other NFs such as AMF/SMF. If this approach is adopted, based on which functionality is used more often, separate scaling of network functions can be supported.

# 2. Proposal

It is proposed to agree on following text proposal.

**** First Change relative to S2-2508121****

# Annex A.X. WT#1.2 User Plane Architecture for 6G Scope

In order to support 6G user plane for a diverse set of applications and traffic patterns, the following are studied:

- Whether and how to enhance application-network collaboration for 6G user plane traffic to enable the UE/AS to provide the network with information that allows the network to adapt to application traffic.
Editor’s Note: whether this issue belongs to UP or QoS is FFS.

- Whether and how to enhance CP-UP interaction for better interoperability.

- Whether and how to extend the service-based architecture for the user plane control interface to UPF.

- Whether and how to support flexible user plane (i.e., UPF (re)selection) for different service requirements considering path performance and capability between access network and data network.

- Whether and how to enhance the user plane handling for session continuity and mobility between access network and data network (i.e., best path selection considering ULCL and PSA-UPF path metrics, and/or for specific services).

- Whether and how to provide a general unified framework on signalling exchange for session management via user plane path, to support resiliency and dynamic fast signalling, while reducing signalling load compared to control plane path.

NOTE 1: Coordination with WT#1.1 and WT#1.2 QoS may be required.

**** Second Change – all text is new ****

# 5.X. Key Issue for WT#1.2 on User Plane Architecture for 6G

This key issue will study how to design the user plane architecture in 6G. The study will cover, but not be limited to, the following aspects:

- Study whether and how to provide a general unified framework on signalling exchange for session management via user plane path, to support resiliency and dynamic fast signalling, while reducing signalling load compared to control plane path.

NOTE 1: Coordination with WT#1.1 and WT#1.2 QoS may be required.

**** Third Change relative to S2-2508122****

# Annex A.X WT#1.2.X: Evolution of QoS Framework for 6G

- Investigate if any new functionality in 6G QoS framework considering the emerging new traffic characteristics and application needs.

NOTE 1: The definition of 6GS QoS concepts in the 6G System provider perspective and in the 6G System usage perspective respectively should have clear boundary (enable clear & measurable connectivity service descriptions for SLAs).

- Whether and how to enhance QoS control mechanisms to

  - better support QoS targets than the best effort approach of existing non-GBR and less resource intensive than the existing GBR, Delay Critical-GBR to improve capacity utilization in the network.

  - to improve QoS for applications with dynamic requirements.

- Whether and how to support signalling exchange for QoS management in a dynamic, low-latency, and resilient way (e.g., over user plane).

NOTE X: Coordination with WT#1.1 and WT#1.2 User Plane is required.

- Whether and what to monitor in relation to QoS Framework.

- Study whether and how to support the UE and network QoS collaboration to improve awareness in the 6GS of connectivity performance needs.

- UE provided information to the network for QoS collaboration

- Application and Network QoS collaboration including what information shall be exchanged by means of ‘application/UE and network collaboration’ [see WT#1.2 User Plane].

NOTE 2: Applications typically consist of an Application Client in the UE and application on the server side.

- Study how to ensure interworking with 5GS.

NOTE 3: The content of the 5GS QoS Framework that will be used as a starting point for discussion on 6G QoS will be determined by moderated discussion process.

NOTE 4: Aspects of traffic detection are assumed to be handled by the WT#1.2 User Plane.

NOTE 5: Impacts on QoS originating from new services will be considered based on and when relevant input is identified and provided by the relevant WTs.

NOTE 6: This WT will coordinate with SA WG4 and RAN WGs.

**** Fourth Change – all text is new ****

# 5.X. Key Issue for WT#1.2 on Evolution of QoS Framework for 6G

This key issue will study how to evolve the QoS Framework in 6G. The study will cover, but not be limited to, the following aspects:

- Study whether and how to support signalling exchange for QoS management in a dynamic, low-latency, and resilient way (e.g., over user plane).

NOTE 1: Coordination with WT#1.1 and WT#1.2 QoS may be required.

**** End of Changes ****