3GPP TSG-WG SA2 Meeting #171	S2-2509161

13 – 17 Oct, 2025, Wuhan, China	(Revision of S2-2508123)

Source:	Ofinno

Title:	[WT#1.2, Policy] Scope and key issue on Policy Framework in 6G

Document for:	Approval

Agenda Item:	20.6.1.2

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: This contribution proposes scope and key issue to Work task 1.2 of the study on Policy Framework for 6G.

# 0. Background

In SA2#170, SA2 discussed proposals on updating WTs for FS_6G_ARC. S2-2508123 related to Policy aspects was discussed as baseline document and finally was postponed.

Following are text proposals in S2-2508123

# Annex A.X. WT#1.2 [Policy] Scope Editor's Note: Describe the technical scope of the proposed Work Task. If applicable, suggest logical subdivision of this WT into smaller sub-WT. This clause is part of the TR Annex. To support Policy Framework in 6GS, the Policy and charging control Framework defined in TS 23.503 for policies (i.e., SM Policy, UE Policy, AM Policy) is considered as starting point for discussion, following aspects are to be studied: - Whether and how to harmonize the UE Policy provisioning procedures for UE Policies in 6GS. - Whether and how to consider user input as input to PCC decisions at the Core Network for 6G. NOTE 1: How the user input is provided to the core network for 6G, e.g. via and AF or not is part of the study. - Whether and how to evolve the External Parameter Provisioning Framework aspects of policy control for 6G. NOTE 2: General Authorization of the external parameter provisioning is handled in the WT#1.2 Network Exposure. 4. Whether and how to support policy control consumers to obtain policies efficiently (e.g. to reduce signalling when requesting policies). 5. Whether and how to simplify the policy control framework for 6G to e.g. reduce the number of PCF services that provide policies to the different policy enforcement points or e.g. how to perform the mapping from application to user plane. NOTE 3: whether and how 6G network slicing functionality maps application to slice for 6G is covered in WT#1.2 Slicing. 6. How to support policy and charging control in roaming in 6G. NOTE 4: This work task takes energy efficiency and consumption considerations for policy decisions into account. NOTE 5: This WT has IWK aspects that are scoped under WT#2. NOTE 6: This WT can study potential enhancements to the policy control framework for 6G services based on the input identified and provided by other WTs. These aspects require further discussion, no agreement to be included in this WT. - Whether and how policies can be provided to the UE to also consider user preference (or UE local configurations) for policy enforcement. - Whether any of the procedures for an AF to influence on policy control, e.g. QoS control, that are available in 5G can be requested by the UE, acting as an AF.

# 1. Discussion

In 5G, policy control was supported by the Policy and Charging Control (PCC) framework defined in TS 23.503, including AM Policy, SM Policy, and UE Policy. While this provided flexibility, it also resulted in significant complexity: policies were distributed across different logical types, the Policy Control Function (PCF) was fragmented both logically and physically, and procedures for delivery and enforcement were often overlapping or redundant.

For 6G, it is expected that the policy framework will be simplified and unified, reducing the fragmentation seen in 5G and providing a more streamlined architecture for policy decision and enforcement. Such improvements should also align with new requirements coming from other areas of the system, including network slicing.

At the same time, network slicing was introduced in 5G as a key feature enabling operators to create differentiated services and business models, while offering users the possibility of premium QoS. At the SA2 level, this was realized by allowing applications to be mapped to network slices, notably via URSP rules that connected App IDs with S-NSSAIs. While effective in concept, this direct App ID-slice mapping raised serious user privacy concerns, since it implicitly revealed which applications were running on the user device.

To mitigate this, Rel-18 adopted requirements from GSMA and introduced the notion of traffic categories, defined both in standardized ranges and operator-specific ranges (as specified in TS 23.503 Table 6.6.2.1-2 and TS 24.526 Table 5.2.1). This allowed operators to map applications to traffic categories, and categories to slices, thereby reducing the exposure of explicit App IDs. However, this approach still relied on operator-specific categorization, leaving residual privacy risks and scalability issues.

In 6G, as network slicing evolves towards simplification and renewed QoS differentiation, the policy framework study should also consider how application traffic can be mapped to the user plane in a flexible and scalable way, while ensuring preservation of user privacy at the UE. This requires close integration between WT#1.2 Slicing (defining mechanisms for application traffic to slice mapping) and WT#1.2 Policy (defining how such requirements are realized in the policy framework).

In addition, since application traffic originates from user activity on the UE, user preferences or inputs should also be taken into account. Such user preferences may affect both policy decisions and policy delivery mechanisms associated with application usage. These aspects should therefore be considered jointly in this study.

## 2. Proposal

This contribution proposes to update the TR 23.801-01 to include the scope of WT#1.2 and the related Key Issue. The text for WT#1.2 Scope on Policy Framework is proposed in the first change of this paper, and it is on top of the latest version of baseline S2-2508123 discussed in SA2#170. The related KI text is proposed in the second change of this paper.

**** First Change relative to S2-2508123****

# Annex A.X. WT#1.2 [Policy] Scope

Editor's Note: Describe the technical scope of the proposed Work Task. If applicable, suggest logical subdivision of this WT into smaller sub-WT. This clause is part of the TR Annex.

To support Policy Framework in 6GS, the Policy and charging control Framework defined in TS 23.503 for policies (i.e., SM Policy, UE Policy, AM Policy) is considered as starting point for discussion, following aspects are to be studied:

- Whether and how to harmonize the UE Policy provisioning procedures for UE Policies in 6GS.

- Whether and how to consider user input as input to PCC decisions at the Core Network for 6G.

NOTE 1: How the user input is provided to the core network for 6G, e.g. via and AF or not is part of the study.

- Whether and how to evolve the External Parameter Provisioning Framework aspects of policy control for 6G.

NOTE 2: General Authorization of the external parameter provisioning is handled in the WT#1.2 Network   Exposure.

4.	Whether and how to support policy control consumers to obtain policies efficiently (e.g. to reduce signalling when requesting policies).

5.	Whether and how to simplify the policy control framework for 6G to e.g. reduce the number of PCF services that provide policies to the different policy enforcement points or e.g. how to perform the mapping from application to user plane.

NOTE 3: whether and how 6G network slicing functionality maps application to slice for 6G is covered in WT#1.2 Slicing.

6.   How to support policy and charging control in roaming in 6G.

NOTE 4:	This work task takes energy efficiency and consumption considerations for policy decisions into account.

NOTE 5:  This WT has IWK aspects that are scoped under WT#2.

NOTE 6:  This WT can study potential enhancements to the policy control framework for 6G services based on the   input identified and provided by other WTs.

**** Second Change – all text is new ****

# 5.X. Key Issue #X Policy Framework in 6G

This key issue will study how to design the policy framework in 6G. This study will cover, but not be limited to, the following aspects:

- Whether and how to enhance policy decision and delivery procedures in an efficient manner, including at least:

  - How to involve user preference (or user input) in policy decision, policy delivery (provisioning, obtaining), and policy enforcement procedures.

  - How to enable the mapping of application traffic to user plane in a flexible and scalable manner, while ensuring preservation of user privacy at the UE.

NOTE:	Bullet 1b will be aligned with the potential requirements on how to map application traffic to slices that are identified in WT#1.2 Slicing study.

**** End of Changes ****