SA WG2 Meeting #171	S2-2509197

Wuhan, China, 13-17 October 2025

Source:	Qualcomm Incorporated

Title:	Interim agreements for FS_Sensing_ARC KI#3

Document for:	Agreement

Agenda Item:	20.2.1

Work Item / Release:	FS_Sensing_ARC/Rel-20

Abstract of the contribution: This contribution proposed the interim agreements for KI#3 of ISAC study.

# 1.  Discussions

Based on the discussion in the SA2 and the solutions documented in TR 23.700-14, this contribution proposed the interim agreements for KI#3 Sensing Entity and Sensing Function Discovery and (Re-)Selection.

# 2.  Proposals

To apply the following change to the TR 23.700-14.

**** First Change ****

### 7.1.Y	Agreed Principles for KI#3

The following principles are agreed in this release to address the key issue on Sensing Entity and Sensing Function Discovery and (Re-)Selection:

-	Sensing Function is selected by the Sensing Gateway Function, based on the parameters of the sensing service request, e.g. target sensing area, sensing service type, etc.;

-	Sensing Entity is selected by the Sensing Function, based on the parameters of the sensing service request;

-	capabilities of the Sensing Entities are provided to the Sensing Function during the direct connection establishment.

-	Sensing Function may trigger Sensing Entity re-selection during a sensing service;

-	Sensing Function may select multiple Sensing Entities to serve a sensing service request.

**** Next Change ****

### 7.2.Z	Topics for further consideration for KI#3

The following topics requires further considerations and would not be pursued in normative phase of this release:

-	sensing service session transfer between the Sensing Functions.

NOTE:  	If a Sensing Function can no longer serve a sensing service request, e.g. a UAV moves out of its service area, it will notify the Sensing Gateway Function to re-select a Sensing Function and initiate another sensing service session with the new Sensing Function.

**** End of Changes ****