SA WG2 Meeting #171	S2-2509436

13 - 17 October, 2025, Wuhan, China                  	(revision of S2-2509382)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.369 | CR | 0029 | rev | 3 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Resolve Security Related ENs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE, Ericsson, Huawei, HiSilicon, Sony, Samsung, CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AmbientIoT-ARC |  |  |  |  |  | Date: |  |  | 2025-10-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The paper is proposed to resolve security related ENs. SA3 has made some progress regarding authentication of AIoT Devices, protection of Device ID and filtering information, and AIoT message protection which are specified in TS 33.369. In the procedure of command, several ENs are removed and some descriptions regarding security are added. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify the security for the AIoT messages in the command procedure Remove related ENs. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The security for AIoT messages in the command procedure are not described; and ENs exist. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

6.2.3	Command Procedure

Figure 6.2.3-1 depicts the command procedure.

The procedure focuses on the messages and parameters used for the communication between AIOTF and NG-RAN regardless of the path to access NG-RAN, see clause 4.2.2.1. The handling of the different communication paths is described in clause 6.2.4.

Figure 6.2.3-1: Command Procedure

1.	The AF sends the Nnef_AIoT_Command Request (AF ID, Command Type, information about the target AIoT Device(s), [External Target Area information], [Approximate number of AIoT Devices], [Approximate D2R message size], [Command type specific parameters], [location information requested]) message to NEF.

Information about the target AIoT Device(s) may include Filtering Information, as described in clause 5.8, or include complete AIoT Device Identifier(s).

The approximate number of AIoT Devices (see clause 5.4), if provided, is used to indicate the number of AIoT Devices expected to respond to this AIoT service operation request, which is sent by AIOTF to the NG-RAN in the assistance information as specified in clause 5.4.

Command Type provides the operation to be performed and the Command type specific parameters provides the required parameters for the operation. The service operations are described in clause 5.2.2.

2.	Step 2 of the Inventory Procedure specified in clause 6.2.2 is performed for External Target Area information processing and AIOTF selection with the following clarifications:

-	If AIOTF selection fails, the NEF rejects the Nnef_AIoT_Command request and step 6 of this procedure is performed instead.

3.	The NEF sends Naiotf_AIoT_Command Request message (AF ID, Command Type, information about the target AIoT Device(s), [Target area information], [Approximate number of AIoT Devices], [Approximate D2R message size], [Command type specific parameters], [location information requested]) message to the selected AIOTF(s).

4.	The AIOTF receives the Naiotf_AIoT_Command Request and checks the parameters included in the request. The AIOTF performs Reader Selection as specified in clause 5.3.3. If no NG-RAN or RAN Reader can be selected, the AIOTF rejects the AIoT Command request with an appropriate cause code.

The AIOTF generates a Correlation ID corresponding to this AIoT service operation request and the Correlation ID is used for the AIOTF to correlate the service operation responses received from NG-RAN to the request. The AIOTF creates the AIoT Session for the AF service operation request, which is identified by the Correlation ID.

The AIOTF performs Reader Selection, see clause 5.3.3. If no NG-RAN or RAN Reader can be selected, the AIOTF rejects the AIoT service operation request with an appropriate cause code.

The AIOTF determines assistance information as described in clause 5.4, taking into account the parameters provided in the AIoT service operation request.

The AIOTF performs AF authorization for AIoT service operation request as described in clause 5.6.

The AIOTF may perform AMF selection as described in clause 5.3.4.

5.	AIOTF sends the Naiotf_AIoT_Command Response message (accept or reject, [cause code]) to the NEF.

6.	NEF sends the Nnef_AIoT_Command Response message (accept or reject, [cause code]) to the AF. If the response was a reject the procedure stops here.

7.	Step 7 to step12 of procedure for Inventory specified in clause 6.2.2 are performed with the following clarifications:

-	In step 7, the AIOTF also includes follow on command indication in the Inventory Request message to inform the NG-RAN command delivery occurs after the inventory.

-	In step 10, the NG-RAN also includes the RAN AIoT Device NGAP ID for each AIoT Device in the Inventory Report as specified in TS 38.413 [10].

-	In step 11, the AIOTF authenticates the AIoT Devices as specified in TS 33.369 [9], and determines whether the command should be sent to an AIoT Device, e.g., by checking the Target AIoT device information. The AIOTF updates the corresponding AIoT device context in the AIOTF to include the RAN AIoT Device NGAP ID.

If none of successful Inventory response is received, Step 8 -11 is not performed and the AIOTF sends a failure report to the NEF in step 12.

8.	For each successful Inventory response received, the AIOTF sends Command Request message (Correlation ID, [Reader ID], NAS Command Request, [Approximate D2R message size], RAN AIoT Device NGAP ID for each AIoT Device) to the NG-RAN directly or as a NGAP AIoT information via an AMF as specified in clause 6.2.4. The NAS Command Request message includes the AIoT data. The Correlation ID is as the same as the Correlation ID generated in step 4. The RAN AIoT Device NGAP ID for each AIoT Device is used by the NG-RAN to determine the AIoT device context in NG-RAN as specified in TS 38.413 [10].

The AIOTF uses the Command Type and Command type specific parameters received in Step 3 to determine the NAS Command Request to send to the AIoT Device, as described in clause 5.2.2.

The protection of the NAS Command Request message is specified in clause 5.3 of TS 33.369 [9].

NOTE:	Command Request(s) can be sent to NG-RAN when inventory procedure is ongoing.

9.	The NG-RAN sends the AS R2D message (NAS Command Request) to the AIoT Device as defined in TS 38.391 [11].

10.	The AIoT Device sends the AS D2R message (NAS Command Response) to the NG-RAN as defined in TS 38.391 [11]. The NAS Command Response message may include the AIoT data.

The protection of the NAS Command Response message is specified in clause 5.3 of TS 33.369 [9].

11.	The NG-RAN responds with a Command Response message (Correlation ID, Reader ID, NAS Command Response, RAN AIoT Device NGAP ID) to the AIOTF directly or as a NGAP AIoT information via an AMF as specified in clause 6.2.4. The AIOTF determines the AIoT device context by the RAN AIoT Device NGAP ID received.

12.	The AIOTF reports the result of the Naiotf_AIoT_Command request to the NEF by sending the Naiotf_AIoT_Command Notify message (a list of AIoT Device(s) response information (AIoT Device ID(s), AIoT data and optionally location of each AIoT Device), AF ID, [Last Report Indication]). If multiple AIOTFs are involved in the procedure, the NEF may receive Naiotf_AIoT_Command Notify messages from multiple AIOTFs.

Based on operator policy, if the location information is requested by the AF and if the location of the reader is configured, the AIOTF uses the Reader ID reported from NG-RAN during inventory in step 7 to determine the AIoT Device Location.

When the last report is sent, the AIOTF ends the AIoT Session.

13.	The NEF informs the AF of the result of the Nnef_AIoT_Command request by sending the Nnef_AIoT_Command Notify message (a list of AIoT Device(s) response information (AIoT Device ID(s), AIoT data and optionally location of each AIoT Device), AF ID, [Last Report Indication]).

END OF CHANGES