3GPP TSG-SA2 Meeting #171	S2-2508643

# Wuhan, China, 13th Oct 2025 - 17th Oct 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 5420 | rev | 3 | Current version: | 19.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | KI#4 PCF event exposure related to signalling load analytics |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2025-10-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | TS 23.502 contains the following: #### 5.2.5.7 Npcf_EventExposure service ##### 5.2.5.7.1 General Service description: This service enables an NF to subscribe and get notified about PCF events for a group of UE(s) or any UE accessing a combination of (DNN, S-NSSAI). The events can be subscribed by a NF consumer are described in clause 6.1.3.18 of TS 23.503 [20]. When the consumer NF is the NWDAF, the event ID “Signalling Storm” including Request type and number of requests corresponding to the request type from NF, Unexpected operational status indicator, etc. is used to collect data for NF related information from PCF for Signalling Storm Analytics as specified in clause 6.x.2 of TS 23.288 [50]. .. Related stage 3 TS 29.523 contains: Table 5.6.2.8-1: Definition of type PcEventNotification sigInfos; array(NfSignallingInfo); C; 1..N; Contains signalling information. It shall be included when the reported event is "SIGNALLING_INFO".; SignallingInfo. And TS 29.571 defines: #### 5.2.4.29 Type NfSignallingInfo Table 5.2.4.29-1: Definition of type NfSignallingInfo Attribute name; Data type; P; Cardinality; Description. nfSigInfoPerWndw; array(NfSignallingInfoPerTimeWindow); O; 1..N; Contains NF signalling information per time window.. avgReqProcTime; Uinteger; O; 0..1; The average processing time (in milliseconds) of each request, i.e. the time duration between receiving the request from an NF and sending the response to the NF.. nfHeartbeatInfo; NfHeartbeatInfo; O; 0..1; Contains NF heartbeat-related information.. unexpStatusInd; boolean; O; 0..1; Indicates whether the NF is at an unexpected status (i.e. deviates from the normal operations, based on thresholds or rules configured by operator). - "true": the NF is at an unexpected status. - "false": the NF is not at an unexpected status. The default value is "false" if omitted.. #### 5.2.4.30 Type NfSignallingInfoPerTimeWindow Table 5.2.4.30-1: Definition of type NfSignallingInfoPerTimeWindow Attribute name; Data type; P; Cardinality; Description. tw; TimeWindow; M; 1; The time window in which the provided signalling information occurred.. nfSigInfoPerService; map(NfSignallingInfoPerService); M; 1..N; Each entry of the map contains NF signalling information for a specific service. The key of the map is the "serviceName" attribute of the NfSignallingInfoPerService data type.. #### 5.2.4.31 Type NfSignallingInfoPerService Table 5.2.4.31-1: Definition of type NfSignallingInfoPerService Attribute name; Data type; P; Cardinality; Description. serviceName; ServiceName; M; 1; The name of the service as specified in clause 6.1.6.3.11 of 3GPP TS 29.510 [29] to which the provided signalling information refers.. numOfReq; Uinteger; O; 0..1; The number of requests received for this service. (NOTE). numOfReqUnresp; Uinteger; O; 0..1; The number of requests received for this service which were not responded. (NOTE). numOfReqReject; Uinteger; O; 0..1; The number of requests received for this service which were rejected. (NOTE). numOfRedMessages; Uinteger; O; 0..1; The number of redundant received messages, i.e. messages which were transmitted multiple times.. numOfPosteriorReq; Uinteger; O; 0..1; The number of posterior requests, i.e. requests that were triggered by a previous request received by the same NF.. Editor's Note: It is FFS, whether the above measurements should be reported per service name or per service instance. #### 5.2.4.32 Type NfHeartbeatInfo Table 5.2.4.32-1: Definition of type NfHeartbeatInfo Attribute name; Data type; P; Cardinality; Description. numOfRetrans; Uinteger; O; 0..1; Number of retransmissions performed.. hbIntvl; Uinteger; O; 0..1; NF heartbeat interval in milliseconds.. However, TS 23.503 Clause 6.1.3.18 does not define an NFSignallingInfo Event. As the Event is only vaguly related to PCC, it may be preferable to define it in TS 23.502 rather than TS 23.503. |
| Summary of change: |  | Align with TS 23.288 and stage 3 TS 23.523 by adding to Npcf_EventExposure Service event exposure parameters required as input to related to signalling load analytics. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misalignment with TS 23.288 and stage 3 TS 23.523 related to Npcf_EventExposure Service event exposure parameters for signalling load analytics |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.5.7.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | S2-2508644 contains an alternative proposal to document the new event in TS 23.503. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st change: Npcf_EventExposure Service

##### 5.2.5.7.1	General

Service description: This service enables an NF to subscribe and get notified about PCF events for a group of UE(s) or any UE.

The events that can be subscribed by a NF consumer are the events described in clause 6.1.3.18 of TS 23.503 [20] and the following events:

- Signalling Information: The PCF reports information related the amount of successful and failed signalling interactions with peer NF service instances, as detailed in Clause 6.22.2 of TS 23.288 [50].

When the consumer NF is the NWDAF, the event ID "Service Signalling characteristics" including Request type and number of requests corresponding to the request type from NF, Unexpected operational status indicator, etc. is used to collect data for NF related information from PCF for Signalling Storm Analytics as specified in clause 6.22.2 of TS 23.288 [50].

The following service operations are defined for the Npcf_EventExposure service:

-	Npcf_EventExposure_Subscribe.

-	Npcf_EventExposure_UnSubscribe.

-	Npcf_EventExposure_Notify.

End of changes